3GPP TSG-SA WG2 Meeting #171	S2-2509170

# Wuhan, China, 13 - 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6462 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on allocation of CN assigned subgroup ID for LP-WUS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN3 has indicated in the LS (S2-2508191/R3-255941) the following: --- Regarding the range of CN assigned subgroup ID for LP-WUS, RAN3 made the following agreements: - The range of CN assigned subgroup ID in NG/Xn/F1 message is 0.. 30. This agreement means that when the AMF allocates CN assigned subgroup ID for the UE, it should consider the least range of subgroup ID available over NG-RAN cells corresponding to the UE registration area. RAN3 would like to provide following background information: according to the TS 38.213, for CN assigned subgroup ID range, dependent on the per-cell PO (Paging Occasion) to LO (LP-WUS Occasion) association: - in case PO-to-LO association is 4, subgroup ID #0 to #6 can be used;. - in case PO-to-LO association is 2, subgroup ID #0 to #14 can be used; - in case PO-to-LO association is 1, subgroup ID #0 to #30 can be used; --- This above needs to be reflected in SA2 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the AMF behavior on allocation of CN assigned subgroup ID for LP-WUS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with RAN conclusion. AMF mistakenly allocates a wrong LP-WUS Subgroup ID to UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.12a.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---First Change---

#### 5.4.12a.1	General

The RAN and UE may use Low Power Wake UP Signal with Paging Subgrouping (LP-WUSPS) to reduce the power consumption of the UE in RRC_IDLE and RRC_INACTIVE over NR (see TS 38.304 [50]).

Editor's note:	The terminology of "LP-WUS" and "LP-WUSPS" will be further aligned with the terminology used in specifications at RAN Side.

The LP-WUSPS can be based on a LP-WUS Subgroup ID allocated by the AMF, or is determined by the RAN on its own as defined in TS 38.304 [50] and TS 38.331 [28].

The AMF, when determining its paging strategy (see clause 5.4.3), should take into consideration whether a gNB is using LP-WUSPS determined by the RAN on its own.

If paging subgroups for LP-WUS are being allocated by the AMF, the AMF should consider the narrowest LP-WUS Subgroup ID range applicable to the cells in the UE's registration area and configure the UE with an LP-WUS Subgroup ID belonging to this narrowest range. All AMFs connected to one gNB (including the AMFs in different PLMNs using 5G MOCN network sharing) shall use a consistent policy in allocating UEs to the paging subgroups.

NOTE:	Because the UE uses the AMF allocated LP-WUSPS across all cells in its TAI-list, and, different, overlapping TAI lists can be allocated to different UEs, then to avoid UE power consumption increasing, it is likely to be necessary that all AMFs with an overlapping coverage area use a consistent policy in allocating UEs to the LP-WUSPS.

The NG-RAN node and the UE determine per cell which LP-WUSPS method to use as defined in TS 38.304 [50] and TS 38.331 [28].

## ---End of Change---