SA WG2 Meeting #171	S2-2509174

# October 13 - 17, 2025, Wuhan, China

Source: 	NTT DOCOMO

Title: 	KI#3: New Sol: Usage of IMS PDN connection for Emergency PDN connection

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#3.

1	Introduction

This contribution proposes a solution for KI#3 on support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC.

This solution assumes the use of a CP bearer for SIP/I1 messages and a UP bearer for voice media in 1 PDN connection.

This solution reduces the emergency call setup time by skipping establishment of Emergency PDN connection and instead using the usual IMS PDN connection if P-GW selected for IMS PDN connection happens to be the same P-GW used for Emergency PDN connection. When used as Emergency PDN connection, ARP priority level is increased.

2	Proposal

It is proposed to add the following text to TR 23.700-19 v1.0.0.

********** First Change **********

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #X |  |  | X |  |

********** Next Change (All new text) **********

6.x	Solution #x: Usage of IMS PDN connection for Emergency PDN connection

6.x.0	High-level solution Principles

This solution is for KI#3 on support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC.

This solution assumes the use of a CP bearer for SIP/I1 messages and a UP bearer for voice media in 1 PDN connection.

This solution reduces the emergency call setup time by skipping establishment of Emergency PDN connection and instead using the usual IMS PDN connection if P-GW selected for IMS PDN connection happens to be the same P-GW used for Emergency PDN connection. When used as Emergency PDN connection, ARP priority level is increased.

6.x.1	Description

The assumptions of this solution are as follows:

- It is assumed that signaling (e.g., SIP messages, I1-like messages) and voice media are transferred in different bearers in 1 PDN connection.

- It is assumed to transfer SIP/I1 signaling in NAS by using a CP bearer, which is a default bearer.

- It is assumed to transfer voice media by using a UP bearer, which is a dedicated bearer.

- The solution works both for "IP solution" and "Non-IP solution".

The principles of this solution are as follows:

- When MME selects P-GW for IMS PDN connection, MME also checks which P-GW to choose for Emergency PDN connection. If both P-GWs are the same, MME sends a corresponding indication to UE.

- When UE has received this indication and determines to make a UE detectable emergency call, the UE does not establish Emergency PDN connection, instead uses the IMS PDN connection. The UE establishes an RRC connection using establishment Cause=emergency and sends SIP INVITE (or a Il-like message equivalent to it) in NAS.

- When MME recognizes establishment Cause=emergency, MME informs to S-GW and eNB to increase the ARP priority level.

- When P-GW recognizes inactivity in bearers of Emergency PDN connection, P-GW informs S-GW to decrease the ARP priority level.

The advantages of this solution are as follows:

- The emergency call setup time can be reduced by the time needed to set up Emergency PDN connection and a dedicated bearer in it.

6.x.2	Procedures

6.x.2.1	Attach procedure

Figure 6.x.2.1-1: Attach procedure

1.	UE sends the Attach request to MME.

2.	MME selects P-GW. MME also checks which P-GW to choose for Emergency PDN connection. If both P-GWs are the same, MME decides to send "Common P-GW with Emergency PDN connection" (a new indicator) to UE.

3.	MME sends to UE the Attach accept containing "Common P-GW with Emergency PDN connection".

6.x.2.2	Emergency call procedure

Figure 6.x.2.2-1: Emergency call procedure

1.	UE decides to make a UE detectable emergency call. UE has received "Common P-GW with Emergency PDN connection" and decides not to establish Emergency PDN connection, instead to use the IMS PDN connection.

2.	UE sends RRCConnectionRequest containing establishmentCause=Emergency to eNB. UE receives RRCConnectionSetup and sends RRCConnectionSetupComplete containing NAS(SIP INVITE for emergency call w/o 5tuple). eNB sends to MME the Initial UE message containing NAS(SIP INVITE for emergency call w/o 5tuple) and RRC Establishment Cause=emergency.

3.	MME sends SIP INVITE for emergency call w/o 5tuple via S-GW to P-GW.

4.	P-GW adds 5tuple.

5.	P-GW sends SIP INVITE for emergency call to P-CSCF.

6.	In parallel to step 3, MME sends to S-GW the Modify Bearer Request containing not only S11-U MME F-TEID and S1 eNodeB F-TEID but also Bearer Level QoS=a higher ARP level both for the default bearer and the dedicated bearer.

7.	S-GW recognizes the Bearer Level QoS IE and sends to P-GW the Modify Bearer Request containing Bearer Level QoS=a higher ARP level both for the default bearer and the dedicated bearer.

8.	P-GW sends the Modify Bearer Response to S-GW.

9.	S-GW sends the Modify Bearer Response to MME.

10.	MME sends to eNB the Initial Context Setup request containing E-RAB Level QoS Parameters containing Allocation and Retention Priority=<higher ARP level> for the dedicated bearer.

11.	eNB sends RRCConnectionReconfiguration to UE. UE sends RRCConnectionReconfigurationComplete to eNB.

12.	eNB sends the Initial Context Setup response to MME.

13.	The procedure in EPS and in IMS continues. An emergency call is setup.

14.	The emergency call is finished.

15.	P-GW recognizes inactivity of the emergency call in the default bearer and the dedicated bearer.

16.	P-GW sends to S-GW the Update Bearer Request containing Bearer Level QoS=normal ARP level both for the default bearer and the dedicated bearer.

17.	S-GW sends to MME the Update Bearer Request containing Bearer Level QoS=normal ARP level both for the default bearer and the dedicated bearer.

18.	MME sends the Update Bearer Response to S-GW.

19. S-GW sends the Update Bearer Response to P-GW.

20. MME sends to eNB the E-RAB Modify request containing E-RAB Level QoS Parameters containing Allocation and Retention Priority=<normal ARP level> for the dedicated bearer.

21.	eNB sends the E-RAB Modify response to MME.

6.x.3	Impacts to Services, Entities and Interfaces

UE:

-	Handling of a new indicator "Common P-GW with Emergency PDN connection". Not to establish Emergency PDN connection when received it.

MME:

-	Handling of a new indicator "Common P-GW with Emergency PDN connection"

-	Based on the RRC Establishment Cause=emergency, decide to prioritize the default bearer and the dedicated bearer in the IMS PDN connection.

S-GW:

-	Recognizing Bearer Level QoS IE in the Modify Bearer Request, sends a Modify Bearer Request to P-GW.

P-GW:

-	Based on inactivity in the IMS PDN connection used as Emergency PDN connection, decide to change the ARP level of the default bearer and the dedicated bearer to be the normal.

********** End of Changes **********