SA WG2 Meeting #171	S2-2509184

13 – 17 Oct 2025, Wuhan, CN                                                                was (S2-2509151)

Source:			Nokia, T-Mobile USA, TurkTelekom

Title:			[WT#4] WT for Integrated Sensing And Communication

Document for:			Agreement

Agenda Item:			20.6.4

Work Item / Release:			FS_6G_ARC / Rel-20

Abstract of the contribution: WT revision proposal on ISAC for 6G based on outcome of SA2 170.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

S2-2508043 in SA2 170 meeting paper was postponed with aggregable content and with some content requiring further clarification.

Observation 1 – Mobility clarification

f)	Sensing continuity in consideration of mobility of sensing entities and/or target objects.

The proposal is to clarify that the mobility that is scoped in is only when the Sensing Entity is a UE.

Observation 2 – Dependencies on WT5.

NOTE X3:	Aspects a), b) and e) require coordination with WT#5. The data framework applicable for collection, distribution and exposure of sensing related data is in scope of WT#5.

It is unclear to us why aspect b) has relation to WT5. Additionally, it is required to only indicate that there is co-ordination required with WT5, but without getting into the details on what WT5 is supposed to do. Hence, the proposal is to clarify that bullet b) is not dependent on WT5 and leave the corresponding note to indicate that there is co-ordination required with WT5.

Observation 3 – Clarification on dependency of work related to Release 20 5G-A and RAN.

The proposal is made to clarify in note X5 that the scope and KIs shall be reviewed and updated in Q1 2026 based on the outcome of R20 5G-A SA2 sensing study outcome and RAN decisions.

**** First Change ****

# Annex A.X. WT#4 Scope

WT#4: Study the Integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported including 3GPP and non-3GPP sources of sensing data.

The following aspects will be studied:

a)	Architecture and functional support for sensing.

b)	Service authorization and revocation.

c)	Discovery, (re-)selection and coordination of Sensing Entities.

d)	Configuration parameters and policy provisioning to support sensing service, e.g., to the selected Sensing Entities, Sensing function, etc.

e)	Collection, distribution of sensing related data and exposure of sensing result.

f)	Sensing continuity in consideration of mobility of sensing entities (i.e. UE) and/or target objects.

g)	Support fusion of sensing data to generate sensing results.

NOTE X1:	WT#4 needs to consider the work and conclusions of R20 FS_Sensing_ARC on gNB-based sensing.

NOTE X2: 	The scope of WT#4 includes sensing modes and their selection, where sensing may involve RAN node and UE.

NOTE X3:	Aspects a) and e) requires coordination with WT#5.

NOTE X4:	Coordination with RAN WG for RAN aspects, SA3 for privacy and consent aspects, and SA5 for OAM aspects will be needed.

NOTE X5: 	The scope of WT#4 will be aligned with RAN for 6G. SA2 will evaluate the situation in Q1 2026, for possible update to scoping and KIs based on R20 FS_Sensing_ARC outcome and RAN decisions and accordingly start solution discussions.

**** Second Change (all text new) ****

# 5.X. Key Issues

### 5.X.1	Description

This Key Issue studies the following with respect Integration of Sensing and Communication over 3GPP Access:

a)	Architecture and functional support for sensing.

b)	Service authorization and revocation.

c)	Discovery, (re-)selection and coordination of Sensing Entities.

d)	Configuration parameters and policy provisioning to support sensing service, e.g., to the selected Sensing Entities, Sensing function, etc.

e)	Collection, distribution of sensing related data and exposure of sensing result.

f)	Sensing continuity in consideration of mobility of sensing entities (i.e. UE) and/or target objects.

g)	Support fusion of sensing data to generate sensing results.

NOTE X1:	WT#4 needs to consider the work and conclusions of R20 FS_Sensing_ARC on gNB-based sensing.

NOTE X2: 	The scope of WT#4 includes sensing modes and their selection, where sensing may involve RAN node and UE.

NOTE X3:	Aspects a) and e) requires coordination with WT#5.

NOTE X4:	Coordination with RAN WG for RAN aspects, SA3 for privacy and consent aspects, and SA5 for OAM aspects will be needed.

NOTE X5: 	The scope of WT#4 will be aligned with RAN for 6G. SA2 will evaluate the situation in Q1 2026, for possible update to scoping and KIs based on R20 FS_Sensing_ARC outcome and RAN decisions and accordingly start solution discussions.

**** End of Changes ****