3GPP TSG-SA WG2 Meeting #171	S2-2509322

# Wuhan, China; 13th – 17th October 2025									(revision of S2-2509047)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1680 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure update for optimizing media routing (UE-Satellite-UE communication) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the IMS call setup procedure for UE–satellite–UE communication specified in TS 23.228, upon receiving an INVITE from the originating P-CSCF, the terminating P-CSCF queries the 5GC for Access Network Information in order to obtain the serving satellite identifier of the terminating UE. However, when the terminating UE is in IDLE state, the terminating P-CSCF may not receive the updated satellite identifier corresponding to the satellite actually serving the UE. This situation is not addressed by the current specification and may result in incorrect or outdated satellite information being used during call setup. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Re-ordering the signaling sequence such that the SIP INVITE request is forwarded to MT UE before determining whether to perform UE-Satellite-UE communication at the terminating side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Call setup may fail or service continuity may be degraded due to outdated satellite identifiers for IDLE UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---First Change---

## AE.5.1	At call setup

When the originating P-CSCF receives the initial INVITE request from the originating UE using regenerative NR satellite access, it selects the IMS-AGW on ground and triggers media negotiation by sending SDP offer to the terminating side indicating that satellite access is used on the originating side. If the terminating P-CSCF activates UE-satellite-UE communication, the originating P-CSCF re-selects an IMS-AGW on satellite. See Figure AE.5.1-1 that describes the procedure. In the procedure, UE A and P-CSCF A represent originating side function or entity, UE B and P-CSCF B represent terminating side function or entity.

NOTE 1:	To support the UE-satellite-UE communication, P-CSCFs (AF) needs to support N5 interface for AF influence on traffic routing and user plane management event subscription/notification.

Figure AE.5.1-1: Session establishment procedure with activation of optimizing media routing

1.	UE A sends a SIP INVITE request, containing an initial SDP offer, to P-CSCF A. The access information in the SIP INVITE request indicates that the UE is accessing from NR satellites access as defined in TS 24.229 [10a].

2.	After receiving the SIP INVITE request indicating NR satellite access, if UE A is not roaming and the P-CSCF A supports UE-Satellite-UE communication, the P-CSCF requests Access Network Information, which includes an indication for requesting satellite identifier information, for UE A from the PCF as described in TS 23.503 [95] via Npcf_PolicyAuthorisation_Create service operation; the event identifier set to Access Network Information Notification. The PCF requests Access Network Information, which includes the identifier of UE A's serving satellite, from the SMF via Npcf_SMPolicyControl_UpdateNotify service operation; the PCRT trigger set to Access Network Information report. Then the PCF receives the Access Network Information via Npcf_SMPolicyControl_Update service operation as described in TS 23.502 [94] and TS 23.503 [95]. This satellite identifier is subsequently conveyed to the P-CSCF in the Access Network Information Notification via Npcf_PolicyAuthorization_Notify service operation.

3.	The P-CSCF A allocates an IMS-AGW on the ground as default. P-CSCF A interacts with IMS-AGW on the ground to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 4-20 are performed if the UE-Satellite-UE communication is possible based on step 2 (i.e. UE A is using NR regenerative payload satellite access and is not roaming). Otherwise, procedures for IMS user plane traffic routing with IMS-AGW on ground are performed (details have been omitted from the diagram).

4.	P-CSCF A updates the SIP INVITE request inserting the identifier of the satellite serving UE A and updates SDP offer with the IMS-AGW transport addresses allocated in step 3. P-CSCF A forwards the updated SIP INVITE request to the terminating side via IMS core (details have been omitted from the diagram).

5.	The P-CSCF B allocates an IMS-AGW on the ground as the default. P-CSCF B interacts with IMS-AGW on the ground to allocate transport resources based on the mechanism defined in TS 23.334 [74].

6-8.	P-CSCF B forwards the updated SIP INVITE request to UE B. UE B returns an 18X response with SDP Answer as a normal SIP message routing procedure.

9.	After receiving the 18X response with SDP Answer, if UE B is not roaming and the P-CSCF B supports UE-Satellite-UE communication, the P-CSCF B requests Access Network Information, which includes an indication for requesting satellite identifier information, for UE B from PCF as described in TS 23.503 [95] via Npcf_PolicyAuthorization_Create service operation; the event identifier set to Access Network Information Notification. The PCF obtains Access Network Information, which may include an identifier of the satellite serving UE B, from the SMF via Npcf_SMPolicyControl_UpdateNotify; the PCRT trigger set to Access Network Information report and Npcf_SMPolicyControl_Update service operations as described in TS 23.502 [94] and TS 23.503 [95]. This satellite identifier, if received, is subsequently conveyed to the P-CSCF in the Access Network Information Notification via Npcf_PolicyAuthorization_Notify service operation.

NOTE 2:	P-CSCF B can decide to include an indication for requesting satellite identifier information based on e.g. access information in the SIP REGISTER request (or other SIP messages) which indicates that UE B is accessing from NR satellites access as defined in TS 24.229 [10a].

10.	Based on the Access Network Information received at step 9, if UE B is using regenerative payload satellite access and if the two satellites identified in steps 4 and 9 are the same or connected with ISL(s), P-CSCF B activates the UE-Satellite-UE communication in IMS.

P-CSCF B shall select an IMS-AGW on satellite serving UE B.

NOTE 3:	How the terminating P-CSCF uses the satellite identifiers to determine whether the two satellites are connected with ISL(s) and are equipped with UPF and IMS-AGW is based on implementation or operators' policy e.g. by querying transport network system or based on node level information reported by the IMS-AGW deployed on the satellites.

Steps 10a-20 are performed if the UE-Satellite-UE communication is activated. Otherwise, SIP message routing procedures with IMS-AGW on the ground are performed after P-CSCF B selects a ground IMS-AGW for usage (details have been omitted from the diagram).

10a-10c.	Based on the decision in step 10, P-CSCF B interacts with IMS-AGW onboard the satellite to allocate transport resources based on the mechanism defined in TS 23.334 [74]. P-CSCF B also releases the IMS-AGW on the ground.

11. P-CSCF B instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service operation. To enable the SMF to insert a UL CL and L-PSA UPF for IMS PDU session to enable UE-Satellite-UE communication, the P-CSCF B includes the following parameters via the request to PCF based on Application Function influence on traffic routing mechanism described in clause 5.6.7 of TS 23.501 [93] and clause 4.3.6 of TS 23.502 [94]:

-	A flow description information which includes the IP address of the IMS-AGW onboard the satellite;

-	A DNAI associated with the IMS-AGW onboard the satellite and optionally corresponding N6 traffic routing information.

NOTE 4:	DNAI value can be derived from the satellite identifier as per operator policy and implementation.

11a.	To be notified of satellite change and perform IMS-AGW relocation at change of satellite as described in clause AE.5.2, P-CSCF B subscribes to notification of User Plane management events (i.e. UP path change) via Npcf_PolicyAuthorization_Update service operation to the PCF, which in turn subscribes to the events from SMF via Npcf_SMPolicyControl_UpdateNotify service operation and receives the event notifications from the SMF via Nsmf_EventExposure_Notify service operation as specified in clause 4.3.6 of TS 23.502 [94]. The P-CSCF B inserts the following parameters in its subscription request (i.e. Npcf_PolicyAuthorization_Update service operation) as specified in clause 5.2.5.3.3 of TS 23.502 [94]:

-	Early and late notifications about UP path management events and acknowledgement of notifications about UP path management events for the early notifications.

NOTE 5:	If Rx is used by P-CSCF, implicit subscription is assumed as part of PCC rules setting.

12. P-CSCF B updates the 18X response to carry the identifier of the satellite serving UE B to indicate that UE-Satellite-UE communication is activated. Then, P-CSCF B returns the updated 18X response to originating side via IMS core (details have been omitted from the diagram).

13. Based on the identifier of the satellite serving UE B in step 12, P-CSCF A determines to perform UE-satellite-UE communication and select an IMS-AGW onboard the satellite.

13a-13c.	P-CSCF A interacts with IMS-AGW onboard the satellite to allocate transport resources for both IMS access and IMS Core network sides. P-CSCF A also releases the IMS-AGW on ground.

14. P-CSCF A instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service operation. To indicate to the SMF to insert a UL CL and L-PSA UPF for the IMS PDU session to enable UE-Satellite-UE communication in IMS, P-CSCF B should send following parameters via the request to PCF based on Application Function influence on traffic routing mechanism described in clause 5.6.7 of TS 23.501 [93] and clause 4.3.6 of TS 23.502 [94]:

-	A flow description information which contains the IP address of the IMS-AGW onboard satellite;

-	A DNAI associated with the IMS-AGW onboard satellite and optionally corresponding N6 traffic routing information.

NOTE 6:	DNAI value can be derived from the satellite identifier as per operator policy and implementation.

14a.	To be notified of satellite change and perform IMS-AGW relocation at change of satellite as described in clause AE.5.2, P-CSCF A subscribes to notification of user plane management events (i.e. UP path change) via Npcf_PolicyAuthorization_Update service operation to the PCF, which in turn subscribes to the events from SMF via Npcf_SMPolicyControl_UpdateNotify service operation and receives the event notifications from the SMF via Nsmf_EventExposure_Notify service operation as specified in clause 4.3.6 of TS 23.502 [94]. The P-CSCF A inserts the following parameters in its subscription request (i.e. Npcf_PolicyAuthorization_Update service operation) as specified in clause 5.2.5.3.3 of TS 23.502 [94]:

-	Early and late notifications about UP path management events and acknowledgement of notifications about UP path management events for the early notifications.

NOTE 7:	If Rx is used by P-CSCF, implicit subscription is assumed as part of PCC rules setting.

15. P-CSCF A updates the SDP answer in the 18X response with the IMS-AGW transport addresses allocated in step 13 and forwards the updated 18X response to UE A.

16. UE A acknowledges the 18X Response and sends the Response Confirmation to P-CSCF A.

17-18. P-CSCF A forwards the Response Confirmation to P-CSCF B. The Response Confirmation includes SDP offer modified/generated by P-CSCF A which contains the IMS-AGW transport address obtained in step 13. Upon receiving the SDP offer, P-CSCF B updates the allocated IMS-AGW on the satellite for UE B with the transport address in the SDP offer.

19. P-CSCF B sends the Response Confirmation to UE B. The Response Confirmation includes the SDP offer modified/generated by P-CSCF B, which contains the IMS-AGW transport address obtained in step 10.

20.	Procedure continues to setup the call as defined in clauses 5.6 and 5.7.

## ---End of Changes---