3GPP TSG-WG SA2 Meeting #172	S2-2509883

Dallas, US, 17 - 21 Nov 2025	(revison of S2-250)

Source:	OPPO

Title:	Proposed Interim Agreements and Conclusions for KI#1

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim Agreements and Conclusions for KI#1.

# 1	Discussion

See DP in S2-2509886.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-14 V1.1.0.

* * * * Start of Changes * * * *

### 7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: One new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. The SF may contain Sensing Control functionality (SCF) and Sensing Processing functionality (SPF). Two endpoints may be supported between SF and gNB respectively for control signalling and sensing data transport.

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: No dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

Principle 4: A Gateway SF may be deployed to store sensing authorization information and to interact with trusted AF for sensing authorization.

NOTE X:	Other functionalities supported by the Gateway SF will be developed during normative phase.

Principle 5: Both the control signalling and sensing data are transported directly between gNB and SF.

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

* * * * Next Change * * * *

## 8.1	Conclusions of KI#1

One new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. The SF may contain Sensing Control functionality (SCF) and Sensing Processing functionality (SPF). Two endpoints may be supported between SF and gNB respectively for control signalling and sensing data transport.

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

No dedicated storage NF is needed to store the Sensing data and Sensing result.

In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

A Gateway SF may be deployed to store sensing authorization information and to interact with trusted AF for sensing authorization.

NOTE X:	Other functionalities supported by the Gateway SF will be developed during normative phase.

Both the control signalling and sensing data are transported directly between gNB and SF.

* * * * End of Changes * * * *