3GPP TSG-WG SA2 Meeting #172	S2-2509993

Dallas, US, 17th – 21st Nov, 2025

Source:	CEWiT

Title:	Conclusion proposal for KI#5

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: To provide conclusions for KI#5 to be incorporated into Clause 8.

# 1.	Discussion

This contribution proposes the conclusions for KI#5. The conclusions are drafted based on the proposals from different companies for KI#5 in 3GPP TR 23.700-14 V1.1.0.

# 2.	Text Proposal

It is proposed to capture the following changes in TR 23.700-14 in Clause 8.

# * * * * First change * * * *

# 8	Conclusions

## 8.e	Conclusion for KI#5: Sensing Result Exposure

# The following principles are agreed as conclusions for KI#5 and is recommended for normative work:

- The sensing service consumer can be AF(s) in the trusted domain within the Core network and 3rd party AF(s) authorized by NEF.

- The sensing service request can contain Target Sensing Area, AF ID, Sensing Service Type (e.g. Object Detection, Object Tracking etc.), Type of Event Reporting (One Time, Periodic, Event-Triggered), Sensing Contextual information and Quality requirements of sensing result.

NOTE 1: The supported list of conditions for Event-Triggered type of reporting shall be specified by RAN3, during the normative phase.

- The SeMF provides the sensing results or configures the SPF to deliver the sensing results,

- to the NEF for 3rd party AF(s) authorized by NEF using the exposure framework

- directly to the AF(s) in trusted domain

- The sensing result can contain Object Presence Detection i.e. Yes or No, Sensed Object count, Sensed Object definition and Sensing Contextual information containing the Quality of Sensing Result

- The exposed sensing results can allow the AF to request for a subsequent sensing service e.g., to track specific sensed object(s)

NOTE 2: The parameters that can be considered between the Sensing Service Consumer and the SeMF can be updated after alignment of some parameters (e.g., sensing service type) with RAN WGs as required.

NOTE 3: Privacy and security aspects related to sensing result exposure shall be coordinated with SA3.

NOTE 4: Charging and OAM implications of exposing sensing results shall be coordinated with SA5.

# * * * * End of changes * * * *