3GPP SA WG2 Meeting #172	S2-2509995

# 13 - 17 November, 2025, Dallas, USA.

Source: 	IPLOOK

Title: 	KI#5 Solution: Dynamic Inter-SMF Coordination for UE-SAT-UE Communication via Onboard UPFs

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#5 to support UE-SAT-UE communication for non-IMS services via UPFs deployed only onboard NGSO satellites. The solution introduces a dynamic inter-SMF coordination mechanism to efficiently establish optimal communication paths between UEs, addressing the scalability challenges in large-scale satellite networks where group members may be served by different SMF instances within an SMF Set.

1	Discussion & Proposals

This contribution discusses the key issue of supporting UE-to-UE communication entirely via UPFs onboard NGSO satellites for non-IMS services. This proposal enhances the architecture by introducing a dynamic coordination mechanism among SMF instances within the same SMF Set.

2	Proposal

It is proposed to capture the solution proposal for the inclusion in the TR 23.700-19 v1.1.0.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 |
| #1 | X | X |  |  |  |
| #2 | X |  |  |  |  |
| #3 | X |  |  |  |  |
| #4 | X | X | X | X |  |
| #5 | X |  |  |  |  |
| #6 | X |  |  |  |  |
| #7 | X | X |  |  |  |
| #8 | X |  |  |  |  |
| #9 | X |  |  |  |  |
| #10 | X |  |  |  |  |
| #11 | X |  |  |  |  |
| #12 |  | X |  |  |  |
| #13 |  | X |  |  |  |
| #14 |  | X |  |  |  |
| #15 |  | X |  |  |  |
| #16 |  | X |  |  |  |
| #17 |  | X |  |  |  |
| #18 |  | X |  |  |  |
| #19 |  | X |  |  |  |
| #20 |  | X |  |  |  |
| #21 |  |  | X |  |  |
| #22 |  |  | X |  |  |
| #23 |  |  | X |  |  |
| #24 |  |  | X |  |  |
| #25 |  |  | X | X |  |
| #26 |  |  |  | X |  |
| #27 | X | X |  |  |  |
| #28 | X |  |  |  |  |
| #29 | X |  |  |  |  |
| #x |  |  |  |  | X |

# * * * * Next change(all new texts)  * * * *

6.x	Solution#x: Dynamic Inter-SMF Coordination for UE-SAT-UE Communication via Onboard UPFs

6.x.0	High-level solution principles

The high-level principles of this solution are:

- The PSA UPF for a PDU Session supporting UE-SAT-UE communication for a 5G VN group shall be deployed onboard the serving NGSO (LEO/MEO) satellite.

- A Primary SMF role is defined for coordination within a 5G VN group. One SMF instance within the SMF Set serving the group shall assume this role.

- A dynamic Inter-SMF Coordination Service is utilized to enable real-time exchange of UE location information (i.e., the serving satellite ID and the N19 tunnel endpoint of its onboard PSA UPF) among SMF instances within the same SMF Set.

- The establishment of direct N19 tunnels between onboard PSA UPFs on different satellites is triggered on-demand, based on the detection of actual communication needs between group members, and is facilitated by the Primary SMF.

- Session and Service Continuity for PDU Sessions with onboard PSA UPFs is achieved through SSC Mode 3 procedures, enhanced with make-before-break PSA relocation during satellite handovers.

- This solution is applicable to non-IMS services (e.g., 5G-LAN) using IP or Ethernet PDU Session types and supports unicast communication.

6.x.1	Description

This solution enhances the 5G architecture to support efficient UE-SAT-UE communication for non-IMS services in scalable NGSO satellite networks. It optimizes the user plane path by deploying the PSA UPF onboard the satellite and introduces a robust control plane mechanism for dynamic path management.

The key architectural enhancements include:

- The PSA UPF is deployed on the satellite, anchoring the PDU session and enabling direct user plane switching in space.

- For each 5G VN group, one SMF within the serving SMF Set acts as the Primary SMF. It maintains a dynamic registry of group members' active sessions and their associated onboard PSA UPF locations.

- A new service-based interface among SMFs within the same SMF Set allows for the registration and query of UE session location information.

- Traffic Handling:

- Case 1 - UEs under the same satellite: Traffic is switched locally by the common onboard PSA UPF.

- Case 2 - UEs under different satellites (with ISL): A direct N19 tunnel is established between the respective onboard PSA UPFs. The tunnel establishment is coordinated by the Primary SMF via the Inter-SMF Coordination Service.

- Case 3 - Fallback: If the optimal path cannot be established (e.g., ISL unavailable), communication falls back to the terrestrial network via the N6 interface.

- Fig 6.x.1-1 Solution Architecture Overview

6.x.2	Procedures

6.x.2.1	Enhancement of 5G VN Group Data

The 5G VN group data in UDR is enhanced with a new attribute. This boolean attribute indicates whether the group is authorized to utilize the fully onboard PSA UPF architecture for UE-SAT-UE communication. The SMF retrieves this attribute during PDU Session Establishment.

6.x.2.2	PDU session establishment for UE-Sat-UE communication

During PDU Session Establishment for a UE belonging to a 5G VN group, the SMF retrieves the group data from UDM/UDR.

If the attribute is set to TRUE, the SMF selects an UPF deployed on the serving satellite to act as the PSA UPF for this session.

The SMF registers the UE's identity, its serving satellite ID, and the N19 tunnel endpoint address of its onboard PSA UPF with the Primary SMF for its 5G VN group.

6.x.2.3	Dynamic Inter-SMF Path Establishment

- Fig 6.x.2.3-1 Dynamic Path Establishment Procedure

1. UE1 sends a data packet destined for UE2.

2. The onboard PSA UPF A receives the packet from UE1 and checks its PDRs. The packet matches a pre-configured "catch-all" PDR due to no specific forwarding rule existing for the destination UE2

3. PSA UPF A sends a PFCP Session Report (Uplink Data Report) to its controlling SMF (SMF A), including the source and destination IP addresses.

4. SMF A analyzes the report and correlates the destination IP address with the 5G VN group membership data. SMF A identifies that UE2 is a member of the same VN group but no optimal direct path currently exists. SMF A decides to initiate the Inter-SMF coordination procedure.

5. SMF A sends a query to the Primary SMF via the Inter-SMF Coordination Service to request the location information of UE2.

6. The Primary SMF forwards the query to SMF B, which is the SMF serving UE2.

7. SMF B responds to the Primary SMF with UE2's PSA UPF information, including the serving satellite ID (Satellite B) and the N19 tunnel endpoint address of PSA UPF B.

8. The Primary SMF returns the target PSA UPF information to SMF A. SMF A sends a PFCP Session Modification Request to PSA UPF A, commanding it to add N19 tunnel FARs targeting PSA UPF B.

9. PSA UPF A responds with a PFCP Session Modification Response indicating successful configuration.

10. SMF A notifies SMF B that the tunnel configuration on UE1's side is complete and coordinates the tunnel establishment.

11. SMF B sends a PFCP Session Modification Request to PSA UPF B, commanding it to add N19 tunnel FARs targeting PSA UPF A.

12. PSA UPF B responds with a PFCP Session Modification Response indicating successful configuration. The direct N19 tunnel between PSA UPF A and PSA UPF B is now established.

13. Subsequent data packets from UE1 to UE2 are forwarded by PSA UPF A via the N19 tunnel.

14. PSA UPF B receives the packets through the N19 tunnel and forwards them to UE2.

15. UE2 receives the data packets from UE1, completing the end-to-end communication via the established satellite-based path.

6.x.2.4	Handover with Onboard PSA Relocation

Fig 6.x.2.4-1 Handover with Onboard PSA Relocation

1. The source gNB (on the current serving satellite) detects the need for handover and sends a handover request to the SMF, including the target satellite ID.

2. SMF sends a PFCP Session Establishment Request to create a new PSA UPF context on the target satellite. The target PSA UPF responds with a PFCP Session Establishment Response confirming context creation.

3. SMF sends a PFCP Session Modification Request to the source PSA UPF, instructing it to begin duplicating downlink traffic to the target PSA UPF while continuing to serve the UE. The source PSA UPF responds with a PFCP Session Modification Response confirming the duplication configuration.

4. The UE performs synchronization and accesses the target satellite through standard radio handover procedures.

5. The target gNB sends a Path Switch Request to the SMF, indicating the UE has successfully accessed the target satellite.

6. SMF sends a PFCP Session Modification Request to the target PSA UPF, activating it to begin sending buffered traffic to the UE. The target PSA UPF responds with a PFCP Session Modification Response confirming activation.

7. SMF sends a PFCP Session Modification Request to the source PSA UPF, instructing it to stop sending traffic directly to the UE but continue duplication to the target PSA for any remaining packets. The source PSA UPF responds with a PFCP Session Modification Response confirming the configuration update.

8. Uplink traffic from peer UEs now routes through the target PSA UPF to the UE.

9. Downlink traffic flows through the newly activated path via the target PSA UPF to the UE.

10. After confirming stable operation on the new path, SMF sends a PFCP Session Deletion Request to the source PSA UPF to release resources. The source PSA UPF responds with a PFCP Session Deletion Response confirming resource release.

6.x.3	Impacts on services, entities, and interfaces

Services:

- -	Enables low-latency, high-efficiency UE-to-UE communication for non-IMS services (e.g., 5G-LAN) within satellite networks.

- -	Supports unicast communication only.

Entities:

- SMF:

- -	Primary SMF functionality is required for coordinating path establishment within a 5G VN group.

- -	Supports the Inter-SMF Coordination Service for real-time exchange of UE/UPF location information.

- -	Enhances PDU Session Management to select onboard PSA UPFs and trigger dynamic N19 tunnel setup.

- -	Manages SSC Mode 3-based PSA relocation with make-before-break logic during satellite handovers.

- UPF (Onboard):

- -	Acts as the PSA UPF onboard the satellite.

- -	Supports establishment of N19 tunnels with peer onboard PSA UPFs based on SMF instructions.

- -	Supports packet buffering and duplication during PSA relocation procedures.

- UDM/UDR:

- -	Stores the enhanced 5G VN group data.

- AMF:

- -	Reports the serving satellite ID to the SMF during relevant procedures (e.g., PDU Session Establishment, Handover).

Interfaces:

- -	N4: Enhanced to support SMF commands for dynamic N19 tunnel establishment between onboard PSA UPFs and for controlling traffic buffering/duplication during relocation.

- -	N19: Used for user plane forwarding between onboard PSA UPFs on different satellites.

- -	New "Inter-SMF Coordination Service": A new service-based interface (e.g., based on Nsmf service) between SMF instances within an SMF Set, used for querying and exchanging UE serving satellite and UPF tunnel endpoint information.

# * * * * End of changes * * * *