SA WG2 Meeting #172	S2-2510101

17 - 21 Nov, 2025, Dallas ,United States

Source:	IPLOOK

Title:	KI #5, New Sol: UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: Introduce a new solution to key issue#5 in TR 23.700-19.

1. Introduction

This contribution proposes a new solution to Key Issue #5:

-	UE-SAT-UE communication via UPF only onboard satellite for Non-IMS services.

2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-19.

* First change *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 |
| #1 | X | X |  |  |  |
| #2 | X |  |  |  |  |
| #3 | X |  |  |  |  |
| #4 | X | X | X | X |  |
| #5 | X |  |  |  |  |
| #6 | X |  |  |  |  |
| #7 | X | X |  |  |  |
| #8 | X |  |  |  |  |
| #9 | X |  |  |  |  |
| #10 | X |  |  |  |  |
| #11 | X |  |  |  |  |
| #12 |  | X |  |  |  |
| #13 |  | X |  |  |  |
| #14 |  | X |  |  |  |
| #15 |  | X |  |  |  |
| #16 |  | X |  |  |  |
| #17 |  | X |  |  |  |
| #18 |  | X |  |  |  |
| #19 |  | X |  |  |  |
| #20 |  | X |  |  |  |
| #X |  |  |  |  | x |

* Second change  *

6.X	Solution #X: UE-to-UE Communication for Non-IMS Services Based on 5G VN Group and Onboard UPF Local Switching

6.X.0	High level principles

This solution is to address Key Issue #5, the following principles are followed in this solution:

-	VN Group as the Service Foundation Principle: Fully reuse and leverage the 5G VN Group communication framework defined by 3GPP standards, modeling the UE-to-UE satellite communication as an instance of a VN Group service.

-	Onboard Local Switching Principle: To achieve low latency and reduce feeder link load, user plane data between UEs belonging to the same VN Group should be locally switched and forwarded directly by the serving onboard UPF.

-	Control Plane Centralization Principle: Core control plane functions such as VN Group management, session establishment, and policy control remain the responsibility of terrestrial core network functions (e.g., SMF, PCF, UDM), ensuring architectural consistency, security, and manageability.

-	No UE Impact Principle: Strictly adhere to the study assumption of Key Issue #5. The UE operates as a standard terminal within a VN Group, complying entirely with existing 5G-LAN standards without requiring any changes.

-	Intra-PLMN, Intra-Group Communication Principle: The solution focuses on communication between UEs within the same PLMN, covered by the same satellite or constellation (connected via ISLs), and subscribed to the same VN Group.

6.X.1	Description

The core concept of this solution is: Instantiating the "UE-SAT-UE communication service" as a specific 5G VN Group service, and enhancing the SMF and onboard UPF to enable the recognition and local switching of communication data within this VN Group at the onboard UPF.

Figure 6.x.1-1: VN Group-based Local Switching Architecture via Onboard UPF

Key Functions:

VN Group Subscription: The operator creates a dedicated VN Group for the UE-to-UE satellite communication service and subscribes the user UEs to this group.

VN Group and Location-based UPF Selection: During PDU Session establishment, the SMF selects an onboard UPF with local switching capability as the PDU Session Anchor, based on a combination of the UE's subscribed VN Group information and its current serving satellite ID. This ensures UEs of the same VN Group under the same satellite coverage area are anchored to the same onboard UPF.

VN Group-Aware Forwarding Rules: The SMF uses the N4 interface to install Packet Detection Rules (PDRs) in the onboard UPF, based on VN Group identifiers and target UE addresses. The SMF also provides Forwarding Action Rules (FARs) instructing the UPF to perform a "Local Switch" action if the destination address of a packet belongs to another UE within the same VN Group that is also anchored at this UPF.

- X.2	Procedures

6.X.2.1     PDU Session Establishment Procedure for a VN Group Member

The procedure is shown in Figure 6.x.2.1-1.

Figure 6.x.2.1-1: PDU Session Establishment Procedure for a VN Group Member

- UE1 initiates a PDU Session Establishment request, using the DNN/S-NSSAI configured for the UE-to-UE service.

- The AMF selects a terrestrial SMF. The SMF retrieves UE1's subscription data from the UDM, which includes the list of VN Groups it belongs to.

- The SMF, using the obtained VN Group information and UE1's current serving satellite ID, selects a suitable onboard UPF from the UPF pool to serve as the PDU Session Anchor.

- The SMF establishes a session context on the onboard UPF via the N4 interface and provisions initial PDRs/FARs. These include rules for detecting traffic destined for other UEs within the VN Group and forwarding actions pointing to local switching.

- The SMF maintains a dynamic internal context for this VN Group, recording the mapping of "UE1 - Serving Satellite ID - Onboard UPF Address".

6.X.2.2     UE-to-UE Data Communication Procedure (UE1 sending data to UE2 within the same group)

The procedure is shown in Figure 6.x.2.2-1.

Figure 6.x.2.1-1: UE-to-UE Data Communication Procedure

- UE1 sends a data packet with the destination address set to UE2.

- The onboard UPF receives the packet and matches it against the PDRs installed by the SMF, identifying this traffic as belonging to the specific VN Group and destined for UE2.

- The onboard UPF checks its local context (configured by the SMF when UE2 attached), confirming that UE2 is also anchored at this UPF and belongs to the same VN Group.

- The UPF executes the "Local Switch" action, directly forwarding the data packet via the N3 tunnel leading to UE2's serving gNB, which then delivers it to UE2.the UE provides location to MME.

6.X.3	Impacts to Services, Entities and Interfaces

SMF:

-	Needs capability for intelligent UPF selection based on (VN Group + Serving Satellite ID);

-	Requires enhanced VN Group member management to dynamically track member anchoring relationships with onboard UPFs;

-	Must be able to generate and provision N4 rules supporting intra-VN Group local switching.

Onboard UPF:

-	Needs to support packet detection based on VN Group identifiers and local switching logic, maintaining local forwarding information for UEs within the group.

PCF:

-	Could formulate authorization and QoS policies specific to onboard local switching.

N4 Interface:

-	Needs new or extended PDRs (e.g., adding VN Group matching criteria) and FARs (e.g., defining a "Local Switch" action). New Information Elements might be needed to synchronize UE address information within the VN Group.

* End of changes *