SA WG2 Meeting #S2-172	S2-2510202

17 - 21 November, 2025, Dallas, USA

# Title:	Reply LS on indication of LMF-based AI/ML Positioning

# Response to:	C3-254615, S2-2508962, LS on indication of LMF-based AI/ML Positioning

# Release:	Rel-19

# Work Item:	AIML_CN

# Source:	SA2

# To:	CT3

# Cc:

# Contact Person:	belen.pancorbo@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

# 1	Overall description

SA2 thanks CT3 for the LS on indication of LMF-based AI/ML Positioning. SA2 has been aware of that CT3 could not reach consensus on the stage-3 modelling on indication of requesting a model for LMF-based AI/ML Positioning and would like to answer CT3 question as below:

CT3 Question: CT3 wants to confirm whether the ML model for LMF-based AI/ML positioning has impact on the following APIs: Nnwdaf_MLModelTraining API, Nnwdaf_MLModelMonitor API, Nadrf_MLModelManagement API, and VFL APIs?

SA2 Answer: SA2 confirm that ML model for LMF-based AI/ML positioning has no impact on the Nnwdaf_MLModelTraining API, Nnwdaf_MLModelMonitor API, Nadrf_MLModelManagement API, and VFL APIs with below information:

TS 23.288 v.19.4.0 clause 6.2F Procedure for ML Model Training and clause 7.10 Nnwdaf_MLModelTraining Service definition do not include any description related to LMF-based AI/ML positioning and the mandatory required Analytics ID does not apply to LMF-based AI/ML positioning.

TS 23.288 v.19.4.0 clause 6.2E.4 Procedure for MTLF-based AI/ML model performance monitoring for LMF-based AI/ML Positioning defined that the LMF may include an ML Model identifier and an ML Model Accuracy Threshold for ML model performance monitoring in Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelInfo_Request, Not in Nnwdaf_MLModelMonitor API.

Also no description on the ML model for LMF-based AI/ML positioning exists in the VFL procedures and related services initiation in Nadrf_MLModelManagement API and VFL APIs in TS 23.288 v.19.4.0 and in TS 23.273 v.19.4.0 clause 5.18.0 NOTE 4 indicated that other NWDAF discovery parameters listed in clause 5.2 of TS 23.288 [37] such as Analytics ID, FL capability type and related time period, S-NSSAI or any roaming capabilities are not included by LMF.

# 2.	Actions

To CT3:

ACTION: SA2 kindly asks CT3 to take the above information into account.

# 3.	Date of Next SA2 Meetings

SA2#173		09 – 13 February, 2026		Goa, IN

SA2#174		13 – 17 April, 2026	 		Malta, MT