3GPP SA WG2 Meeting #172	S2-2510407

17 - 21 November, 2025, Dallas, USA	(revision of S2-25xxxxx)

Source:	LG Electronics

Title:	KI#2: Solution Update: Autonomous registration and data transfer from AIoT Device

Document for:	Approval

Agenda item:	20.5.1

Spec:	3GPP TR 23.700-30

Version:	0.2.0

Work Item / Release:	FS_AmbientIoT-ARC_Ph2 / Rel-20

Abstract of the contribution: This paper proposes updates to solution#9 in the TR 23.700-30.

1	Discussion

Regarding Key Issue#2: Support of DO-A Capable AIoT Devices, this paper proposes updates to solution #9: Autonomous registration and data transfer from AIoT Device in the TR 23.700-30 V0.2.0.

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-30.

* * * * Start of 1st Change * * * *

6.9	Solution #9: Autonomous registration and data transfer from AIoT Device

6.9.0	High-level solution Principles

The following principles apply:

-	The Reader broadcasts system information that defines contention-based uplink resources for initial access of AIoT Device based on the DO-A activation request from the AIOTF.

NOTE:	The NG-RAN receiving DO-A activation request from the AIOTF for broadcasting access parameters needs coordination with RAN WGs.

-	An AIoT Device, upon power-on or sensing an event to report, uses the broadcasted information to autonomously register and once registered to the network, it can transmit data to the AF via the network.

-	The AIOTF is enhanced to support the AF request for the DO-A activation, handle registration and data transfer request from the DO-A capable AIoT Device forwarded by the Reader, authenticate and authorize the request, create device context, respond to the AIoT Device with the assigned temporary ID(i.e. T-ID).

6.9.1	Description

This solution addresses Key Issues #2 by defining the procedure for the DO-A capable AIoT Device applicable to both Topology 1 (using RAN Readers) and Topology 2 (using UE Readers). It enables a DO-A capable AIoT Device to trigger its own registration with the network to autonomously inform the network of its presence without a prior network command and to send data to the designated AF based on the AF requested DO-A activation.

6.9.2	Procedures

The DO-A procedure can be initiated by the AF to request the activation for one or more DO-A capable AIoT Devices or a group of DO-A capable AIoT Devices in an area and/or Time Duration to execute the autonomous registration and data transfer.

The DO-A procedure can be also activated based on configuration in the network. In this case, the AIOTF is configured with information to select NG-RAN node(s) and optionally Readers and send DO-A activation request.

6.9.2.1	DO-A procedure for Topology 1

Figure 6.9.2.1-1: Device initiated Registration and Data Transfer procedure for Topology 1

1.	The AF requests for the AIoT DO-A activation which may include the External Target Area information and/or information about the target AIoT Device(s) and/or Time Duration of DO-A traffic to the NEF.

Editor's note:	How to handle and coordinate the requests from multiple AFs is FFS.

2.	The NEF selects the AIOTF(s) as described in clause 5.3.1 of TS 23.369 [3] and sends the AIoT DO-A activation request from the AF to the selected AIOTF(s).

3.	The AIOTF determines the DO-A activation either based on configuration or the request from the AF and selects NG-RAN node(s) and optionally RAN readers based on the configured NG-RAN node selection information. The AIOTF may also consider the Target Area information and/or information about the target AIoT Device(s) and/or Time Duration for DO-A if received in step 1 and the stored last known RAN Reader information if available e.g. by means of prior Inventory procedure.

4.	The AIOTF sends the AIoT DO-A activation response to the AF via NEF.

5.	The AIOTF sends the DO-A activation request to each selected NG-RAN node, either directly or through the selected AMF which may include the Requested Service Area Information and/or the AIoT Identification Information and/or Time Duration for DO-A.

6.	The NG-RAN allocates/deallocates the RAN Reader with the access parameters related to Random Access considering the Requested Service Area and/or Time Duration of DO-A traffic, which the RAN Reader then, starts/stops broadcasting access parameters related to Random Access for AIoT Device for autonomous transmissions based on the DO-A activation request from the AIOTF.

NOTE 1:	The mechanism for broadcasting access parameters needs coordination with RAN WGs.

7.	The AIoT Device listens to access parameters from the RAN Reader and performs Random Access to establish the AS connection with the RAN Reader.

8.	The AIoT Device transmits the AIoT Registration Request message including the AIoT Device Permanent Identifier. The RAN Reader receives the request and forwards the AIoT Registration Request to the AIOTF directly or via an AMF.

When the privacy protection for the AIoT Device Permanent Identifier applies, the AIoT Device generates T-ID from KAIOT_root using the AIoT Device Permanent Identifier and RANDAIOT_d, and includes the T-ID and RANDAIOT_d in the AIoT Registration Request message. In this case, the AIOTF obtains the AIoT Device Permanent Identifier corresponding to T-ID from the ADM to identify the AIoT device.

NOTE 2:	The security aspects for authentication, authorization and the use of T-ID needs coordination with SA WG3.

9.	The AIOTF validates the AIoT Registration Request message with DO-A capability of the AIoT Device and the target AF information in the AIoT Device Profile Data provisioned, the Target Area information and/or information about the target AIoT Device(s) configured or received in step 1.

If validation fails, the AIOTF issues failure response to the AIoT Device and the rest of the procedure is not performed.

Upon successful, the AIOTF updates the AIoT Device Profile Data in the ADM with the Last known AIOTF information.

The AIOTF retrieves the AIoT Device T-ID from the ADM, associating it with the last known Reader and the target AF and stores the mapping between the RAN Reader ID, AIoT Device T-ID and the AF ID in the AIoT Device context.

10.	The AIOTF sends an AIoT Registration Accept message back to the device via the RAN Reader. The message contains the assigned AIoT Device T-ID for the AIoT Device.

The AIoT Device receives the AIoT Registration Accept message and uses the AIoT Device T-ID for subsequent communication.

11.	The AIoT Device sends the event-driven or periodic AIoT Data Transfer Request message including the assigned AIoT Device T-ID.

The AIOTF validates the request using the AIoT Device Profile and sends the message to the target AF found in the AIoT Device context directly or via NEF. The AIOTF sends the AIoT Data Transfer Response message to the AIoT Device via the RAN Reader including a new AIoT Device T-ID.

6.9.2.2	DO-A procedure for Topology 2

Figure 6.9.2.2-1: Device initiated Registration and Data Transfer procedure for Topology 2

1.	During the registration, the UE indicates the support for the UE Reader Capability to the AMF. The AMF checks if the UE is authorized to act as a Reader based on subscription data. The AMF assigns the UE Reader ID and stores the mapping information between candidate UE Reader(s) and the NG-RAN in the local context.

2.	The AF requests for the AIoT DO-A activation which may include the External Target Area information and/or information about the target AIoT Device(s) and/or Time Duration of DO-A traffic together with an indication for the use of the UE Reader to the NEF.

Editor's note:	How to coordinate and handle requests from multiple AFs is FFS.

3.	The NEF selects the AIOTF(s) as described in clause 5.3.1 of TS 23.369 [3] and sends the AIoT DO-A activation request from the AF to the selected AIOTF(s).

4.	The AIOTF determines the DO-A activation either based on configuration or the request from the AF and selects the AMF considering the Target Area information if received in step 2 or based on local configuration.

The AIOTF, based on the indication for the use of the UE Reader in the request or based on local configuration to select UE reader(s), retrieves/subscribes to the AMF to obtain the available UE Reader(s) for the selected NG-RAN and selects UE Reader(s). The AIOTF may also consider the stored last known UE Reader information if available e.g. by means of prior Inventory procedure.

5.	The AIOTF sends the AIoT DO-A activation response to the AF via NEF.

6.	The AIOTF sends the DO-A activation request to the selected AMF including the Requested Service Area Information and/or the AIoT Identification Information and/or Time Duration of DO-A traffic together with the authorized UE reader information (e.g. UE Reader ID(s)).

The AMF selects serving NG-RAN node corresponds to the UE Reader selected by the AIOTF.

NOTE 1:	The aspects of NG-RAN selection and UE Reader selection depend on the outcome of KI#1 study.

7.	The NG-RAN allocates/deallocates the authorized UE Reader with the access parameters related to Random Access considering the Requested Service Area Information and/or Time Duration of DO-A traffic, which the UE Reader then starts/stops broadcasting access parameters related to Random Access for AIoT Device for autonomous transmissions.

NOTE 2:	The mechanism for broadcasting access parameters needs coordination with RAN WGs.

8.	The AIoT Device listens to access parameters from the UE Reader and performs Random Access to establish the AS connection with the UE Reader.

9.	The AIoT Device transmits the AIoT Registration Request message including the AIoT Device Permanent Identifier to the UE Reader. The UE Reader receives the request and forwards the AIoT Registration Request to the NG-RAN and the NG-RAN, aware of which AIOTF the message from UE Reader is destined by step 6, sends the received request to the AIOTF via an AMF.

When the privacy protection for the AIoT Device Permanent Identifier applies, the AIoT Device generates the T-ID from KAIOT_root using the AIoT Device Permanent Identifier and RANDAIOT_d, and includes the T-ID and RANDAIOT_d in the AIoT Registration Request message. In this case, the AIOTF obtains the AIoT Device Permanent Identifier corresponding to the T-ID from the ADM to identify the AIoT device.

NOTE 3:	The security aspects for authentication, authorization and the use of T-ID needs coordination with SA WG3.

10.	The AIOTF validates the AIoT Registration Request message with DO-A capability of the AIoT Device and the target AF information in the AIoT Device Profile Data provisioned, the Target Area information and/or information about the target AIoT Device(s) configured or received in step 2.

If validation fails, the AIOTF issues failure response to the AIoT Device and the rest of the procedure is not performed.

Upon successful, the AIOTF updates the AIoT Device Profile Data in the ADM with the Last known AIOTF information.

The AIOTF retrieves the AIoT Device T-ID from the ADM, associating it with the last known UE Reader and the target AF and stores the mapping between the UE Reader ID, AIoT Device T-ID and the AF ID in the AIoT Device context.

NOTE 3:	The security aspects for authentication, authorization and the use of AIoT Device T-ID needs coordination with SA WG3.

11.	The AIOTF sends an AIoT Registration Accept message back to the AIoT Device via the AMF, NG-RAN and the UE Reader. The message contains the assigned AIoT Device T-ID for the AIoT Device.

The AIoT Device receives the AIoT Registration Accept message and uses the AIoT Device T-ID for subsequent communication.

12.	The AIoT Device sends the event-driven or periodic AIoT Data Transfer Request message including the assigned AIoT Device T-ID.

The AIOTF validates the request using the AIoT Device Profile and sends the message to the target AF found in the AIoT Device context directly or via NEF. The AIOTF sends the AIoT Data Transfer Response message to the AIoT Device via the UE Reader including a new AIoT Device T-ID.

6.9.3	Impacts on services, entities and interfaces

AIoT Device:

-	Support to initiate the registration request message based on the access parameters broadcasted by the Reader.

-	Support to receive the T-ID to be used for subsequent autonomous message.

Reader (RAN / UE Reader):

-	Support to broadcast access parameters for the DO-A capable AIoT Device(s).

-	Support to receives AIoT Device initiated registration and data transfer messages and forwarding it to the AIOTF.

AIOTF:

-	Support procedures for handling the AF request for the DO-A activation and DO-A capable AIoT Device initiated registration and data transfer.

-	Support to validate requests from DO-A capable AIoT Device, context creation, authentication and authorization, T-ID assignment.

-	Support to route the message between the DO-A capable AIoT Device and the AF.

* * * * End of Changes * * * *