SA WG2 Meeting #S2-172	S2-2510451

17 - 21 November, 2025, Dallas, USA

Title:	[DRAFT] Reply LS on indication of LMF-based AIML Positioning

Response to:	LS on indication of LMF-based AI/ML Positioning (C3-254615 / S2-2509862)

Release:	Release 19

Work Item:	AIML_CN

Source:	SA2

To:	CT3

Cc:

Contact person:	Sihan Cheng

Sihan.Cheng@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA2 thanks CT3 for the LS on indication of LMF-based AI/ML Positioning (C3-254615 / S2-2509862).

SA2 discussed the question of CT3 and concluded the following answer:

Question: Whether the ML model for LMF-based AI/ML positioning has impact on the following APIs: Nnwdaf_MLModelTraining API, Nnwdaf_MLModelMonitor API, Nadrf_MLModelManagement API, and VFL APIs?

Answer: The ML Model for LMF-based AI/ML positioning has no impact on the Nnwdaf_MLModelTraining API, Nnwdaf_MLModelMonitor API or VFL API. For Nadrf_MLModelManagement API, the only impact is the addition of a new consumer (i.e., the LMF) .

For Nnwdaf_MLModelTraining API, it is used for Horizontal Federated Learning as defined in clause 7.10.1 of TS 23.288, and the consumer of this service is limited to the NWDAF containing MTLF. In addition, the NWDAF containing MTLF determines whether to train an ML model for LMF-based AI/ML positioning based on a request from the LMF or an internal trigger, as defined in clause 6.22.4 of TS 23.273. There is no trigger for an NWDAF containing MTLF to train an ML model for LMF-based AI/ML positioning based on a request from another NWDAF containing MTLF. Thus, there is no impact on this API.

For Nnwdaf_MLModelMonitor, it is for NWDAF containing AnLF to register the use and monitoring capability for an ML Model into the model provider NWDAF, i.e. NWDAF containing MTLF, as defined in clause 7.9.1 of TS 23.288. It will be used when the NWDAF containing AnLF is involved (such as providing analytics output or analytics accuracy). For LMF-based AI/ML Positioning, either LMF or NWDAF containing MTLF may perform performance monitoring, instead of NWDAF containing AnLF, as defined in clause 5.18.0 of TS 23.273. Thus, there is no impact on this API.

For Nadrf_MLModelManagement, LMF may invoke the Nadrf_MLModelManagement_RetrievalRequest (Storage Transaction Identifier or one or more unique ML Model identifier(s)) service operation to get the ML Model stored in ADRF, as defined in clause 6.2B.7 of TS 23.288. In this procedure, only Storage Transaction Identifier or ML Model identifier could be used. Thus, the only impact is the addition of a new consumer (i.e., the LMF) due to the ML model for LMF-based AI/ML positioning.

For VFL, it is a federated learning technique without exchanging/sharing local data set, as defined in clause 3.1 of TS 23.288, where the VFL participants are limited to NWDAF and AF. For training an ML model for LMF-based AI/ML positioning, the NWDAF containing MTLF or other training entities can collect the required data. Thus, there is no impact on this API.

2	Actions

To CT3 group

ACTION: 	SA2 kindly asks CT3 to take the above information into account.

3		Dates of next TSG SA WG 2 meetings

TSG-SA2 Meeting #173		09-13 Feb 2026	India

TSG-SA2 Meeting #174		13-17 Apr 2026	Malta