SA WG2 Meeting #172	S2-2510468

17 – 21 November, 2025, Dallas, US

Source:	Samsung

Title:	KI#2 Interim Agreements

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC/Rel-20

Abstract of the contribution: This contribution proposes Interim Agreements for KI#2.

1. Discussion

Rel-20 the system architecture to support DO-A capable Ambient IoT Devices in Topology 1 and Topology 2 can be derived concensus of Solutions and Rel-20 Architecture assumptions and KI agreements.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-30.

* * * Start of first Change (all new text) * * * *

7.1.X	Agreed Principles for KI#2

The following principles apply for system architecture to support DO-A capable Ambient IoT Devices in Topology 1 and Topology 2:

-	The AIoT Device informs the network of its presence autonomously by trigger parameters like received radio signals, sending data indication, power-charged indication. The triggering parameters and the paremeters in the D2R message can be pre-cofigured in the AIoT Device or can be configured by the Operator or by the 3rd party.

-	AIoT Devices need to consider power consumption, and that is not so different in DO-A Capable AIoT Devices.

-	AIoT Device sends data to the AIOTF autonomously, from Device to Reader with Temp ID with security information, from Reader to AIOTF with information pre-configured by OAM in Topology 1 and with information configured during UE Reader registration and authorization procedure in Topology 2.

-	Support for routing the data received by AIOTF from an AIoT Device to an AF is the same as R19 from AF to AIOTF.

-	The Inventory and Command procedures defined in TS 23.369 [3] to support DO-A capable AIoT Devices are the same as in Rel-19, with the only differeicen that the Notify messages from the Device can be generated in DO-A mode.

-	Naiotf, Namf and Nnef interface can be enhanced to to support DO-A capable AIoT Device.

* * * End of Change * * * *