3GPP TSG-WG SA2 Meeting #172	S2-2510484

Dallas, USA, 17th – 21th Nov., 2025

Source:	Lenovo

Title:	Update of interim agreements for FS_Sensing_ARC KI#4

Document for:	Agreement

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This contribution proposed update of interim agreements for KI#4 of FS_Sensing_ARC study.

1. Introduction

Based on the interim conclusion in TR 23.700-14, this paper provides the updated interim agreements on KI#4 on Sensing Data and the Associated Information Collection and Transport

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-14.

# * * * * First change * * * *

7.1.4	Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	Sensing Entity can support one time, periodical Sensing Data to the Sensing Function.

Editor’s Note:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

NOTE:	The architecture aspect of this KI’s interim conclusions will be aligned with KI#1 conclusions.

- There’s direct data connection between Sensing RAN node and Sensing Function. Sensing RAN node reports the sensing data and the associated information to Sensing Function via the direct data connection.

- Sensing Function triggers direct data connection establishment towards Sensing RAN node during the configuration procedure of the Sensing RAN node for the sensing service.

  - Sensing Function exchanges data connection information (e.g. IP address, terminal endpoint info) with the Sensing RAN node directly.

- The direct data connection between Sensing RAN node and Sensing Function is per node. The detailed protocol used for the data connection requires coordination with RAN3.

# * * * * End of changes * * * *