SA WG2 Meeting #172	S2-2510497

17 - 21 November, 2025, Dallas, USA

Source:	Xiaomi

Title:	[WT#1.1] KI on UE-CN interaction for control signalling for 6G

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: The clarification on WT#1.1 on UE-CN interaction for control signalling for 6G.

# 1. Justifications

The contribution provides clarification on WT#1.1 for UE-CN interaction for control signalling for 6G based on the discussion on WT#1.1 during SA2#171(S2-2509819) and further discussion in CC on November 04, 2025.

1)	Scope of WT#1.1

All the content of the WT for WT#1.1 is agreeable expect some update on the NAS definition is needed.

NAS(Non-Access Stratum) is defined as following in TS 21.905,

Non-Access Stratum: Protocols between UE and the core network that are not terminated in the UTRAN.

And, the latest defined of NAS for in WT#1.1 using "the highest stratum of the control plane" as following

Non-Access Stratum: The Non-Access Stratum (NAS) forms the highest stratum of the control plane between the UE and the 6G CN.

However, there can be control signalling of NAS and over NAS. So it is proposed to update the NAS (Non-Access Stratum) definition using wording similar as that in TS 21.905 as following:

Non-Access Stratum: The Non-Access Stratum (NAS) represents the protocols of the control plane between the UE and the 6G CN that are not terminated in the 6G accessing network.

Proposal#1: Update the NAS (Non-Access Stratum) definition using wording similar as that in TS 21.905.

2)	KI of WT#1.1

Proposal#2: The content of the KI is proposed to get aligned with the WT description, so as the construction style of the items.

Proposal#3: Update Note 5 to indicate that the involved 6G System Procedures including the RAN-CN, within CN interactions are only the ones related to the control signalling between UE and CN.

# 2. Proposal

It is proposed to add the following to the TR 23.801-01 “Study on Architecture for 6G System” based on S2-2509819 for WT description, and KI as following:

**** First Change ****

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Non-Access Stratum: The Non-Access Stratum (NAS) represents the protocols of the control plane between the UE and the 6G CN that are not terminated in the 6G accessing network.

# Annex A.X. WT#1.1 Study the support for control signalling for 6G System

1.	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

b.	Void

NOTE 1: 	It is assumed that this WT covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2: For the above bullet a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

2. 	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 3: This WT1.1 bullet 2a can include any transport mechanism such as using NAS or UP or new plane. WT1.1 bullet 2a can have dependency on other work tasks e.g. WT3, WT4, WT5, WT6, WT1.1 bullet 1a.

**** Second Change ****

# 5.X. Key Issue #1.1: Aspects for non-access stratum functionality and UE-CN control signalling

This Key issue studies the following aspects:

1.	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE 1: 	It is assumed that this key issue covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2: For the above bullet a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

2. 	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 3: This KI1.1 bullet 2a can include any transport mechanism such as using NAS or UP or new plane. KI1.1 bullet 2a can have dependency on other key issues defined for e.g. WT3, WT4, WT5, WT6, WT1.1 bullet 1a.

NOTE 4:	Interdependency of NFs within 6G CN to support the 6G System procedure is to be minimized, considering also the efficiency of e2e communication.

NOTE 5: 	6G System Procedures will include potential interaction between RAN-CN, interaction within CN NFs related to the control signalling between UE and CN. The signaling interaction between RAN-CN need coordination and alignment with RAN3.

NOTE 6: Interworking and migration aspects, where applicable, are covered by key issues for WT#2. Impact to IWK due to features introduced by this key issue will be covered by this key issue.

**** End of Changes ****