SA WG2 Meeting #172	S2-2510498

17 - 21 November, 2025, Dallas, USA

Source:	Xiaomi

Title:	[WT#3.1] KI on AI for 6G network

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: The clarification on WT#3.1 on AI for 6G network.

# 1. Justifications

The contribution provides clarification on WT#3.1 for AI for 6G network based on the discussion in S2-2509836 merging all the input contributions on WT#3.1 during SA2#171 and further discussion in CC on October 28, 2025.

1)	Controversial note on intent

There is following note which is controversial with the other description for the network receiving intent and handling accordingly.

NOTE 2:	It is not required to support intents in a network.

Proposal#1: Remove NOTE 2 which is controversial with other descriptions.

2)	Dynamically control the use of AI capabilities in its 6G CN

There is following item to dynamically control the use of AI capabilities in its 6G CN, the object to be controlled is not clear. It should be controlling the network to use the AI capabilities in its 6G CN, not controlling the UE. So the clarification is needed.

6.	enable the operator to dynamically control the use of AI capabilities in its 6G CN, i.e. support different operator-configurable levels of autonomy based on operational needs, including the option to not use any AI capabilities.

Proposal#2: Clarify the object to be dynamically controlled for the use of AI capabilities in its 6G CN is the network.

# 2. Proposal

It is proposed to add the following to the TR 23.801-01 “Study on Architecture for 6G System” based on S2-2509836 for WT description, and add the KI with the same content as following:

**** First Change ****

# Annex A.3. WT#3 Scope

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, AI framework).

NOTE 1: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

WT3.1:	AI for 6G architecture

The AI for the 6G architecture shall be multi-vendor interoperable, reliable and sustainable.

Study whether and how to provide an architecture for AI to fulfil the following:

1.	enable the 6G CN to fulfil requests from UEs and AFs leveraging AI technologies in the 6G CN (e.g. AI agent), using 6G CN functionality available in the network:

1.1	when the requests from the UEs and AFs include intent

NOTE 1:	An intent refers to expectations including requirements, goals, conditions, and constraints, without specifying how to achieve them. Intent cannot remain unspecified. What intent means in SA2 specifications will be determined by the study.

1.2	determine the constraints on the use and expression of intents to be processed and interpreted unambiguously by the AI capable entities in the 6G CN and define the mechanisms required to support these constraints, if any.

NOTE 3:	It is assumed it is not required for the MT stack of UE to produce nor understand the intent.

1.3	when the requests from the UEs and AFs do not include intent (e.g. like registration request in previous generations)

NOTE 4:	The MT stack of UE is assumed to be agnostic to whether the network uses AI capable entities (e.g. AI agent, AI-enabled NFs) to address UE requests not including intent.

1.4	design 6G system procedures in 6G CN so that parts of these procedures can be composed dynamically using AI capable entities.

NOTE 5:	This should not result in duplicated procedures in 3GPP specifications.

NOTE 6:	It is assumed that the 6G study will still define 6G system procedures for all 3GPP features, without requiring AI capabilities

2.	enable closed-loop operations and learning techniques such as reinforcement learning, in 6G CN

3.	enable entities in 6G CN to access network AI capabilities provided by 6G NFs

4.	enable AI capable entities in 6G CN to access trusted external capabilities provided by AF

5.	enable the monitoring of the performance of AI capable entities in 6G CN

6.	enable the operator to dynamically control the network's use of AI capabilities in its 6G CN, i.e. support different operator-configurable levels of autonomy based on operational needs, including the option to not use any AI capabilities.

7.	support roaming scenarios, e.g. how visited networks using or not using AI technologies can interwork with home network using or not using AI technologies.

8.	enable NFs of the 6G CN to have AI/ML capabilities, e.g. ML model provisioning, inferencing, training and monitoring

9.	enable the 6G CN AI architecture to ensure interoperability with 5G CN AI architecture if needed, for the purpose of maintaining a consistent service for UEs across 5G and 6G

NOTE 7: It is assumed that operators can decide whether to deploy AI capabilities in their network.

NOTE 8:	Aspects related to SA3, SA5, SA6 will be coordinated with each group respectively during the study.

NOTE 9:	Further alignments with SA1 consolidated requirements may be needed.

NOTE 10:	This sub work task does not cover AI agents in UE.

**** Second Change ****

# 5.X. Key Issue #3.1: AI for 6G Network

Study how to support and enable use of AI in 6G (e.g. AI agent, AI framework).

NOTE 1: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

The AI for the 6G architecture shall be multi-vendor interoperable, reliable and sustainable.

Study whether and how to provide an architecture for AI to fulfil the following:

1.	enable the 6G CN to fulfil requests from UEs and AFs leveraging AI technologies in the 6G CN (e.g. AI agent), using 6G CN functionality available in the network:

1.1	when the requests from the UEs and AFs include intent

NOTE 1:	An intent refers to expectations including requirements, goals, conditions, and constraints, without specifying how to achieve them. Intent cannot remain unspecified. What intent means in SA2 specifications will be determined by the study.

1.2	determine the constraints on the use and expression of intents to be processed and interpreted unambiguously by the AI capable entities in the 6G CN and define the mechanisms required to support these constraints, if any.

NOTE 3:	It is assumed it is not required for the MT stack of UE to produce nor understand the intent.

1.3	when the requests from the UEs and AFs do not include intent (e.g. like registration request in previous generations)

NOTE 4:	The MT stack of UE is assumed to be agnostic to whether the network uses AI capable entities (e.g. AI agent, AI-enabled NFs) to address UE requests not including intent.

1.4	design 6G system procedures in 6G CN so that parts of these procedures can be composed dynamically using AI capable entities.

NOTE 5:	This should not result in duplicated procedures in 3GPP specifications.

NOTE 6:	It is assumed that the 6G study will still define 6G system procedures for all 3GPP features, without requiring AI capabilities

2.	enable closed-loop operations and learning techniques such as reinforcement learning, in 6G CN

3.	enable entities in 6G CN to access network AI capabilities provided by 6G NFs

4.	enable AI capable entities in 6G CN to access trusted external capabilities provided by AF

5.	enable the monitoring of the performance of AI capable entities in 6G CN

6.	enable the operator to dynamically control the network's use of AI capabilities in its 6G CN, i.e. support different operator-configurable levels of autonomy based on operational needs, including the option to not use any AI capabilities.

7.	support roaming scenarios, e.g. how visited networks using or not using AI technologies can interwork with home network using or not using AI technologies.

8.	enable NFs of the 6G CN to have AI/ML capabilities, e.g. ML model provisioning, inferencing, training and monitoring

9.	enable the 6G CN AI architecture to ensure interoperability with 5G CN AI architecture if needed, for the purpose of maintaining a consistent service for UEs across 5G and 6G

NOTE 7: It is assumed that operators can decide whether to deploy AI capabilities in their network.

NOTE 8:	Aspects related to SA3, SA5, SA6 will be coordinated with each group respectively during the study.

NOTE 9:	Further alignments with SA1 consolidated requirements may be needed.

NOTE 10:	This sub work task does not cover AI agents in UE.

**** End of Changes ****