3GPP TSG-SA WG2 Meeting #172	S2- 2510503

17 - 21 November, 2025, Dallas, USA	(revision of S2-2508768)

Title:	23.700-14: Interim Agreements for Key Issue#5: Sensing Result Exposure

Source:	ETRI

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract:	This contribution analyses candidate solutions for sensing result exposure (Sol#1, 2, 3, 19, 23, 27–33, 37) and proposes agreed principles for KI#5 in TR 23.700-14.

Proposal

The study of sensing result exposure in 6G has produced a range of solutions (Sol#1-3, 19, 23, 27–33, 37). These solutions address different aspects of how sensing results can be requested, authorized, delivered, and consumed by authorized entities.

Based on the evaluation of these solutions and follow-up discussions after SA2 #171, this revision refines the interim agreements for Key Issue #5 (Sensing Result Exposure) in TR 23.700-14 to reflect the latest consensus. Key observations are:

-	Exposure to authorized consumers: Sol#1, Sol#3, Sol#27, Sol#28, Sol#30, Sol#31, Sol#33 and Sol#37 propose mechanisms to expose results to trusted Application Functions (AFs) directly or to untrusted AFs via the Network Exposure Function (NEF), ensuring authorization and security. This demonstrates the fundamental requirement for controlled exposure mechanisms.

-	Multiple reporting modes: Sol #2, Sol #3, Sol #19, Sol #27, Sol #28, Sol #30, Sol #31, Sol #32, Sol #33, and Sol #37 support one-time, periodic, and event-triggered reporting, indicating consensus that diverse modes are required.

-	Processed results: Sol #23, Sol #27, Sol #31, Sol #33, and Sol #37 propose exposure of aggregated, pre-processed, or AI/ML-assisted sensing results, complemented by contextual information such as time, location, and confidence level.

-	Real-time and subscription/notification exposure with QoS: Sol #29, Sol #30, Sol #32, Sol #33, and Sol #37 introduce mechanisms to support both real-time and subscription-based exposure, including QoS and reliability considerations for latency-sensitive applications.

-	Storage and reuse of sensing results: Sol #23, Sol #27, Sol #31, and Sol #33 enable storage or caching of sensing results so that they can be reused for subsequent sensing service requests without initiating new sensing operations.

| Solution | Addressed Issue | Justification (Summary) | Applicable Principle(s) |
| --- | --- | --- | --- |
| Sol#1 | Provides the basic exposure framework between SF and consumer. | Initial sensing service framework defining how a Sensing Function (SF) provides sensing results to authorized AFs / NFs. | P1 |
| Sol#2 | Establishes single-shot (one-time) reporting mode. | Procedure where a Sensing Support Function (SSF) exposes one-time sensing results to the AF through NEF. | P1, P2 |
| Sol#3 | Adds periodic reporting and exposure path definition. | AF → NEF → SF procedure supporting repeated or periodic sensing reports. | P1, P2 |
| Sol#19 | Introduces event-trigger reporting from edge. | Edge-based exposure of event-triggered sensing results for low-latency applications. | P2 |
| Sol#23 | Introduces processed results and caching NF. | Distributed SePF chain aggregates results and enables storage/re-use for later requests. | P3, P5 |
| Sol#27 | Integrates multi-mode reporting and aggregated outputs. | Unified SF exposes object maps with contextual info; supports one-time, periodic, event modes. | P1, P2, P3, P5 |
| Sol#28 | Reinforces trusted/untrusted AF handling and event modes. | AF requests via NEF; SF returns results with optional event trigger. | P1, P2 |
| Sol#29 | Introduces real-time exposure and QoS handling. | Differentiates real-time (high volume) vs subscription (notification) result delivery. | P4 |
| Sol#30 | Adds QoS & reliability to multi-mode exposure. | Exposure to trusted / untrusted AFs including QoS and contextual info support. | P1, P2, P4 |
| Sol#31 | Defines storage NF and aggregated reporting. | GSMF/SSMF manage aggregated results and store them for reuse; supports CP or UP delivery. | P1, P2, P3, P5 |
| Sol#32 | Event types & QoS metrics for reporting. | SeMF supports different event types and quality indicators (confidence, accuracy). | P2, P4 |
| Sol#33 | Combines exposure, QoS control and storage. | Gateway Sensing NF exposes results (periodic/event) with QoS and caching capability. | P1, P2, P3, P4, P5 |
| Sol#37 | Adds intelligent processing and multi-mode delivery. | SCF exposes AI/ML-assisted results to AF via NEF; supports multi-mode reporting. | P1, P2, P3, P4 |

P1: Exposure of sensing results to authorized consumers (via NEF for untrusted AFs).

P2: Support for multiple reporting modes (one-time, periodic, event-triggered).

P3: Exposure of processed / aggregated / AI-assisted results with contextual info.

P4: Real-time and subscription/notification exposure with QoS and reliability.

P5: Storage / caching of sensing results for reuse in future requests.

Based on these findings, agreed principles have been derived. It is proposed to approve the following Interim Agreements for Key Issue#5 in TR 23.700-14.

* * * First Change * * * *

7.1.Y	Agreed Principles for KI#5: Sensing Result Exposure

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

To support KI#5 of Sensing Result Exposure, the following principles are concluded:

-	The system supports exposure of sensing results to authorized consumers (e.g., AFs, NFs). For untrusted AFs, the exposure is mediated via the Network Exposure Function (NEF) using existing exposure framework. (Sol#1, Sol#3, Sol#27, Sol#28, Sol#30, Sol#31, Sol#33, Sol#37)

-	The system supports multiple reporting modes, including one-time, periodic, and event-triggered, according to the sensing service request from the consumer. (Sol#2, Sol#3, Sol#19, Sol#27, Sol#28, Sol#30, Sol#31, Sol#32, Sol#33, Sol#37)

-	The system supports exposure of processed sensing results, including aggregated, pre-processed, or AI/ML-assisted results, complemented by contextual information (e.g., time, location, and confidence levels). (Sol#23, Sol#27, Sol#31, Sol#33, Sol#37)

-	The system supports both real-time and subscription/notification-based exposure of sensing results, with QoS and reliability according to the requested sensing service parameters. (Sol#29, Sol#30, Sol#32, Sol#33, Sol#37)

-	The network may store or cache sensing results for reuse in subsequent sensing service requests without initiating new sensing operations. (Sol#23, Sol#27, Sol#31, Sol#33)

NOTE 1: Privacy and security aspects related to sensing result exposure shall be coordinated with SA3.

NOTE 2: Charging and OAM implications of exposing sensing results shall be coordinated with SA5.

* * * Second Change * * * *

7.2.Z	Topics for further consideration for KI#5

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future Release.

-	Further study is needed on how to define the sensing result schema, including granularity and mandatory/optional parameters to be exposed to consumers.

-	Clarification is required on which contextual information and sensing QoS metrics (e.g., confidence, latency, accuracy) should be standardized for exposure.

-	Criteria should be studied for selecting between control-plane and data-plane exposure based on data volume, latency, and reliability requirements.

* * * End of Changes * * * *