SA WG2 Meeting #172	S2-2510656

Dallas, US, 17 Nov – 21 Nov, 2025	(revision of S2-2509053)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0110 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service operation clarifications and alignments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. The role of Transaction ID for AIOTF/NEF service To support the implicit sub-notify mechanism for ambient IoT inventory or command service, some parameters require to be clarified to complete the related service operations and procedures: 1. Transaction ID in response: provided by Producer (e.g., NEF AIOTF), the identifier of the transaction that is created for the inventory/command request. This ID can be used by the Consumer to identify the NF Service Consumer request. 2. Notification Endpoint in the request: provided by Consumer, this is used to indicate where the notifications shall be delivered. In the context of Ambient IoT, the key point is to enable the consumer to link the notifications to the correct inventory/command request. Thus, for each notify message towards the consumer, the producer includes the Transaction ID, and the consumer can use the Transaction ID to link to the previous service request. This “Transaction ID” is same as the “Transaction Reference ID” used in other services as defined in 23.502: like Trigger, ChargeableParty, AFsessionWithQoS, ServiceParameter, AF_request_for_QoS etc. See the usage example of “Transaction Reference ID” in ServiceParameter notify service: “The Transaction Reference ID identifies the AF request for service specific parameter provisioning that the notification (i.e., notification of an authorization update or reporting a subscribed event) is related to.” Thus, it is proposed to change the current Transaction ID term to “Transaction Reference ID” to align with the traditional implicit sub-notify mechanism. . The term "AF Transaction ID" is used in the AF request, provided by AF in the request, identifies the AF request, this "AF Transaction ID" is returned in the response back to AF, see usage example of this for TrafficInfluence service: "The AF Transaction Id refers to the request”. This is defined in TS23.502 and TS29.522, for examples AMInfluence, ECSAddress, TrafficInfluence, PolicyAuthorization NF services. The traditional AF Transaction ID is used differently from AIoT sub-notify mechanism, it is proposed to change the current AF transaction id term to “Transaction Reference ID” to align with the traditional implicit sub-notify mechanism. Proposal 1: update the NEF service operations and AIOTF service operations to align the terms (i.e., change transaction id, AF transaction id to transaction reference id). 2. AMF implicit sub-notify service operation For inventory-only procedure, - For each involved gNB, the AMF transfers one inventory request message from AIOTF towards gNB, and transfers at least one feedback message from gNB towards AIOTF. For inventory and command procedure, - For each involved gNB, the AMF transfers one inventory request message from AIOTF towards gNB, and transfers at least one feedback message from gNB towards AIOTF. - Later, for each target device, the AMF transfer one command request message from AIOTF towards gNB, and transfers at least one feedback message from gNB towards AIOTF. - In this case, the same NGAP session is used for inventory phase and command phase, this means the AMF needs to share same transaction for different message delivery requests from one AIOTF towards the same gNB in inventory phase and command phase The AMF maintains the notification endpoint for a transaction per AIOTF ID, per Correlation ID and per gNB for the inventory-only procedure, also for inventory and command procedure. If the AMF manages the notification endpoint per transaction (link to one AIOTF ID, one Correlation ID and one gNB): Option 1: The AMF assigns and returns the Transaction Reference ID for the AIOT3 transaction during inventory transfer. The AMF identifies the messages relevant for a AIOT3 transaction using Transaction Reference ID. The use of Transaction Reference ID (instead of Endpoint) in subsequent message delivery (like command request per device) avoids creation of transaction context every time. The AIOTF links the feedback messages to the inventory request or command request sent before using Transaction Reference ID. This aligns with the existing sub-notify mechanism principle. Option 2: The AMF identifies the messages relevant for a AIOT3 transaction using AIOTF ID, RAN ID, Correlation ID. The AIOTF links the feedback messages to the AF request or command request sent before using Correlation ID inside the container. This special optimization also works, but is different with the current sub-notify mechanism, and is not recommended in this proposal. Proposal 2: update the AMF service operation to return the transaction reference ID to align the implicit sub-notify service operation in the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update the NEF service operations and AIOTF service operations to align the terms for Transaction Reference ID and AF transaction ID 2. Update the AMF message delivery service operation to add the Transaction Reference ID, notification endpoint and align command names. 3. Align the styles to document the parameters 4. Update message type to align with RAN3 progress throughout 5. editorial corrects |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete implicit sub-notify mechanism for AIoT service operations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4.2, 6.2.4.3, 7.2.2, 7.2.3, 7.2.4, 7.3.1, 7.3.2, 7.3.3, 7.4.2, 7.4.3, 7.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.4.2	Message delivery to NG-RAN

When the AIOTF sends NGAP message, the AMF receives an Namf_AIoT_MessageDelivery request and sends the corresponding NGAP message to NG-RAN. The additional steps used for indirect interface between AIOTF and NG-RAN are shown in Figure 6.2.4.2-1.

Figure 6.2.4.2-1: Procedure for NGAP message delivery to NG-RAN using indirect connectivity via an AMF

1.	The AIOTF sends Namf_AIoT_MessageDelivery Request message (NGAP AIoT information, NG-RAN ID, AIOTF ID, Correlation ID, AIoT NGAP Message Type, Notification endpoint or Transaction Reference ID, [RAN AIoT Device NGAP ID]) to the AMF.

The AIoT NGAP Message Type identifies the NGAP message to send to NG-RAN and NGAP AIoT information is provided to NG-RAN in the NGAP message. The message includes Notification endpoint if the AIoT NGAP Message Type indicates "Inventory Request", otherwise "Transaction Reference ID". The message includes RAN AIoT Device NGAP ID if the AIoT NGAP Message Type indicates "Command Request".

If the AMF receives a Notification endpoint, the AMF creates a context for the transaction and stores the NG-RAN ID, AIOTF ID, Correlation ID and Notification endpoint. The AIOTF is implicitly subscribed to Namf_AIoT_Notify events.

2.	The AMF responds to the AIOTF with a result indicating whether the AMF will handle the request. If a failure is indicated by the result indication, then the procedure stops and the remaining steps are skipped.

If the AMF accepts the request and receives a Notification endpoint rather than Transaction Reference ID, the AMF creates a context for the transaction and stores the NG-RAN ID, AIOTF ID, Correlation ID and Notification endpoint in the context, this context is used as routing information to route multiple NGAP messages from NG-RAN to AIOTF, see clause 6.2.4.3. The AIOTF is implicitly subscribed to Namf_AIoT_Notify events.

3.	The AMF sends an NGAP message (AIOTF ID, Correlation ID, NGAP AIoT information, [RAN AIoT Device NGAP ID]) to the target NG-RAN.

4.	The procedure for handling NGAP messages from NG-RAN is used to route NGAP messages from NG-RAN to the AIOTF, see clause 6.2.4.3.

# * * * * Next change * * * *

6.2.4.3	Message delivery to AIOTF

When the AMF receives an NGAP message from NG-RAN, for example, responses to a procedure or events like Inventory Reports, the AMF determines which AIOTF to send a Namf_AIoT_Notify to and sends the information from NG-RAN to that AIOTF. The additional steps used for indirect interface from NGAP towards AIOTF is shown in Figure 6.2.4.3-1.

Figure 6.2.4.3-1: Procedure for NG-RAN event handling using indirect connectivity via an AMF

1.	Before any NGAP message can be routed from NG-RAN to the AIOTF, the AMF needs routing information to be able to route the messages from NG-RAN to the AIOTF, see clause 6.2.4.2. The same context created in clause 6.2.4.2 includes such routing information and can be used to route multiple NGAP messages from NG-RAN.

2.	NG-RAN sends an NGAP message (AIOTF ID, Correlation ID, NGAP AIoT information, [RAN AIoT Device NGAP ID]) to an AMF. NGAP AIoT information is determined by NG-RAN depending upon the operation the NGAP message relates to.

3.	AMF determines the Notification endpoint using the AIOTF ID and the Correlation ID received in step 1 from NG-RAN, and then sends the Namf_AIoT_Notify message (AIoT NGAP Message Type, NGAP AIoT information, Transaction Reference ID) to the AIOTF.

Step 2 and step 3 are repeated for each NGAP message received from NG-RAN.

If the message received from NG-RAN is an Inventory Failure indicating an error or an AIOT Session Release Complete, then the AMF releases the context which is associated with the AIOTF ID and the Correlation ID, after sending the Namf_AIoT_Notify message to the AIOTF.

# * * * * Next change * * * *

7.2.2	Naiotf_AIoT_Inventory service operation

Service operation name: Naiotf_AIoT_Inventory.

Description: The NF consumer requests an inventory operation for one or multiple AIoT Device(s).

Inputs, Required:

-	AF ID.

-	At least one of the following parameters are included:

-	Target Area information for the inventory operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information(see clause 5.8) for multiple target AIoT Devices.

-	Notification Endpoint.

Inputs, Optional:

-	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

-	Time Interval for result aggregation.

-	Location information requested.

Outputs, Required: Transaction Reference ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

# * * * * Next change * * * *

7.2.3	Naiotf_AIoT_Command service operation

Service operation name: Naiotf_AIoT_Command.

Description: The NF consumer requests a command operation for one or multiple AIoT Device(s).

Inputs, Required:

-	AF ID.

-	At least one of the following parameters are included:

-	Target Area information for the command operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information(see clause 5.8) for multiple AIoT Devices.

-	Notification Endpoint.

-	Command Type: Read, Write or Permanent Disable.

Inputs, Optional:

-	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

-	Approximate message size from the AIoT Device for Read Operation.

-	If the Command Type is Read, the offset to read application data from and the length of application data to read shall be included.

-	If the Command Type is Write, the offset where to write the application data, the application data to write and its length shall be included.

-	Location information requested.

Outputs, Required: Transaction Reference ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

# * * * * Next change * * * *

7.2.4	Naiotf_AIoT_Notify service operation

Service operation name: Naiotf_AIoT_Notify

Description: The AIOTF uses this service operation to notify the results or status of the service operation towards the NF consumers. If the NF consumer invokes the Naiotf_AIoT_Inventory, or Naiotf_AIoT_Command service operation, the NF consumer implicitly subscribes to the results of the requested service operation.

Inputs, Required:

-	Common report information: Transaction Reference ID.

Inputs, Optional:

-	List of AIoT Device ID or Failure Cause in case of Failure.

-	Read command specific report information: Information obtained from each target AIoT Device corresponding to each reported AIoT Device ID.

-	The Last Report Indication, indicating the notify is the last notify for an AIoT service operation.

-	Location information of each AIoT Device.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * Next change * * * *

7.3	AMF services

7.3.1	General

AMF supports to expose AIoT services towards the AIOTF as described in Table 7.3.1-1. The Namf_AIoT AMF service is used when the NG-RAN and the AIOTF communicate indirectly via an AMF.

Table 7.3.1-1: NF services provided by the AMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_AIoT | MessageDelivery | Request/Response | AIOTF |
|  | Notify | Subscribe/Notify | AIOTF |

# * * * * Next change * * * *

7.3.2	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: The NF consumer requests to send AIoT NGAP Messages towards NG-RAN.

Inputs, Required:

-	NGAP AIoT Information to deliver to NG-RAN.

-	NG-RAN ID.

-	AIoT NGAP Message Type ("Inventory Request" or "Command Request" or "Session Release Command").

-	AIOTF Identifier and Correlation Identifier, this is to allow identifying the association between NG-RAN and AMF.

-	Notification Endpoint or Transaction Reference ID.

Outputs, Required: Transaction Reference ID, Result indication (Success or Failure), Failure Cause in case of Failure.

# * * * * Next change * * * *

7.3.3	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: The NF consumer requests to receive notifications for AIoT NGAP messages from NG-RAN. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the Namf_AIoT_Notify.

Inputs, Required:

-	AIoT NGAP Message Type ("Inventory Reponse", "Inventory Failure", "Inventory Report", "Command Response", "Command Failure", "Session Release Complete").

-	AIoT Information received from NG-RAN.

-	Transaction Reference ID.

Input, Optional: None.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

# * * * * Next change * * * *

7.4.2	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_AIoT_Inventory

Description: The consumer requests to perform an inventory operation for an AIoT Device or multiple AIoT Devices.

Input, Required:

-	AF ID.

-	At least one of the following parameters are included:

-	External Target Area information.

-	Either AIoT Device ID(s) or AIoT Device ID filter information for the inventory operation.

-	Notification Endpoint.

Input, Optional: Information to be used for resource allocation(Approximate number of AIoT Devices), time interval, location information requested.

Output, Required: Transaction Reference ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

# * * * * Next change * * * *

7.4.3	Nnef_AIoT_Command service operation

Service operation name: Nnef_AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices.

Input, Required:

-	AF ID.

-	At least one of the following parameters are included:

-	External Target Area information.

-	Either AIoT Device ID(s) or AIoT Device ID filter information for the command operation.

-	Notification Endpoint.

-	Command type (Read, Write, or Permanent Disable).

Input, Optional: Information to be used for resource allocation(Approximate number of AIoT Devices), if the Command Type is Read, the offset to read application data from and the length of application data to read shall be included, if the Command Type is Write the offset where to write the application data, the application data to write and its length shall be included, Approximate message size from the AIoT Device for Read command type, location information requested.

Output, Required: Transaction Reference ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

# * * * * Next change * * * *

7.4.4	Nnef_AIoT_Notify service operation

Service operation name: Nnef_AIoT_Notify

Description: The consumer receives notification of the status or results of the requested service operation. If the consumer invokes the Nnef_AIoT_Inventory, or Nnef_AIoT_Command service operation, the consumer implicitly subscribes to the results of the requested service operation.

Input, Required:

-	Transaction Reference ID.

Input, Optional:

-	a list of AIoT Device ID(s), Failure Cause in case of Failure.

-	Read command specific report information: Information obtained from each target AIoT Device.

-	The Last Report Indication, indicating the notify is the last notify for an AIoT service operation.

-	Location information of each target AIoT Device.

Output, Required: Result indication.

Output, Optional: None.

# * * * * End of changes * * * *