3GPP TSG-SA WG2#172	S2-2510760

Dallas, USA, 17-21 November 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	FS_Sensing_ARC: Additional principles for KI#1 on system architecture to support sensing

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_Arc / Rel-20

Abstract of the contribution: This contribution proposed additional principles for the KI#1 on system architecture to support sensing.

# 1.	Discussion

In SA2#171, several general principles were agreed for the KI#1 for FS_Sensing_Arc on system architecture to support sensing. A few issues were left open. This contribution proposed the additional principles to address these open issues:

- 	Whether to involve AMF in sensing data transport. As discussed in SA2#171, majority companies prefer to avoid impacts to the AMF for the sensing data transport. This is due to the capability limitation of both the NGAP stack and the AMF, for the large volume of sensing data or even frequent small messages. Therefore, it is proposed in this contribute to conclude that a direction connection between SE and SF is used, and AMF is not involved in the sensing data transport.

-	Whether to involve AMF in the sensing service control signaling. There were several disadvantages of involving AMF, including the impacts to the AMF interfaces and the NGAP stack. Additionally, given that the signaling is non-UE specific, there would be additional logic to handle the AMF selection and also additional context in the gNB to manage such connections. Furthermore, since the AMF does not need to participate in the sensing service operation, as it is purely between the SF and SE and there is no UE involvement, there is no reason to force the AMF to be on the path. Therefore, it is proposed to also not involve AFM for the sensing service operation control.

-	How the direct connection is established. For the limited use case in this release, it is proposed to use the OAM configuration to trigger SE to establish the direct connection with the SF. This would allow the SE to also made its availability and capability known to the SF, to facilitate the selection as in KI#3. There were some comments regarding the number of connections to be supported by the SF. However, this should not be a concern as the OAM can control how many SEs need to be active and connect to which SF. The establishment of the direct connection itself would not place extra load on the SF, since no sensing data is sent.

-	Whether a Gateway function is needed to terminate and authorize the sensing service request from the sensing consumers. There were several options proposed, e.g. using NEF to perform the sensing service authorization, or using the SF to terminate and authorize the sensing service request. The issues of using NEF are two folds: firstly, it would require the NEF to understand the sensing service parameters which also requires NEF updates whenever sensing service evolves; and secondly, the NEF is not always used for sensing, e.g. when consumer is a trusted AF. The issue for using SF to direct terminate and authorize the sensing service request is that the limited flexibility in selecting the SF, i.e. if SFs can have different sensing service capabilities, the SF terminating the request may not be able to process the request (e.g. the service types). Therefore, it is proposed to have a Gateway function to terminate the sensing service request, and it can select the SF based on the sensing service parameters. The Gateway function could collocate with SF in deployment.

-	Relationship with the LCS framework. There were use cases where the LCS framework was used for obtaining the target area, e.g. when UE ID is used as a service request parameter. However, for the deployment considerations, there shouldn't be any dependency between the sensing service and the LCS framework, i.e. sensing operation should be possible even if the network does not deploy LCS framework, e.g. sensing service operation can reject the request based on UE ID if it cannot translate that to location information.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-14:

# >>>>BEGINNING OF CHANGES<<<<

### 7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

-	one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

-	NO dedicated storage NF is needed to store the Sensing data and Sensing result.

-	In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

- 	A direct connection between SE, i.e. gNB, and SF (SPF) is used for sensing data transport, without AMF involvement.

-	A direct connection between SE, i.e. gNB, and SF (SCF) is used for sensing service operation control, without AMF involvement.

-	The direct connection is established by the SE, i.e. gNB, based on OAM configuration towards the SF.

NOTE 6: 	the protocols used for the direct connection is determined in normative phase based on RAN WGs feedback.

-	A Gateway function terminates the sensing service request from the sensing consumer (or via NEF) to authorize the request and select the SF based on the sensing service request parameters.

-	In this release, sensing service operation is independent of LCS framework.

# >>>>END OF CHANGES<<<<