3GPP TSG-SA WG2 Meeting #172	S2-2509946

Dallas, USA, November 17 – 21, 2025										(revision of S2-2508384)

Source:	CATT

Title:	KI#1 interim agreements

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes updates to interim agreements for KI#1 “Transfer of data over UP for UE data collection”.

1. Introduction

For KI#1 “Transfer of data over UP for UE data collection”, topics for further consideration are as described in clause 7.2.1 of TR 23.700-04.

Following are some analysis and proposals to address these topics:

1)	How the 5GC NF controls UE data transfer

This topic is related to the following topics:

-	How the configuration information for UE data transfer is sent to the UE.

The following are proposed based on the solutions to KI#1 in the TR:

The 5GC NF initiates UE data transfer, i.e. sending configuration information for UE data transfer (e.g. data type, model type, data transfer time window/frequency/area) towards the UE over the UP, based on triggers from the AF (via the NEF) or the UE.

The 5GC NF may modify UE data transfer, i.e. updating the configuration information for UE data transfer towards the UE over the UP, based on triggers from the AF (via the NEF), the UE or the NG-RAN, or based on local configuration (i.e. internal triggers of the 5GC NF).

The 5GC NF terminates UE data transfer, when the data transfer is finished, or based on triggers from the AF (via the NEF), the UE or the NG-RAN.

2)	Whether and how the UE controls UE data transfer

The following are proposed based on the solutions to KI#1 in the TR:

The UE can send a request to the 5GC NF for UE data transfer from this UE, e.g. based on application-layer interactions with the AF or UE local configuration. The 5GC NF can accept this request and send configuration information for UE data transfer towards the UE, or reject this request.

The UE can reject an UE data transfer request from the 5GC NF, e.g. due to unacceptable configuration information for UE data transfer.

The UE can send a request to the 5GC NF for modifying UE data transfer from this UE (i.e. request for updating the configuration information of UE data transfer, e.g. data transfer frequency), e.g. based on UE status (e.g. UE performance or energy saving) or application-layer interactions with the AF.

The UE can send a request to the 5GC NF for terminating UE data transfer from this UE, e.g. based on UE status or application-layer interactions with the AF.

3)	Whether an existing 5GC NF or a new 5GC NF is used for controlling UE data transfer

Among the existing 5GC NFs, the DCCF seems the most appropriate one to be used for controlling UE data transfer, considering that the DCCF performs data collection coordination for network data analytics and already supports providing collected data to the AF and the NEF.

On the other hand, the DCCF does not support UP functionalities, i.e. establishing UP connection with the UE, nor supports providing UE data transfer related configuration information to the NG-RAN (see below discussion). Such functional extensions (i.e. new interfaces towards the UE and the NG-RAN) seem quite different from its existing functionalities.

Another candidate existing 5GC NF which can be used for controlling UE data transfer is the NWDAF. The NWDAF can receive UE application data via the DCAF, and then extending NWDAF functionality to further receive UE AS layer data via the User Plane seems natural.

Introducing a new 5GC NF means a lot of work to be done on defining new interfaces/services with existing NFs, and especially adds complexities in implementation and deployment. Therefore this should be avoided if possible.

So it is propose to reuse the NWDAF for controlling UE data transfer. Or, the decision can be made after the RAN WG(s) chooses whether to use UP or CP for UE data transfer.

4)	How the configuration information for UE data transfer is sent to the UE

See 1). Considering that the configuration information for UE data transfer could be of large volume, it is more efficient to transfer it over the UP.

5)	How the 5GC NF sends its UP information (i.e. IP address or FQDN of the 5GC NF), for standardized UE data transfer from the UE to the 5GC NF.

Triggered by the UE data transfer request from the AF (possibly via the NEF) or from UE (via the AMF), the 5GC NF sends its UP information (i.e. IP address or FQDN of the 5GC NF) to the UE via the AMF over the CP.

6)	How the 5GC NF checks the reported UE data to support the full visibility of standardized UE data contents.

As RAN2 clarified in S2-2500025/R2-2411152, “visibility of data content signifies that the MNO will be able to be aware of, access, and comprehend the content of the collected/reported data without the need of SLA. Thus, full visibility allows the MNO to verify/match the data specified/configured to be collected and the data that is being reported.”

So the 5GC NF needs to know the two kinds of information/data, “data specified/configured to be collected” and “data that is being reported”, to support the full visibility of standardized UE data content.

From the 5GC NF point of view, the “data specified/configured to be collected” is reflected/indicated by the types and formats (including the value ranges) of the data specified/configured to be transferred to the AF (UE side model training server), which can be contained or indicated by the configuration information for UE data transfer in the request to the UE (and possibly also to the NG-RAN), while “data that is being reported” is indicated by the types and formats of the data transferred by the UE to the 5GC NF over the UP.

So the 5GC NF can check the types and formats (including the value ranges) of the reported UE data against the configuration information to the UE for UE data transfer, to support the full visibility of standardized UE data contents.

To support the data verification, a UP protocol is needed to clearly specify “the types and formats of the data configured to be transferred” (which is indicated by the 5GC NF to the UE in the UE data transfer request) and “the types and formats of the data transferred by the UE” (which is indicated by the UE in the transferred UE data payload). Since the UE data collection is for ML model training for use cases defined by RAN1, the types and formats of the data needs to be defined by RAN1. After RAN1 determines the types and formats of the data for the use cases (e.g. CSI, BM), the UP protocol can be specified by RAN1 or coordinated with RAN1.

7)	Whether and how to select UEs for UE data collection

According to S2-2508119/R2-2506752 "Reply LS on UE data collection and data transfer": "RAN2 have not discussed how the UEs for data collection are selected.”, "RAN2 concluded that in some use-cases additional information such as (e.g., UE data collection and feature related radio capabilities, and user consent may be relevant for selection of UEs, but RAN2 have not discussed where (i.e., gNB, CN, UE server) this information would be considered." So there is no clear clue or suggestion provided by RAN2 WG on whether and how to select UEs for UE data collection. This needs further discussion in SA2.

8)	Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter)

It is proposed to transfer the collected UE data (which is the AS layer measurement data from the UE for ML model training for RAN use cases) and UE regular traffic (which is application layer data) in different PDU Sessions, to avoid traffic handling complexity due to the different requirements on priority, latency, reliability and etc. So there is no need to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level.

9)	Whether the UE needs to be authorized for UE data collection.

For the UP solution, the UE data transfer configuration information provided by the 5GC NF contains the types and formats of data which are allowed/authorized to be collected and transmitted, as well as data collection time window, area, etc., and such configuration information should apply to all the UEs which report the data. Also the 5GC NF supports the full visibility of standardized UE data contents, which guarantees that the data reported by the UE conforms to the UE data transfer configuration information. So there seems no need to authorize the UEs separately for UE data transfer as well as UE data collection. If the UE authorization for UE data collection is considered due to radio resource consumption or charging, it would be a bit complicated considering e.g. the UE sided model training (e.g. for CSI and BM) is also used for improving the network performance.

10)	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF

The 5GC NF provides the requirement of UE data transfer (i.e. UE data transfer configuration information) based on AF/UE request and operator policy, which may include authorized reporting area, time window, data types, reporting frequency(/period) and etc.

The NG-RAN provides measurement configuration (/UE data collection configuration) to the UE, which can also include similar configuration information for UE data collection, possibly based on the request from the UE. See the agreement on UE side data collection in RAN2#129 meeting report, “For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop).  FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network.”, “The network can provide or release the data collection configuration (at any point in time), with or without UE request.”

For UP solution, the misalignments between the UE data transfer configuration by the 5GC NF and UE data collection configuration by the NG-RAN, e.g. misalignments of time window, area, or date types, may lead to contradictions with the operator policy or failing to fulfil the data collection request from the AF (UE side model training server).

So, for UP solution, in order or ensure that the UE data collection configuration by the NG-RAN aligns with the UE data transfer configuration by the 5GC NF, it is proposed that the 5GC NF also sends the UE data transfer configuration to the NG-RAN. Then the NG-RAN can decide the UE data collection configuration taking into account of the requirement of UE data transfer.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A UP path is used between the UE and a 5GC NF (which can be the NWDAF), for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	The 5GC NF sends its UP information (i.e. its IP address or FQDN) to the UE via the AMF, which triggers the UE to establish the PDU session for standardized UE data transfer.

-	The 5GC NF sends data transfer configuration information to the UE over the UP to control UE data transfer. The data transfer configuration information can be modified based on operator policy and network/UE status.

-	The 5GC NF initiates and terminates the data session(s) towards the UE(s) for UE data transfer, based on the request(s) from the AF or UE.

-	The 5GC NF checks whether the reported UE data (e.g. data types and formats) match the data transfer configuration information (including data configured to be transferred), to support the full visibility of standardized UE data contents.

-	The 5GC NF sends the data transfer configuration information also to the NG-RAN, so that the NG-RAN can decide the UE data collection configuration taking into account of the requirements of UE data transfer.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

NOTE:	Differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level is not supported.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

* * * Next Change * * * *

7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	Whether and how to select UEs for UE data collection.

-	Whether the UE needs to be authorized for UE data collection.

* * * End of Change * * * *