3GPP TSG-SA WG2 Meeting #172	S2-2510032

# Dallas, US, November 17 - 21, 2025											(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.167 | CR | 0379 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Availability of IMS Emergency Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In scenarios where the UE is in HPLMN coverage, HPLMN exhibits support for IMS emergency services and tries to establish an IMS emergency call via PS, the emergency services attempt fails and emergency services in CS are not possible (e.g., CS domain is not available due to 2G/3G sunset), according to current specifications emergency calls are not possible. Such scenarios can lead to life threatening situations and it has led to fatal situations when such failures happened in the field. Current specs leave the actions up to UE implementation totally which may lead to unpredictable behavior unfortunately. Thus, there should at least be a recommendation what the UE can do to perform emergency services or at least for UE to inform the end user that there is a failure and the end user could take other actions to obtain emergency services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To enable a more robust emergency service in failure cases, as an implementation option in the UE, the UE is allowed to select another PLMN to obtain emergency services, even if the UE is currently camping on the HPLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Emergency service is not available in scenarios where VoIMS emergency call is not successful in HPLMN and CS domain is not available in HPLMN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### BEG OF CHANGES

# H.5	Domain Priority and Selection Rules for Emergency Session Attempts

This clause details the domain priority and selection (see clause 7.3) for UE that attempts to make an emergency session for UTRAN, E-UTRAN or NG-RAN radio access networks based on the UE attach status to CS or PS domains and the network support for IMS emergency and IMS voice over PS.

The following table (Table H.1) defines these rules based on the UE (last 2 columns) for different initial conditions (first 4 columns) when an emergency session is initiated.

NOTE:	The behaviour of the UE in limited service state is defined in e.g. TS 23.122 [41], TS 36.304 [58] and TS 38.304 [54] and is supported by the broadcast of the ims-EmergencySupport bit (see TS 36.331 [55]; TS 38.331 [56]). The behaviour is that a UE supporting voice services performs cell reselection to a cell that does support emergency voice calls.

For NG-eCall (eCall over IMS) domain selection in clause H.6 applies. This clause is not applicable for NG-eCall.

When a 5G ProSe enabled UE (as specified in TS 23.304 [53]) does not have direct connection to the network for emergency service, the UE can obtain emergency service via 5G ProSe UE-to-Network Relay as specified in Annex M.

If the emergency services attempts using PS and CS have failed in a PLMN, and even this is the (E)HPLMN, the UE may select another PLMN that broadcasts support for emergency services to attempt emergency services. After the emergency service is terminated, the UE needs to perform PLMN selection e.g. to re-select to (E)HPLMN (as described in TS 23.122 [41]) to obtain regular services. [Alt 1]

Table H.1: Domain Selection Rules for emergency session attempts for UTRAN, E-UTRAN or NG-RAN radio access networks

|  | CS Attached (NOTE 9) | PS Attached | VoIMS | EMS | First EMC Attempt | Second EMC Attempt |
| --- | --- | --- | --- | --- | --- | --- |
| A | N | Y | Y | Y | PS | CS if available and supported (NOTE 7, NOTE 8) |
| B | N | Y | N | Y | PS or CS if the emergency session includes at least voice. PS if the emergency session contains only media other than voice. | PS if first attempt in CS CS if first attempt in PS (NOTE 7, NOTE 8) |
| C | N | Y | Y or N | N | PS if ESFB is "Y" (NOTE 5). Else CS or PS for another 3GPP RAT with EMS or ESFB set to "Y" if available and supported and if the emergency session includes at least voice. Else PS for another 3GPP RAT with EMS or ESFB set to "Y" if available and supported if the emergency session contains only media other than voice. | PS if first attempt in CS CS if first attempt in PS (NOTE 7, NOTE 8) |
| D | Y | N | Y or N | Y or N | CS if the emergency session includes at least voice. PS if available and EMS or ESFB is "Y" and emergency session contains only media other than voice. | PS if available and EMS or ESFB is "Y" |
| E | Y | Y | Y | Y | If the emergency session includes at least voice, follow rules in TS 22.101 [8] which say to use the same domain as for a non-EMC (NOTE 2) PS if the emergency session contains only media other than voice. | PS if first attempt in CS CS if first attempt in PS |
| F | Y | Y | Y or N | N | PS if ESFB is "Y" (NOTE 5). Else PS for another 3GPP RAT with EMS if available and supported or CS, if the emergency session includes at least voice. | CS if first attempt in PS PS for another 3GPP RAT if available and supported and EMS or ESFB is "Y" if first attempt in CS. |
| G | Y | Y | N | Y | CS if the emergency session includes at least voice. PS if the emergency session contains only media other than voice. | PS |
| EMC = Emergency Session. EMC includes also normal calls initiated in the CS domain that are treated by the CS CN as emergency calls. VoIMS = Voice over IMS over PS sessions support as indicated by IMS Voice over PS session supported indication as defined in TS 23.401 [28], TS 23.060 [2] and TS 23.502 [49]. EMS = IMS Emergency Services supported as indicated by Emergency Service Support indicator as defined in TS 23.401 [28], TS 23.060 [2], TS 23.501 [48] and TS 23.502 [49]. ESFB = Emergency Services Fallback for 5GS as defined in TS 23.501 [48] and TS 23.502 [49]. NOTE 1: If the UE selects the CS domain and initiates a normal call using the dialled local emergency number (see clause 7.1.2) and the UE enters limited service state (e.g. due to a Location Registration failing), then the UE camps on an acceptable cell (see TS 23.122 [41]) and may proceed with the EMC by initiating an emergency call in limited service state. NOTE 2: Use of the same domain as for a non-EMC is restricted to UTRAN, E-UTRAN and NG-RAN access (e.g. excludes WLAN). NOTE 3: This NOTE applies to a UE in dual registration mode as defined in TS 23.501 [48]. A dual registration mode UE that is registered to both EPC and 5GC assumes attachment, for the purpose of the "PS Attached" column, to whichever of EPC or 5GC indicates EMS as "Y". When both EPC and 5GC indicate EMS as "Y", the UE shall assume attachment to either EPC or 5GC based on implementation. A UE that is registered to both EPC and 5GC does not use emergency services fallback and ignores the ESFB condition when performing domain selection. NOTE 4: The other 3GPP RAT for row C and row F can be any of UTRA, E-UTRAN connected to EPC, E-UTRA connected to 5GC or NR connected to 5GC that is supported by the UE and differs from the RAT to which the UE is currently attached in the PS domain (or is assumed to be attached based on NOTE 3). NOTE 5: The condition 'ESFB is "Y"' only applies for a UE that is camped on or connected to 5GS via NR or via E-UTRA and that supports Emergency Services Fallback. In that case the emergency services will be provided over E-UTRAN or E-UTRA connected to 5GC as defined in procedures in TS 23.502 [49]. The condition 'ESFB is "Y"' is taken into consideration by the UE only when the network has indicated EMS = "N" for the RAT on which the UE is camping or connected. NOTE 6: For 5GS, the value of the column "EMS" is for the RAT that UE is camped on or is connected to. NOTE 7: As an implementation option, when the first attempt uses PS and fails for reasons other than related to IMS, the second attempt may use PS with a different 3GPP RAT. In this case the UE can make a third attempt using CS. If the attempts using PS and CS have failed in a PLMN, and even this is the (E)HPLMN, the UE may select another PLMN that broadcasts support for emergency services to attempt emergency services. After the emergency service is terminated, the UE needs to perform PLMN selection e.g. to re-select to (E)HPLMN (as described in TS 23.122 [41]) to obtain regular services. [Alt2] NOTE 8: As an implementation option, when the first attempt uses PS and fails because the first discovered P-CSCF is not available, additional attempts may use PS with alternate P-CSCFs (see clause E.1.1.1 of TS 23.228 [1] and clause 4.3.2.2.1 of TS 23.502 [49]). In this case the UE can make an attempt using CS after making failed PS attempts on all discovered alternate P-CSCFs. If the attempts using PS and CS have failed in a PLMN, and even if it is the (E)HPLMN, the UE may select another PLMN that broadcasts support for emergency services to attempt emergency services. After the emergency service is terminated, the UE needs to perform PLMN selection e.g. to re-select to (E)HPLMN (as described in TS 23.122 [41]) to obtain regular services. [Alt2] NOTE 9: With E-UTRAN access, "CS attached" means that the UE has performed a combined Attach/TAU and has been accepted by the network for EPS and non-EPS services. If the UE has been accepted for EPS and "SMS-only" then, for the purposes of this table, the UE should behave as if it is not CS attached. |  |  |  |  |  |  |

### END OF CHANGES