3GPP TSG-WG SA2 Meeting #172	S2-2510034

Dallas, USA, Oct 17 - 21, 2025

Source:	China Mobile

Title:	Proposal on Interim Agreements for KI#6

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreements for KI#6 for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

Solution #3, #15, #20, #27, #34, and #35 are addressing KI#6 based on TR 23.700-14 V1.1.0.

Here is a summary of the configuration update aspects of the above solutions:

Solution #3:

- The AF subscribes to a sensing service via the Core Network. The SF finds Sensing Entities and configures them. A dedicated path reports 3GPP Sensing Data to the SF.

Solution #15:

- The SF keeps an inventory of Sensing Entities and their capabilities, and for each service request, it selects and configures the appropriate Sensing Entities. Activate, deactivate, or reconfigure commands are exchanged.

Solution #20:

- Focuses on provisioning general configuration parameters to Sensing Entities, separate from a specific sensing service request. This is done by a "sensing operation server" via the SCF. The provisioned configuration parameters are exchanged

- Solution #20 Update (TdocS2-2509772, not handled due to time limitations in WUHAN meeting):

The configuration parameters for Sensing Entities can be triggered by SF itself, except for AF, under the following scenarios

During multiple Sensing Services, SF can be triggered to update configuration parameters (such as Sensing Service quality of service) for Sensing Entities.

SF can also be triggered to update configuration parameters (such as updating the Sensing Service quality of service to achieve energy savings) for the Sensing Entities based on the sensing results.

Solution #27:

- Provides a detailed breakdown of sensing service request parameters and output. The SF authorizes the request, configures Sensing Entities, collects data, and generates the result.

Solution #34:

- Introduces a Gateway Sensing Function (GSF) for discovery, authorization, and routing, with a direct connection to the Sensing Entity (RAN) for configuration and data transfer.

Solution #35:

- Proposes two methods for the Sensing Service Function to transmit parameters to the Sensing Entities (RAN).

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V1.x.0.

* * * * Start of Changes (all new test) * * * *

7.1.Y	Agreed Principles for KI#6

The Sensing Function initiates the configuration update procedures, triggered by the AF or SF itself, directly towards the Sensing Entity(s).

The configuration parameters include the target sensing area and the Sensing Service quality of service.

NOTE 1: The content of the configuration parameters and the configuration process between the SF and the Sensing Entity(s) will be coordinated with the RAN WG(s).

* * * * End of Changes * * * *