SA WG2 Meeting #172	S2-2510104

Nov. 17-21, 2025, Dallas, USA	(revision of S2-2509785)

Source:	China Mobile

Title:	[WT#1.2, Enhanced User Plane Architecture] Scope and Key Issue for Enhanced User Plane Architecture

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses areas to study for enhanced user plane mechanisms that enable high scalability for user plane transport and functional entities.

- Justifications

The changes proposed in this pCR are based on the postponed baseline document S2-2509785.

This pCR are comprised of the discussions and revisions for the items that need further discussions, i.e., UP path transport technology(ies).

- UP path transport technology(ies)

The 6G user plane should not only enhance the performance & capability of the mobile communication system (or MCS), but also provide more flexibility and extensibility for the transmission technologies over UP paths. The UP protocol GTP-U is an overlay protocol that involves many encapsulated IEs for extensions. Comparably, the SRv6 is an underlay transmission protocol that can be deployed to natively accommodate information extensions via IPv6 extension headers. Further, SRv6 is a traffic engineering technology that helps more effective path selection (or ‘establishment’).

Further, QUIC is more widely deployed. QUIC has been verified to be effective for mobile traffic transmission. Plus, there are many IETF WGs that are leveraging the QUIC protocol for extensions, e.g., SCONE, a protocol based on QUIC header extension suitable for mobile network, is being standardized in IETF that can leverage on-path network elements (e.g., UPFs) to offer the runtime status from network’s perspective.

In summary, it is recommended to include the corresponding item in the WT.

# **** First Change ****

# Annex A.X. WT#1.2 User Plane Architecture

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied taking the 5GS user plane framework as a starting point for discussion:

- Whether and how to enhance CP-UP functional split for better multi-vendor interoperability.

NOTE x1: Study whether the N4 interface may be extended to be an SBI.

- Whether and how user plane flexibility (by user plane function (re)selection) for different service requirements, session continuity and mobility can be enhanced with consideration of path performance and capability between access network and data network.

NOTE z: The outcome of this study may include architectural requirements that could influence the design of user plane protocols in stage 3 working groups.

- Study whether and how to improve UP architecture for resiliency, scalability, efficiency and load balancing, compared with 5G.

- Study whether there are any UP architectural requirements and/or improvement that could influence the design or choice of user plane transport technology(ies) in the 6G network.

NOTE x2: SA WG2 will not design or select user plane transport protocol(s). However, an output of this work task may be architectural requirements that stage 3 working groups may consider when making transport protocol related decisions.

**** End of Changes ****