3GPP TSG-SA WG2#172	S2-2510406

Dallas, USA, 17-21 November 2025	(was S2-2508xxxx)

Source:	Qualcomm Incorporated

Title:	KI#1: Interim agreements

Document for:	Discussion/Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC/ Rel-20

Abstract of the contribution: Interim conclusions on KI#1: ADC establishment in alerting phase for early media.

# 1.	Discussion

Multiple solutions have been agreed on KI#1: ADC establishment in alerting phase for early media.

More specifically, Solution #2 focus on the scenario of NW initiated data channel application in early media session with subscription.

Based on the solutions outlined above, it is recommended that following principles are adopted as part of the interim agreements regarding KI#1.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-19:

# >>>>BEGINNING OF CHANGES (all new text) <<<<

### 7.1.Y	Agreed Principles for KI#Y

The following interim conclusions for Key Issue #1 ADC establishment in alerting phase for early media are made:

-	For NW-initiated data channel use in early media sessions:

-	Users subscribe to Data Channel Application in Alerting Phase (DCAP) service and set DC application preferences in HPLMN; these settings are stored in the HSS as part of IMS subscription data.

-	Upon receiving an INVITE, IMS AS activates DCAP service with the application data channel to the target UE based on configuration.

-	IMS AS updates SDP with indication of DCAP service to add/delete the pre-configured application data channel during the early session.

-	The application data channel is activated only after IMS AS receives 180 Ringing from the serving UE.

-	The DCAP service can coexist with other early session services, such as CAT and CRS.

# >>>>END OF CHANGES<<<<