3GPP TSG-SA WG2 Meeting #172	S2-2510513

13 - 17 November, 2025, Dallas, USA

Source: 	Samsung

Title: 	wayforward on conclusion for KI#1 IMS voice data transport

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes to delay the decision of IMS voice data transport over GEO link for normative phase until getting feedback from SA1 and SA4.

1	Discussion

Based on above key interim conclusion for IMS voice data and the solutions on the table, further interim conclusions need to be provided for following key points on KI#1:

- IMS voice data over IP(i.e. RoHC) vs non-IP(i.e. reduced header)

Question 2 (To RAN1): Can RAN1 provide any data regarding the probability that such number of consecutive packets e.g. 16 or 64 can be lost or erroneously decompressed? How often such event can occur?

RAN1 reply:

Assuming line-of-sight condition for the duration of the call, no change in large scale parameters (e.g. fixed shadowing), and using an NTN TDL-C channel model (which is a channel model previously used by RAN1 and SA4 evaluations), the probability of having 16 or 64 packets consecutively lost is negligible under typical operating conditions (e.g. up to 10% packet error rate)

For the case of line-of-sight not being guaranteed for the duration of the call, an intermittent blockage (e.g. according to the land mobile satellite channel model in TR 38.811) may still occur in practice, in which case the probability of having 16 or 64 packets consecutively lost may not be negligible. RAN1 has not reached consensus on whether to consider this case.

per LS response from RAN1(R1-2508096),

Since such more than 16 consecutive loss implies that SN of RoHC can be mis-synchronized. So we may interpret as

- ROHC performance stability can be considered guaranteed for LOS scenarios. However for NLOS scenarios, ROHC performance stability cannot be guaranteed.

It looks apparent drawback of the RoHC option. And, we are still waiting for LS responses from SA1 and SA4 on the feedback for feasibility check on non-IP approach.

In short we can not proceed having both options as normative way forward because the multiple options could make the UE and system unnecessarily complicated.

Hence, we’d like to propose to postpone the decision between options and how to handle the serious problem in ROHC until receiving LS reply on their feasibility analysis from SA1 and SA4.

2 Proposal

It is proposed to capture the following contents into TR 23.700-19 v1.1.0.

# * * * * First Change* * *

8.1	Interim conclusions for Key Issue #1

Editor's note:	Other interim conclusions for Key Issue#1 is FFS.

The following interim conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

NOTE: which voice packets transport over the NB-IoT (GEO) user plane shall be determined considering feedback from SA1 and SA4 to S2-2507636 between :

a)	mechanism for IP packets using Robust Header Compression (RoHC); and

b)	mechanism for Non-IP packets using removal of parts or all of RTP/UDP/IP headers and restoration in the UE and network.

# * * * * End of change * * * *