3GPP TSG-WG SA2 Meeting #172	S2-2510663

# Dallas, US, 17th Nov – 21st Nov, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0136 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on AIOTF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specification does not clearly define the AIOTF selection procedure when an AIoT service operation request includes AIoT Device Filter Information. The following clarifications should be specified: Case 1: If the request includes both AIoT Device Filter Information and External Target Area information, the NEF determines AIOTF instances(s) by providing the NRF with Target Area information and the NRF returning AIOTF instance(s) that match the provided Target Area information. Case 2: If the request includes only AIoT Device ID filter information (without External Target Area information), the NEF determines the AIOTF instance(s) based on locally configured information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The specification should clearly define the AIOTF selection mechanism when the AF provides AIoT Device ID filter information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current ambiguity in the specification may lead to AIOTF selection failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.3.1	AIOTF Discovery and Selection

The AIOTF discovery and selection functionality is to determine an AIOTF(s) to handle an AIoT service operation request.

The NEF determines AIOTF instances(s) by providing the NRF with Target Area information and the NRF returning AIOTF instance(s) that match the provided Target Area information, or by using local configuration (e.g., if AF includes Filtering Information in the Information about the target AIoT Device(s) in a service operation request).

A service operation request received by the NEF from an AF may include External Target Area information and the NEF uses it to determine the Target Area information that is provided to the AIOTF and NRF, if used. The External Target Area information is a pre-configured External Area Identifier or geographic area (e.g., a civic address or GAD shapes). The Target Area information is a list of AIoT Areas.

NOTE:	The mapping between AIoT Areas and External Area Identifiers information provided by an AF is configured in the NEF.

When an AIoT service operation request indicates individual AIoT Device(s), the AIOTF instance(s) may be selected by taking into account the last known AIOTF instance(s) (e.g., AIOTF ID/address) for those AIoT Device(s) obtained from the ADM.

# * * * * End of changes * * * *