3GPP TSG-SA WG2#172	S2-2510764

Dallas, USA, 17-21 November 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	FS_Sensing_ARC: Additional principles for KI#3 on Sensing Entity and Sensing Function Discovery and (Re-)Selection

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_Arc / Rel-20

Abstract of the contribution: This contribution proposed additional principles for the KI#3 on Sensing Entity and Sensing Function Discovery and (Re-)Selection.

# 1.	Discussion

In SA2#171, several general principles were agreed for the KI#3 for FS_Sensing_Arc on Sensing Entity and Sensing Function Discovery and (Re-)Selection. This contribution proposed the additional principles based on the architecture discussion in KI#1.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-14:

# >>>>BEGINNING OF CHANGES<<<<

### 7.1.3	Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection:

-	The Sensing Function selection is based on NEF's local configuration or NRF query.

-	The Sensing Function registers itself to the NRF, with its NF profile.

-	Sensing Function is selected based on at least its supported sensing area.

-	Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request.

-	Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

- The Sensing Function obtains the information about the Sensing Entities via either OAM or during the direct connection establishment.

# >>>>END OF CHANGES<<<<