3GPP TSG SA WG2 Meeting #87	TD S2-113970

10- 14 October 2011, Jeju, South Korea

Source:	ZTE

Title:	Clarifications on How the HeNB to get the FQDN of the fixed Broadband access networks

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	BBAI / Rel-11

Abstract of the contribution: This contribution proposes to add some descriptions for clarification on how the HeNB to get the FQDN of fixed broadband access networks.

Introduction：

It has been decided that the FQDN of the fixed Broadband access networks, if available, is sent from HeNB to HeNB GW/MME in the procedures in TS 23.139 for femto scenarios. But it is not clear how the H(e)NB gets this information. This contribution clarifies that the FQDN of the fixed broadband access network is preconfigured in the H(e)NB or provisioned by H(e)MS. If the FQDN is available in the H(e)NB, the HeNB may send this information to MME.

/************************** Start of Changes ***************************/

### 9.1.1	E-UTRAN Initial Attach

This clause is related to the case when the UE performs initial attachment to the E-UTRAN network via a HeNB. The HeNB is already registered in the network.

NOTE: 	Enhancements to the Initial Attach procedure rest on the assumption that the HeNB sends the HeNB local IP address, the UDP port if NAT/NAPT is detected, and/or the FQDN of the fixed Broadband access networks to the MME in UE associated S1 signalling (refer to the definition of Initial UE Message in TS 36.413). The FQDN of the fixed broadband access networks is optionally preconfigured in the HeNB or provisioned to the HeNB by HeMS.

Figure 9.1.1: UE Attach procedure via HeNB

Step 1 The description of this step is the same as for step 1 in TS 23.401, clause 5.3.2.1

Step 2 The description of this step is the same as for step 1 in TS 23.401, clause 5.3.2.1 with the following additional information the HeNB local IP address, the UDP port if NAT/NAPT is detected, and/or the FQDN of the fixed Broadband access networks.

Steps 3-14  are the same as in TS 23.401 clause 5.3.2.1 with the following additional information included in Create Bearer Request message sending between MME and SGW, and between SGW and PGW: HeNB Local IP address, the UDP port if NAT/NAPT is detected, and/or the FQDN of the fixed broadband access network.

.Step 14.1 The PCRF sends the Gateway Control Session establishment/QoS Rules provisioning message to the BPCF per TS 23.203.  The message includes the IMSI, HeNB local IP address and UDP port if NAT/NAPT is detected.

Once this step completes the PCRF is aware the fact that the UE is connected to the network via a 3GPP Femto and checks first whether sufficient resources are available in the BBF access before it provisions PCC rules at the PCEF.

14.2. The PCRF acknowledges the IP CAN session establishment/modification.

/************************** End of Changes ***************************/