| 3GPP TR 23.801-01 V0.3.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Architecture for 6G System; Stage 2 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Architectural Assumptions and Requirements	8

4.1	Architectural Assumptions	8

4.2	Architectural Requirements	9

5	Key Issues	9

5.1	Key Issue #1: Study the support for control signalling for 6G System	9

5.2	Key Issue #2: SBA framework	10

5.3	Key Issue #3: Support of Network Slicing in the 6G system	10

5.4	Key Issue #4: User Plane Architecture	10

5.5	Key Issue #5: QoS Framework for 6G	11

5.6	Key Issue #6: Policy and charging control framework	11

5.7	Key Issue #7: Network Exposure	12

5.8	Key Issue #8: Network Sharing in the 6G system	13

5.9	Key Issue #9: Localized Service Access	14

5.10	Key Issue #10: FWA / Fixed Wireless Access	14

5.11	Key Issue #11: Support of non-3GPP access	14

5.12	Key Issue #12: Voice Services for 6G	15

5.13	Key Issue #13: Emergency Voice Services for 6G	15

5.14	Key Issue #14: Location Services for 6G	16

5.15	Key Issue #15: Messaging Services for 6G	16

5.16	Key Issue #16: Other Essential/Regulatory Services for 6G	16

5.17	Key Issue #17: Migration and Interworking	16

5.18	Key Issue #18: AI for 6G architecture	17

5.19	Key Issue #19: 6G Network for AI	18

5.20	Key Issue #20: Integrated Sensing and Communication	19

5.21	Key Issue #21: 6G data framework in SA2	19

5.22	Key Issue #22: 6G Computing Support	20

5.23	Key Issue #23: Support of 6G NTN	21

5.24	Key Issue #24: Analyse 5GS IoT features and solutions	21

5.X	Key Issue #X: Key Issue Title	22

6	Solutions	22

6.0	Mapping of Solutions to Key Issues	22

6.X	Solutions to KI#X	22

6.X.Y	Solution #X.Y: <Solution Title>	22

6.X.Y.0	High-level solution Principles	22

6.X.Y.1	Description	22

6.X.Y.2	Procedures	22

6.X.Y.3	Impacts on Services, Entities and Interfaces	22

7	Interim agreements	23

7.1	Agreed Principles	23

7.1.Y	Agreed Principles for KI#Y	23

8	Consolidated 6G architecture interim agreements	23

9	Conclusions	23

Annex A:	Work Tasks	24

A.0	Introduction	24

A.0a	Mapping between Work Tasks and Key Issues	24

A.X	Work Task X	24

A.1	Work Task 1	25

A.1.1	Work Task 1.1: Study the support for control signalling for 6G System	25

A.1.2	Work Task 1.2	25

A.1.2.1	SBA framework	25

A.1.2.2	Network Slicing	25

A.1.2.3	User Plane Architecture	26

A.1.2.4	QoS Framework for 6G	26

A.1.2.5	Policy Framework	26

A.1.2.6	Network Exposure	28

A.1.2.7	Network Sharing in the 6G system	29

A.1.2.8	Localized service access	29

A.1.2.9	FWA	29

A.1.3	Work Task 1.3: Non-3GPP access	29

A.1.4	Work Task 1.4: 6G Essential Services	30

A.2	Work Task 2: Migration and Interworking	30

A.3	Work Task 3: AI	31

A.3.0	General	31

A.3.1	WT#3.1: AI for 6G architecture	31

A.3.2	WT#3.2 6G Network for AI:	32

A.4	Work Task 4: Integrated Sensing and Communication	33

A.5	Work Task 5: 6G data framework in SA WG2	33

A.6	Work Task 6: 6G Computing Support	34

A.7	Work Task 7: Support of 6G NTN	35

A.8	Work Task 8: Cellular IoT enablers in 6G	35

Annex B:	Topics for further resolution	37

B.X	Topics for further resolution for KI#X	37

Annex C:	Change History	38

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The study will work towards goals endorsed at TSG#107 (Mar 2025) to "create lean and streamlined standards for 6G, e.g. by dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations, etc. Any exception to the above shall be well justified".

# 1	Scope

The present document studies system architecture for 6G mobile networks for improvement of existing services and support of new services, to meet the 6G system requirements as captured by TS 22.abc [x].

Editor's note:	The above reference should be replaced with a reference to normative specification, when available. Until the normative work is available, the study can take into account progress made in SA WG1 and TSG RAN.

The study investigates the architectural requirements, assumptions, high-level principles, key issues, as well as solution and system architecture recommendations for 6G architecture.

The conclusions of this study will form the basis for the normative work.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[6]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[7]	RP-252912: "Revised SID: Study on 6G Radio", 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025.

[8]	3GPP TS 22.101: "Service aspects; Service principles".

[9]	3GPP TS 22.261: "Service requirements for the 5G system".

[10]	3GPP TS 22.105: "Services and service capabilities".

[11]	3GPP TR 22.870: "Study on 6G Use Cases and Service Requirements".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

6G Core Network (6G CN): In this study, it refers to the Core Network for the 6G System.

NOTE 1:	To what degree 6G CN can be considered as evolution of 5GC or as a new Core Network will be determined during the study.

6G RAN: In this study, it refers to the radio access network for 6G radio access technology (6GR) being studied by RAN WGs RP-252912 [7].

6G System (6GS): 3GPP System consisting of 6G RAN, 6G CN and UE and supports also connectivity of UEs via specific non-3GPP access(es).

NOTE 2:	Which specific non-3GPP access(es) are supported is studied under WT#1.3.

Non-Access Stratum: The Non-Access Stratum (NAS) is the stratum for the control plane between the UE and the 6G CN.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

Editor's note:	This clause documents the common architecture assumptions identified for the study.

The following is assumed for 6GS architecture:

1.	The framework of SBA specified for 5GC is assumed as a starting point for discussion.

2.	This study assumes that control and user plane of the 6G RAN connects to a single core network type, i.e. 6G CN. It is assumed that the 6G RAN is a single technology framework based on a stand-alone architecture.

NOTE:	This does not exclude network sharing scenarios such as Multi-Operator Core Network.

3.	The 6G system is assumed to maintain the RAN and CN functionality split as in 5GS.

4.	The study aims to avoid duplication of functionality between 6G RAN and 6G CN.

5.	Real-time voice and video services under operator's control and in 3GPP scope (MMTel) will be provided by IMS.

6.	The 6G system is assumed to natively support both Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN).

## 4.2	Architectural Requirements

The followings are architectural requirements of 6G System:

The 6G System shall:

1.	Support roaming.

2.	Support multi-vendor interoperable interfaces.

The 6G System should:

3.	Follow design principles such as: utilization of cloud nativeness, improve sustainability and energy efficiency, robustness and resiliency.

4.	Minimize inter dependencies between NFs when introducing a new feature or procedure for an NF.

The architectural requirements apply to all WTs and corresponding key issues in this study.

# 5	Key Issues

Editor's note:	This clause identifies key issues to be addressed.

## 5.1	Key Issue #1: Study the support for control signalling for 6G System

Key Issue #1.1

1.	Study the support for control signalling for 6G System, including at least the following:

a)	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE 1:	It is assumed that this key issue KI#1.1 bullet 1a covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2:	For the above KI#1.1 bullet 1a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

NOTE 3:	6G System Procedures will include interaction between RAN-CN, interaction within CN NFs. The signalling interaction between RAN-CN need coordination and alignment with RAN WG3.

NOTE 4:	Overall Interworking and migration aspects are covered by key issues for WT#2 (KI#17). Impact to IWK due to features introduced by this key issue will be covered by this key issue in alignment with WT#2 (KI#17).

Key Issue #1.2

1.	Study the support for control signalling for 6G System, including at least the following:

a)	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 5:	This KI#1.2 bullet 1a can include any transport mechanism such as using NAS or UP or new plane. KI#1.2 bullet 1a can have dependency on other key issues defined for e.g. KI#1.1 bullet 1a, KI#18, KI#19, KI#20, KI#21, KI#22.

NOTE 6:	6G System Procedures will include interaction between RAN-CN, interaction within CN NFs. The signalling interaction between RAN-CN need coordination and alignment with RAN WG3.

NOTE 7:	Overall Interworking and migration aspects are covered by key issues for WT#2 (KI#17). Impact to IWK due to features introduced by this key issue will be covered by this key issue in alignment with WT#2 (KI#17).

NOTE 8:	This KI#1 covers operator services.

NOTE 9:	Solutions may apply for only KI#1.1 or only KI#1.2 or both KI#1.1 and KI#1.2.

## 5.2	Key Issue #2: SBA framework

The following is key issue for SBA related work task.

1	Study whether and how to optimize NF/NF service registration, discovery and selection for efficient message forwarding compared with 5G.

2	Study whether and how to improve NF/NF service resiliency, scalability, efficiency and load balancing, compared with 5G.

NOTE:	This key issue covers generic aspects for SBA framework. The study of other key issues can also result in potential enhancements on the SBA framework and will be studied as part of respective key issues.

## 5.3	Key Issue #3: Support of Network Slicing in the 6G system

This Key Issue aims to study the support of Network Slicing in the 6G system, including the following aspects:

1.	Study the overall design and functionalities of the network slicing in 6G assuming the network slicing in 5GS as starting point for discussions including the following:

a)	Identify and address any areas of possible improvement and simplification of network slicing.

b)	Identify and address any improvements of the application traffic mapping to network slice(s)/user plane connection(s).

NOTE 1:	Any feedback from network slicing deployments can be considered in order to facilitate improvements.

NOTE 2:	Coordination between bullet 1b and the KI#6 on Policy is expected.

2.	Study impacts of the network slicing in 6G on the interworking and migration.

NOTE 3:	Impacts to RAN are possible and coordination with RAN WGs is expected.

## 5.4	Key Issue #4: User Plane Architecture

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied taking the 5GS user plane framework as a starting point for discussion:

1.	Whether and how to enhance CP-UP functional split and interaction for better multi-vendor interoperability.

2.	Whether and how to enhance user plane flexibility (by user plane function (re)selection) for different service requirements, session continuity and mobility with consideration of user plane function capability and path performance between access network and data network.

NOTE 1:	This work task may require coordination with RAN WGs for user plane interface.

3.	Whether and how to enhance resilience, scalability, and high availability of user plane function.

NOTE 2:	The outcome of this study may include architectural requirements based on which stage 3 working group can work on the user plane protocols. The stage 3 protocol aspects, including how to design and select user plane protocols, are not in the scope of stage 2.

## 5.5	Key Issue #5: QoS Framework for 6G

This key issue is based on the WT#1.2 on the QoS Framework for 6G. This KI includes following aspects:

1.	Investigate whether and what new functionality in 6G QoS framework is required considering, the emerging new traffic characteristics and application needs, e.g. AI/ML application traffic.

NOTE 1:	Which working groups will determine the characteristics of AI traffic is subject to TSG SA plenary (SA#110) decision.

NOTE 2:	Coordination with KI#19 on 6G network for AI may be needed.

2.	Whether and how to enhance QoS mechanisms to:

a)	support QoS targets fulfilling application’s QoS requirements (e.g. dynamic QoS requirements) in a less resource intensive manner than the existing GBR, Delay Critical-GBR.

3.	Study whether and what enhancements are needed to adjust the QoS targets, in case the current QoS targets cannot be met, to minimize the impacts to application operation or user experience.

4.	Study whether and how to support the UE, application and network QoS collaboration to improve awareness in the 6GS of application traffic needs, e.g. traffic pattern, dynamic QoS requirements and to improve awareness in the application/UE of what can be provided by the network, e.g. maximum bitrate.

NOTE 3:	Applications consist of UE aspect and DN aspects.

5.	Whether and how to enhance QoS monitoring, based on the identified needs, e.g. to determine the end-to-end per-packet delay, packet loss rate between the PSA-UPF and the UE for the verification of the QoS target fulfilment.

6.	Study whether and what enhancements are needed to enforce QoS differentiation, e.g. in case of frequent address change of the server, application traffic with multiplexed media flows.

NOTE 4:	QoS aspects of interworking will be considered based on the KI#17 for migration and interworking.

NOTE 5:	This KI will coordinate with SA WG4 and RAN WGs.

NOTE 6:	The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion.

NOTE 7:	This KI may study impacts to the QoS for 6G based on the input identified and provided by other KIs.

## 5.6	Key Issue #6: Policy and charging control framework

To support the Policy and charging control in 6GS, the policy and charging control framework defined in 5G for policies (i.e. SM Policy, UE Policy, AM Policy) is considered as starting point for discussion, the following aspects are to be studied:

1.	Whether and how to simplify the policy and charging control framework for 6G, including:

a)	whether and how to reduce the number of policy control services e.g. the same policy control association for both AM and UE policies or the number of interactions to efficiently provide policies to the different policy enforcement points.

NOTE 1:	The example above does not preclude that other examples are covered in the study phase if the reason for simplification is identified.

b)	whether and how to support the UE to request UE policies from the 6G CN and/or the 6G CN to provide UE policies to the UE.

2.	Whether and how to consider user preferences during UE policy evaluation at UE, without conflicting with network provided UE policies.

NOTE 2:	The statement that refers to considering user preferences refers to the ability to influence the evaluation of UE policies.

3.	Whether and how to improve the External Parameter Provisioning aspects of policy control for 6G.

NOTE 3:	General Authorization of the external parameter provisioning is handled in the KI#7 - Network Exposure.

4.	Whether and how to improve events notification about policies to the AF (e.g. to notify the AF that policy and charging control, e.g. charging key changed).

5.	Whether and what parameters the UE can provide as input for PCC decision, e.g. related to QoS control to the 6G CN. The UE input for PCC decision related to QoS is based on the progress in KI#5 - QoS.

NOTE 4:	The example above does not preclude that other examples are covered in the study phase if those are identified.

NOTE 5:	The AF input for session management defined in 5G can be treated as starting point for discussion.

NOTE 6:	The UE can decide if and when to provide the above input the request to the network.

NOTE 7:	The UE is not assumed to know whether AF input for QoS is available.

NOTE 8:	Whether the UE can provide any input for PCC decisions as the AF provides for UE or AM policies are out of the scope of this bullet.

NOTE 9:	The assumption is that either the AF or the UE provides input for PCC decision for e.g. QoS control.

The following notes are applicable to all bullets in this key issue:

NOTE 10:	If the energy related information, either based on 5G mechanisms, or 6G mechanisms, is available then this key issue will study how to take energy efficiency and consumption aspects into account for policy decisions.

NOTE 11:	This key issue has IWK aspects that are scoped under KI#17 - Migration and Interworking.

NOTE 12:	This key issue can study potential enhancements to the policy control framework for 6G services based on the input identified and provided by other key issues.

NOTE 13:	This key issue depends on the overall 6G roaming architecture based on the output of KI#1 - Support of 6G Control signalling. Solutions for this key issue will address roaming if applicable.

NOTE 14:	Any improvements of the application traffic mapping to network slice(s)/user plane connection(s) are out of the scope of this key issue.

NOTE 15:	Identify and address any feedback from markets where policy control framework technologies are already being commercially deployed.

## 5.7	Key Issue #7: Network Exposure

This key issue is to study whether and how to support a common network exposure framework in 6G. The capabilities that are exposed via the framework fall into one of two categories.

1.	The first category is new capabilities. New capabilities refer to capabilities that are not exposed in the 5G network. This category of exposed capabilities is dependent on other key issues. This key issue will include a review of exposure requirements that derive from other key issues.

2.	The second category refers to capabilities that are exposed in the 5G network and are used as a starting point for discussion of this key issue. This category of exposed capabilities may be dependent on other key issues.

Since this key issue depends on other key issues, time will not be allocated for solutions for this key issue by SA WG2 before SA2 #175.

NOTE 1:	This key issue requires coordination with SA WG3, SA WG6, and SA WG5.

NOTE 2:	This key issue has dependency on service requirements to be specified by SA WG1.

NOTE 3:	Information exposure is based on operator policies/SLAs.

NOTE 4:	Authorization, Privacy protection and user consent aspects require coordination with SA WG3, and application layer user consent aspects may require coordination SA WG6.

Aspects that can be studied under this key issue are:

1.	How to support a common network exposure framework in 6G for the exposure of network capabilities to an AF.

a)	Whether and how to support a common network exposure framework in 6G for the exposure of network capabilities to AFs, in an access independent manner (to the extent that exposure in an access independent manner is possible). The exposure framework of the 5GS (i.e. exposure via the NEF) is assumed to be the starting point for discussion of this key issue.

b)	Whether and how to support co-existence with the 5G exposure framework.

NOTE 5:	The common network exposure framework in 6G in SA WG2 does not duplicate the SA WG6 Framework.

NOTE 6:	Examples of network capabilities include information exposure such as information about UE related events, network events, analytics, and information provisioning such as UE related network configuration parameters and service specific parameters.

c)	Whether and how to support privacy protection, user consent, data anonymization, and service authorization mechanisms.

d)	Whether and how to restrict Network Exposure procedures in roaming, aiming to avoid information being sent between the home network and visited network when not allowed or when not necessary.

2.	How to support Capability Exposure to the UE and Application Endpoints.

Depending on the outcome of the QoS key issue, this key issue is about whether and how to integrate solutions from the QoS key issue for collaboration between the UE, applications and network in a common exposure framework. As a prerequisite for this key issue, the QoS key issue is expected to determine if specific information needs to be exchanged. This key issue will cover:

a)	Based on the work in the QoS key issue, whether and how to integrate solutions from the QoS key issue into a common exposure framework in 6G in order to improve awareness in applications and the UE of what information can be provided by the network.

NOTE 7:	Exposure should be done in an operator-controlled manner.

NOTE 8:	Solutions to this key issue can be based on the control plane and/or user plane.

b)	Whether there is a need to and how to have the common framework support how to discover what information can be provided by the network and authorize the network to provide specific information to the applications and the UE.

3.	Whether and how to support Intent-Based Exposure.

a)	Whether, how, and what intents should be supported on the northbound interface (i.e. the interface to an AF in a DN) of a common network exposure framework in 6G. The intent-based work in this key issue is limited to the northbound interface of a common network exposure framework in 6G and needs to be coordinated with the AI key issue. AI aspects will be covered under the AI key issues and will not be covered under this key issue.

## 5.8	Key Issue #8: Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

1)	How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

## 5.9	Key Issue #9: Localized Service Access

The scope of localized service access is to study the overall architecture design and enablers for localized service access in 6G, which consists of the following:

1.	Whether and how to support localized service access provided via PLMN, and study how to perform authorization and authentication of UE for localized service access, and minimize service interruption during UE mobility, including:

a)	Whether, how and which aspects (e.g. provisioning, updating, retrieval) for the part of subscription (that is needed for localized service access) can be performed/managed via the NFs present locally in the network providing localized service access.

NOTE 1:	Network selection and access control are not to be studied for bullet 1 and bullet 2.

NOTE 2:	For the case when localized service access is provided via PLMN, solutions are expected not to introduce additional UE functionality that is specific for localized service access (i.e. anything that requires UE dependency beyond what is needed for general PLMN access).

NOTE 3:	Authentication aspects need to be coordinated with SA WG3

2.	Study whether and how to make use of 5G NPN (SNPN and PNI-NPN) for localized service access in 6G.

NOTE 4:	Which of the existing features will be supported will be determined during the study.

NOTE 5:	Access control principles defined for NPN (SNPN and PNI-NPN) in 5G are assumed to be reused. Network selection principles defined for SNPN are assumed to be re-used, but remit stays within CT WG1.

NOTE 6:	Session and service continuity between PLMN and SNPN using direct interface between PLMN and SNPN is out of scope of this work task.

## 5.10	Key Issue #10: FWA / Fixed Wireless Access

The key issue aims to:

1)	Analyse issues encountered in 5G deployments to efficiently support FWA and determine requirements to be taken by other KIs for 6G. The result of this analysis will serve as the basis for architectural requirements to relevant Key Issues related with WT 1.1 and WT 1.2.

NOTE:	As the result of this analysis will serve as the basis for architectural requirements to Key Issues related with other Work Tasks, this analysis needs to conclude for June 2026.

## 5.11	Key Issue #11: Support of non-3GPP access

NOTE 1:	The UE can access the 6G CN via non-3GPP access even if there is no 3GPP access available.

NOTE 2:	Coordination with SA3 for authentication aspects will be needed for this KI.

NOTE 3:	The dependency/interaction of non-3GPP access and 3GPP access should be minimized.

For non-3GPP access:

1.	Study how to support untrusted non-3GPP access in 6G System architecture.

2.	Study how to support service continuity between 3GPP access and non-3GPP access in the above bullet(s).

3.	Study how to support Interworking aspects between 5GS and 6GS related to non-3GPP access.

NOTE 4:	Coordination with other KIs is needed.

NOTE 5:	Solutions to above bullets may support connectivity via non-3GPP access only.

NOTE 6:	Solution discussion on Simultaneous connectivity via 3GPP and non-3GPP access with only per flow granularity can only start from SA2#175, but with low priority at SA2#175 if time allows.

For Non/Seamless WLAN Offload:

4.	How to support UE policy necessary for Non-Seamless WLAN Offload (NSWO).

## 5.12	Key Issue #12: Voice Services for 6G

This Key Issue will investigate how to support voice services for the 6G system (referring to the mechanisms used in 5GS to support voice services as the starting point for discussion) - the following aspects will be studied:

1.	How the 6GS natively supports IMS based voice over the 6G access network, including how a UE supporting 6G RAN selects and connects to 6G network for voice service.

2.	Whether and how to update the IMS specification to support an IP-CAN in the 6GS.

3.	How to support the voice continuity between 6G RAT and selected 3GPP RATs or between 6G RAT and selected non-3GPP Accesses when 6G RAT supports voice natively.

4.	How to support migration to Vo6G.

5.	Whether and how to support voice services if they are not provided in the serving 6G Network (Assuming this covers both domain selection and RAT/System selection at the time of the call).

6.	How to support the compatibility between 6G CN and the existing deployed IMS with minimum impacts on the existing deployed DIAMETER interfaces (e.g. Rx, Cx, Sh).

NOTE 1:	For bullet 6, coordination with CT is required, including to consider whether SA WG2 or a CT WG leads the study.

NOTE 2:	Support of voice services via 6G NTN need to be coordinated with WT#7 (KI#23).

NOTE 3:	Interworking, migration and service continuity aspects need to be coordinated with WT#2 (KI#17). Interworking with 2G/3G and CS (Circuit switch) for voice are not considered in this Key issue.

NOTE 4:	The existing IMS architecture is used as a starting point for the study.

NOTE 5:	The study will be aligned with RAN and coordinated where needed.

NOTE 6:	This Key issue should consider roaming aspects.

## 5.13	Key Issue #13: Emergency Voice Services for 6G

This key issue will investigate how to support emergency voice services over the 6G system using the capabilities defined for voice service in Key Issue 12.

Additionally, this key issue will study:

1.	The functionality required in the UE and the 6G system to enable emergency voice service for 6GS.

2.	The functionality required in the UE and the 6G system to establish emergency session via 6GS for the emergency voice service.

3.	How to identify emergency services in 6GS in case of non-UE detectable emergency call, such that the required treatment for an emergency call is applied.

4.	How to support a UE in Limited-Service state so that it can establish emergency session via 6GS.

5.	Whether and how to support emergency voice services if they are not provided in the serving 6G Network (Assuming this covers both domain selection and RAT/System selection at the time of the call).

## 5.14	Key Issue #14: Location Services for 6G

This Key Issue will investigate how to support location service in 6G - the following aspects will be studied:

1.	The architecture to support location services in 6GS.

2.	How to support regulatory location services for emergency voice service in 6GS.

3.	How to support location service exposure.

NOTE 1:	The focus in this release of the specifications will be on the required location service capability to support emergency and other regulatory services.

NOTE 2:	A unified location service architecture should be pursued (using 5GS as the starting point for discussion), by considering the location service requirement in this KI and from other KI(s) if any.

NOTE 3:	Coordination with RAN WG for RAN aspects, SA3 for privacy and user consent aspects and SA5 for OAM and charging aspects will be needed.

## 5.15	Key Issue #15: Messaging Services for 6G

For messaging service in 6G this Key Issue will study:

1.	How to support short message services (SMS) in 6GS as per the service requirements specified in TS 22.101 [8], TS 22.261 [9], TS 22.105 [10].

NOTE:	For SMS to Emergency centre support it is assumed that the outcome of 5G-Advanced Rel-20 study will be adopted also for 6G.

## 5.16	Key Issue #16: Other Essential/Regulatory Services for 6G

This Key Issue will study the following services:

1.	How to support Multimedia Priority Services (MPS) in 6GS.

2.	How to provide priority treatment to access control, signalling and media packets delivery for Mission Critical Services (MCX Services) in 6GS.

NOTE 1:	It is assumed that 6GS will support Public Warning System (PWS). Stage 2 capabilities for PWS will be specified by TSG CT.

NOTE 2:	The scope will keep alignment with TSG RAN.

## 5.17	Key Issue #17: Migration and Interworking

This key issue studies how to support migration and interworking, including:

1.	How to support migration to 6GS.

2.	How to support interworking with 5GS.

3.	Whether and how to support interworking with EPS.

NOTE 1:	Whether to support interworking with EPS will depend on SA WG1 requirement.

4.	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is required to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:	The detailed migration study scope and migration options will be coordinated and aligned with RAN.

NOTE 4:	Additional migration options beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR, will be studied no earlier than June 2026 in alignment with TSG RAN (if needed).

NOTE 5:	It is assumed that interworking for roaming is within the scope of this work task.

NOTE 6:	This work task focuses on general interworking procedures. The specific interworking aspects studied in other KIs (if any) need to follow the general interworking procedures.

## 5.18	Key Issue #18: AI for 6G architecture

Study how to support and enable use of AI in 6G (e.g. AI agent, AI framework).

NOTE 1: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA WG1 work is still in progress, adapting the definition of AI agent from SA WG1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

The AI for the 6G architecture shall be multi-vendor interoperable, reliable and sustainable.

NOTE 2:	It is assumed that operators can decide whether and which AI capabilities and technologies to deploy in their network.

Study whether and how to provide an architecture for AI to fulfil the following:

1.	Enable the 6G CN to leverage AI capabilities and technologies in the 6G CN (e.g. AI agent), subject to operator policies and configuration and using 6G CN functionalities available in the network:

a)	Determine how to fulfil requests from the UEs or AFs when intent is included and, in order to ensure interoperability of intents and supporting system test cases, determine the constraints on the use and expression of intents sent from the UEs and the AFs so that the intents can be processed and interpreted unambiguously by the AI capable entities in the 6G CN, and define the mechanisms required to support these constraints.

b)	Determine how to fulfil requests when intent is not included.

c)	In order to enable AI capable entities in 6G CN to dynamically compose parts of procedures to fulfil requests from UEs and AFs, determine the design principles and constraints for the modularisation of the procedures for the 6G CN that will be applicable to the system procedures defined in the normative work. How to enable the AI capable entities to compose these procedures in SA2 specifications will be determined by the study.

NOTE 3:	An intent refers to expectations including requirements, goals, conditions, guidelines, and constraints, without specifying how to achieve them. What intent means and how to specify intents in SA2 specifications will be determined by the study. Intent is to be specified as a result of this key issue.

NOTE 4:	Whether to support intents in a 6G CN is up to operator choice. It is assumed it is not required for the MT stack of UE to produce nor understand the intent. The MT stack of UE is assumed to be agnostic to whether or not the network uses 6G CN AI capable entities (e.g. AI agent, AI-enabled NFs) to address UE requests not including intent.

NOTE 5:	This should not result in duplicated procedures in 3GPP specifications. It is assumed that the 6G study will still describe 6G system procedures for the 3GPP features, without necessarily requiring AI capabilities.

2.	Enable closed-loop operations and learning techniques such as reinforcement learning, in 6G CN.

3.	Enable entities in 6G CN to access network AI capabilities provided by 6G NFs.

4.	Enable AI capable entities in 6G CN to access trusted external capabilities provided by AF.

NOTE 6:	Whether and what external capabilities provided by AF is accessed by AI capable entities will be discussed as part of the study.

5.	Enable the monitoring of the performance of all AI capable entities in 6G CN.

6.	Enable the operator to control the network's use of AI capabilities in its 6G CN, i.e. support different operator-configurable levels of autonomy based on operational needs, including the option to use or not use any AI capabilities.

7.	Support roaming scenarios, e.g. how visited networks using or not using AI technologies can interwork with home network using or not using AI technologies.

8.	Enable NFs of the 6G CN to have AI/ML capabilities, ML model provisioning, inferencing, training and monitoring.

9.	Enable the 6G CN AI architecture to ensure interoperability with 5G CN AI architecture if needed, for the purpose of maintaining a consistent service for UEs across 5G and 6G.

NOTE 7:	How to process the intent when AI capabilities are not being used is to be studied as part of this key issue.

NOTE 8:	Aspects related to SA3, SA5, SA6 will be coordinated with each group respectively during the study.

NOTE 9:	Further alignments with SA WG1 consolidated requirements may be needed.

NOTE 10:	This key issue does not cover AI agents in UE, which is covered by KI#19.

NOTE 11:	Coordination across the KIs in the 6G study is needed.

NOTE 12:	This key issue will not duplicate the work in the scope of KI#1.

## 5.19	Key Issue #19: 6G Network for AI

This key issue is based on the WT#3.2 about the 6G network for AI.

NOTE 1: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA WG1 work is still in progress, adapting the definition of AI agent from SA WG1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

1.	Study whether and how to support an AI agent on a UE to discover another AI agent on a different UE via the 6G network(s).

2.	Study whether and how to enable communication for AI agents on different UEs via the 6G network(s) e.g. identification and authorization of an AI agent on a UE.

3.	Study whether and how to enhance network capability exposure functionalities to AI agent on AF(s).

NOTE 2:	Coordination with KI#7 for Network Exposure is required.

NOTE 3:	During the study, which aspects will belong to SA WG2 or SA WG6, or required coordination with SA WG6 on the application layer aspects will be discussed, if needed. And coordination with SA WG3 on security aspects may be needed.

NOTE 4:	When AI agent is on the UE, this AI agent is assumed not part of the MT. It is also assumed that this KI is not replacing UE to Core Network interaction, NAS.

NOTE 5:	The AI agent in this KI is outside of RAN and Core Network for 6G.

4.	Study whether and how the 6G CN can provide AI services (i.e. AI inferencing and AI training) to applications (AF or in UE).

5.	Study the potential system impacts based on the characteristics of AI traffic.

NOTE 6:	Coordination with KI#5 QoS may be needed.

NOTE 7:	System enhancement related to QoS for AI traffic will be handled by KI#5 QoS.

NOTE 8:	Which working groups will determine the characteristics of AI traffic is subject to TSG SA plenary (SA#110) decision.

NOTE 9:	Coordination with SA WG3, SA WG5 and SA WG6 may be needed on AI inferencing and AI training.

NOTE 10:	Coordination with KI#21 and KI#22 may be needed.

NOTE 11:	New service charging part may be coordinated with SA WG5 Charging group.

## 5.20	Key Issue #20: Integrated Sensing and Communication

Study the integration of Sensing and Communication over 3GPP access, covering RAN node and UE operating as 3GPP Sensing Entities in the different sensing modes in alignment with TSG RAN, and considering multiple sources of sensing data.

The following aspects will be studied:

1)	Architecture and functional support for Sensing Service, including:

a)	Sensing Service authorization and revocation.

b)	Discovery and (re-)selection of Sensing function and Sensing Entities (i.e. RAN nodes, UEs), including sensing modes selection.

c)	Configuration parameters and policy provisioning to support sensing service, e.g. to the selected Sensing Entities, Sensing function, etc.

d)	Collection and transport of 3GPP sensing related data for sensing result generation.

e)	Exposure framework of sensing result to the Sensing service consumer (i.e. AFs, UEs, Core NFs).

NOTE 1:	How the Sensing service consumer makes use of the sensing results is not part of this key issue.

2)	Whether and how to support Sensing Service in consideration of the mobility of the UE as sensing entity.

3)	Whether and how to support providing sensing data, which is not obtained from 3GPP radio signals, to the Sensing Function for sensing result generation.

NOTE 2:	This includes sensing data obtained using technologies that are different from 3GPP defined ISAC mechanisms, such as sensing based on IEEE 802.11, camera, sonar, lidar, etc.

NOTE 3:	The scope of this KI assumes RAN node and UE operating as Sensing Entities in different sensing modes in alignment with TSG RAN.

NOTE 4:	Coordination with other KIs will be needed if identified, e.g. Sensing data aspects with KI#21, architectural aspect with KIs corresponding to WT#1, etc.

NOTE 5:	Coordination with RAN WG for RAN aspects, SA WG3 for privacy and user consent aspects, and SA WG5 for OAM and charging aspects will be needed. Coordination with SA WG6 may be needed on aspects to be identified during the study.

NOTE 6:	The scope of this KI will be aligned with RAN for 6G in Q1 2026. A checkpoint is set for Q1 2026 to revisit WT/KI descriptions and determine if solutions can be discussed in Q2 2026.

## 5.21	Key Issue #21: 6G data framework in SA2

NOTE 1:	Coordination (as requested by SA plenary LS SP-251261) with SA WG5 is required. Common functionalities for data handling may be identified between SA WG2 and SA WG5 to be coordinated with each other to avoid incompatible or duplicated solutions for data framework.

The Key issue is based on WT#5, which includes the following aspects:

1.	Identify high level use cases (e.g. AIML in the core, UE data collection for UE-sided model training, Sensing) potentially subject to the data framework.

NOTE 2:	This identification does not predetermine whether the use cases will ultimately adopt the data framework solutions. The coordination with other WTs for specific use cases is needed.

NOTE 3:	The use of the term “data framework” does not imply that all use cases that will be identified need to be supported by the same set of functionalities, procedures, NFs, services and/or use the same data structure/format.

2.	Study required data framework functionalities to support the identified use cases, e.g.:

a)	data discovery and data registration.

b)	data collection and transfer (including configuration of the data source (e.g. about collected data, reporting period and mechanisms to use), transfer of the collected data (including data distribution)).

c)	data labelling/metadata handling.

d)	data processing (e.g. data anonymization, data analysis, data generation, etc.).

e)	data storage and retrieval.

f)	data exposure.

The functionalities above may need to consider the quality of data, latency and data volume.

NOTE 4:	The bullet 1 and bullet 2 can be studied in parallel, e.g. each solution is expected to include or refer to the corresponding use case(s).

NOTE 5:	Coordination with SA WG5 may be required for charging aspects.

NOTE 6:	Consideration on MNO's requirements of visibility and controllability during data handling processes is required.

NOTE 7:	The outcome of 5GS studies in R20 and normative work in previous releases on data handling will be considered.

3.	Study potential system impacts of user consent, privacy, security and data governance from SA WG2 architectural and system level perspective for data generated in 6G system corresponding to the use cases and functionalities in aspects 1 and 2.

NOTE 8:	Coordination with SA WG3 is required for user consent, privacy, security and data governance related topics to avoid duplicated or incompatible design. SA WG2 should study first which use cases and functionalities in aspects 1 and 2 above apply to aspect 3 and then coordinate with SA WG3. SA WG2 should consider the output of SA WG3 before specifying any system procedure on this topic.

## 5.22	Key Issue #22: 6G Computing Support

Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination among UE, core network and application server, exposure of computing service in the core network, etc.).

In order to support computing, the following aspects need to be studied:

1.	Identification of the architectural requirements and computing resource(s):

a)	Derive architectural requirements for scenario(s) and service requirements defined by SA WG1 to be enabled by the Computing Service.

b)	Whether and how to define the computing resource (e.g. computing resource type and/or status, location of the computing resource, etc.).

NOTE 1:	The use cases and the terms of Computing Service and the Service Hosting Environment defined in TR 22.870 [11] can be used as starting point for further refinement during solution development phase. The detailed computing resource type (e.g. software component, hardware component, etc.) can be identified during study phase.

2.	Enablement/authorization of computing service to UE or AF.

a)	Whether and how to enable Computing Service.

b)	Whether and how the operator network may expose the computing resource related information (e.g. its hosting capability (e.g. using compute resources at Service Hosting Environment)) and/or network metrics to AF.

3.	Coordination of communication and computing, service continuity and QoS aspects:

a)	Whether and how to coordinate (e.g. within or outside the core network) communication (for the traffic transferred over user plane) and computing resource(s) in the Computing Site.

b)	Whether and how to identify, collect/monitor, provision the computing resource related information.

c)	Whether and how to improve the service experience.

d)	Whether there is a need to improve QoS to satisfy Computing Service requirements.

e)	Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g. due to UE mobility, computing load balancing, AF influence etc.).

4.	Discovery and (re-)selection of compute site(s) for the Computing Service:

a)	Whether and how to discover and (re-)select Computing Site(s) for the Computing Service.

NOTE 2:	Whether and how to utilise and/or enhance Edge Computing mechanisms specified in 5G to address the aspects in the scope of this KI will be discussed in the study phase.

NOTE 3:	Which of above aspects should fall into SA WG5 and/or SA WG6 scope will be further identified in study phase, coordination with SA WG5 and SA WG6 may be needed.

NOTE 4:	Potential coordination with other KIs (e.g. KI#1 on service enablement and UE-CN interaction, key issues on QoS, exposure framework and UP aspects, etc.) is needed.

NOTE 5:	Computing Site represents the computing resources in a specific location of network topology. The computing site can offer computing services by allocating its computing resources to run application workloads upon demand of a Computing Service consumer. From networking perspective, the Computing Site is located within 6G CN (via e.g. user plane function in 6G) or Data Network (either owned and controlled by the operator or 3rd party).

## 5.23	Key Issue #23: Support of 6G NTN

Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 1:	The detailed scope for this KI will be coordinated and aligned with RAN in Q1 2026. A checkpoint is set to Q1 2026 to revisit WT/KI descriptions if needed before discussing solutions.

NOTE 2:	This KI has IWK aspects that are scoped under WT#2 (KI#17).

NOTE 3:	This KI has dependency on mobility aspects that are scoped under WT#1.1 (KI#1).

NOTE 4:	This KI has voice aspects that are scoped under KIs corresponding to WT#1.4.

## 5.24	Key Issue #24: Analyse 5GS IoT features and solutions

This key issue will study how to support cellular IoT enablers in 6G including:

1	Analyse whether and which 5GS cellular IoT features are applicable to 6G and identify which of these features that should be further studied.

NOTE 1:	Enhancements in 6G to the selected 5GS IoT features are not precluded.

NOTE 2:	Features driven by this key issue are expected to be generic and may apply to any UEs in 6G.

NOTE 3:	The detailed scope for this key issue will be coordinated and aligned with Massive Communication (IoT) requirements from RAN and SA WG1. Ambient IoT, NB-IoT and non-3GPP access are not in the scope of the key issue.

## 5.X	Key Issue #X: Key Issue Title

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | #P | #Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #Y |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 6.X	Solutions to KI#X

### 6.X.Y	Solution #X.Y: <Solution Title>

#### 6.X.Y.0	High-level solution Principles

Editor’s note: 	A description of which KI(s), and which parts of each KI, the solution addresses should be included, in addition to the high-level solution principles.

Editor's note:	Where possible similar/overlapping solution proposals should be documented as a single solution in the TR.

Documentation of more than one solution from the same company for the same KI is allowed but it shouldn't be expected that all of them will be documented in the TR.

#### 6.X.Y.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Further (sub-)clause(s) may be added to capture details.

#### 6.X.Y.2	Procedures

Editor's note:	This clause will describe the high-level procedures and information flows for the solution.

#### 6.X.Y.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#Y

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in Annex B gets resolved and a principle is agreed.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

# 8	Consolidated 6G architecture interim agreements

Editor's note:	This clause can be used to consolidate system architecture aspects based on the consensus building around solutions proposed in clause 6. Whether to use this clause and how it relates to clause 9 will be determined as the study progresses.

# 9	Conclusions

Editor's note:	This clause will capture conclusions for the study.

By consensus interim agreements can become part of the final conclusions of the study.

Annex A:
Work Tasks

# A.0	Introduction

Editor's note:	This Annex will capture updates to WT scope based on the 6G SID in SP-250806. The plan is to update the Study Item Description with updated Work Task descriptions from this Annex.

This Annex contains clarifications of Work Task scope, as preparation for Key Issue drafting.

# A.0a	Mapping between Work Tasks and Key Issues

Table A.0a-1: Mapping of Work Tasks to Key Issues

| Work | Key Issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tasks | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| WT#1.1 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 SBA |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 network slicing |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 user plane |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 QoS |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 policy framework |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 network exposure |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 network sharing |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 localized service access |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 FWA |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.3 |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.4 |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |
| WT#2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| WT#3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| WT#3.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| WT#4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| WT#5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| WT#6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| WT#7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| WT#8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |

# A.X	Work Task X

Editor's note:	Description of Work Task X. Clauses can be used if needed.

# A.1	Work Task 1

## A.1.1	Work Task 1.1: Study the support for control signalling for 6G System

1.	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE 1:	It is assumed that WT#1.1 bullet 1a covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2:	For the above bullet 1a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

2. 	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 3:	This WT#1.1 bullet 2a can include any transport mechanism such as using NAS or UP or new plane. WT1.1 bullet 2a can have dependency on other work tasks e.g. WT#1.1 bullet 1a, WT#3, WT#4, WT#5, WT#6.

NOTE 4:	This WT#1.1 covers operator services.

## A.1.2	Work Task 1.2

### A.1.2.1	SBA framework

The following is work scope for SBA related work task.

1	Study whether and how to optimize NF/NF service registration, discovery and selection for efficient message forwarding compared with 5G.

2	Study whether and how to improve NF/NF service resiliency, scalability, efficiency and load balancing, compared with 5G.

NOTE:	This WT covers generic aspects for SBA framework. The study of other WTs can also result in potential enhancements on the SBA framework and will be studied as part of respective WTs.

### A.1.2.2	Network Slicing

The scope of the network slicing work task includes:

1.	Study the overall design and functionalities of the network slicing in 6G assuming the network slicing in 5GS as starting point for discussions including the following:

a.	Identify and address any areas of possible improvement and simplification of network slicing.

b.	Identify and address any improvements of the application traffic mapping to network slice(s)/user plane connection(s).

NOTE 1:	Coordination between bullet 1b and the WT 1.2 on Policy is expected.

2.	Study impacts of the network slicing in 6G on the interworking and migration.

NOTE 2:	Impacts to RAN are possible and coordination with RAN WGs is expected.

### A.1.2.3	User Plane Architecture

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied taking the 5GS user plane framework as a starting point for discussion:

1.	Whether and how to enhance CP-UP functional split and interaction for better multi-vendor interoperability.

2.	Whether and how to enhance user plane flexibility (by user plane function (re)selection) for different service requirements, session continuity and mobility with consideration of user plane function capability and path performance between access network and data network.

NOTE 1:	This work task may require coordination with RAN WG for user plane interface.

3.	Whether and how to enhance resilience, scalability, and high availability of user plane function.

NOTE 2:	The outcome of this study may include architectural requirements based on which stage 3 working group can work on the user plane protocols. The stage 3 protocol aspects, including how to design and select user plane protocols, are not in the scope of stage 2.

### A.1.2.4	QoS Framework for 6G

1.	Investigate whether and what new functionality in 6G QoS framework is required considering, the emerging new traffic characteristics and application needs, e.g. AI/ML application traffic.

NOTE 1:	Which working groups will determine the characteristics of AI traffic is subject to SA plenary (SA#110) decision.

NOTE 2:	Coordination with WT#3.2 may be needed.

2.	Whether and how to enhance QoS mechanisms to:

a.	Support QoS targets fulfilling application’s QoS requirements (e.g. dynamic QoS requirements) in a less resource intensive manner than the existing GBR, Delay Critical-GBR.

3.	Study whether and what enhancements are needed to adjust the QoS targets, in case the current QoS targets cannot be met, to minimize the impacts to application operation or user experience.

4.	Study whether and how to support the UE, application and network QoS collaboration to improve awareness in the 6GS of application traffic needs, e.g. traffic pattern, dynamic QoS requirements and to improve awareness in the application/UE of what can be provided by the network, e.g. maximum bitrate.

NOTE 3:	Applications consist of UE aspect and DN aspects.

5.	Whether and how to enhance QoS monitoring, based on the identified needs, e.g. to determine the end-to-end per-packet delay, packet loss rate between the PSA-UPF and the UE for the verification of the QoS target fulfilment.

6.	Study whether and what enhancements are needed to enforce QoS differentiation, e.g. in case of frequent address change of the server, application traffic with multiplexed media flows.

NOTE 4:	QoS aspects of interworking will be considered based on the WT#2.

NOTE 5:	This WT will coordinate with SA WG4 and RAN WGs.

NOTE 6:	The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion.

NOTE 7:	This WT may study impacts to the QoS for 6G based on the input identified and provided by other WTs.

### A.1.2.5	Policy Framework

To support the Policy Framework in 6GS, the policy and charging control Framework defined in 5G for policies (i.e. SM Policy, UE Policy, AM Policy) is considered as starting point for discussion, the following aspects are to be studied:

1.	Whether and how to simplify the policy control framework for 6G, including:

a.	whether and how to reduce the number of policy control services e.g. the same policy control association for both AM and UE policies or the number of interactions to efficiently provide policies to the different policy enforcement points.

NOTE 1:	The example above does not preclude that other examples are covered in the study phase if the reason for simplification is identified.

b.	whether and how to support the UE to request UE policies from the 6G CN and/or the 6G CN to provide UE policies to the UE.

2. 	Whether and how to consider user preferences during UE policy evaluation at UE, without conflicting with network provided UE policies.

NOTE 2:	the statement that refers to considering user preferences refers to the ability to influence the evaluation of UE policies.

3.	Whether and how to improve the External Parameter Provisioning aspects of policy control for 6G.

NOTE 3:	General Authorization of the external parameter provisioning is handled in the WT#1.2 Network Exposure.

4.	Whether and how to improve events notification about policies to the AF (e.g. to notify the AF that e.g. charging key changed).

5.	Whether and what parameters the UE can provide as input for PCC decision, e.g. related to QoS control to the 6G CN. The UE input for PCC decision related to QoS is based on the progress in WT#1.2 QoS.

NOTE 4:	The example above does not preclude that other examples are covered in the study phase if those are identified.

NOTE 5:	The AF input for session management defined in 5G can be treated as starting point for discussion.

NOTE 6:	The UE can decide if and when to provide the above input the request to the network.

NOTE 7:	The UE is not assumed to know whether AF input for QoS is available.

NOTE 8:	Whether the UE can provide any input for PCC decisions as the AF provides for UE or AM policies are out of the scope of this bullet.

NOTE 9:	The assumption is that either the AF or the UE provides input for PCC decision for e.g. QoS control.

The following notes are applicable to all bullets in this WT:

NOTE 10:	If the energy related information, either based on 5G mechanisms, or 6G mechanisms, is available then this WT will study how to take energy efficiency and consumption aspects into account for policy decisions.

NOTE 11:	This WT has IWK aspects that are scoped under WT#2.

NOTE 12:	This WT can study potential enhancements to the policy control framework for 6G services based on the input identified and provided by other WTs.

NOTE 13:	This WT depends on the overall 6G roaming architecture based on the output of WT#1.1. Solutions for this WT will address roaming if applicable.

NOTE 14:	Any improvements of the application traffic mapping to network slice(s)/user plane connection(s) are out of the scope of this WT.

NOTE 15:	Identify and address any feedback from markets where policy control framework technologies are already being commercially deployed.

### A.1.2.6	Network Exposure

This work task is to study whether and how to support a common network exposure framework in 6G. The capabilities that are exposed via the framework fall into one of two categories.

1.	The first category is new capabilities. New capabilities refer to capabilities that are not exposed in the 5G network. This category of exposed capabilities is dependent on other work tasks. This work task will include a review of exposure requirements that derive from other work tasks.

2.	The second category refers to capabilities that are exposed in the 5G network and are used as a starting point for discussion of this work task. This category of exposed capabilities may be dependent on other work tasks.

Since this work task depends on other work tasks, time will not be allocated for solutions for this work task by SA WG2 before SA2#175.

NOTE 1:	This work task requires coordination with SA WG3, SA WG6, and SA WG5.

NOTE 2:	This work task has dependency on service requirements to be specified by SA WG1.

NOTE 3:	Information exposure is based on operator policies/SLAs.

NOTE 4:	Authorization, Privacy protection and user consent aspects require coordination with SA WG3, and application layer user consent aspects may require coordination SA WG6.

Aspects that can be studied under this work task are:

1.	How to support a common network exposure framework in 6G for the exposure of network capabilities to an AF.

a.	Whether and how to support a common network exposure framework in 6G for the exposure of network capabilities to AFs, in an access independent manner (to the extent that exposure in an access independent manner is possible). The exposure framework of the 5GS (i.e. exposure via the NEF) is assumed to be the starting point for discussion of this work task.

b.	Whether and how to support co-existence with the 5G exposure framework.

NOTE 5:	The common network exposure framework in 6G in SA2 does not duplicate the SA WG6 Framework.

NOTE 6:	Examples of network capabilities include information exposure such as information about UE related events, network events, analytics, and information provisioning such as UE related network configuration parameters and service specific parameters.

c.	Whether and how to support privacy protection, user consent, data anonymization, and service authorization mechanisms.

d.	Whether and how to restrict Network Exposure procedures in roaming, aiming to avoid information being sent between the home network and visited network when not allowed or when not necessary.

2.	How to support Capability Exposure to the UE and Application Endpoints.

Depending on the outcome of WT#1.2 (QoS), this work task is about whether and how to integrate WT#1.2 (QoS) solutions for collaboration between the UE, applications and network in a common exposure framework. As a prerequisite for this work task, WT#1.2 (QoS) is expected to determine if specific information needs to be exchanged. This work task will cover:

a.	Based on the work in WT#1.2 (QoS), whether and how to integrate solutions from WT#1.2 into a common exposure framework in 6G in order to improve awareness in applications and the UE of what information can be provided by the network.

NOTE 7:	Exposure should be done in an operator-controlled manner.

NOTE 8:	Solutions to this work task can be based on the control plane and/or user plane.

b.	Whether there is a need to and how to have the common framework support how to discover what information can be provided by the network and authorize the network to provide specific information to the applications and the UE.

3.	Whether and how to support Intent-Based Exposure.

a.	Whether, how, and what intents should be supported on the northbound interface (i.e. the interface to an AF in a DN) of a common network exposure framework in 6G. The intent-based work in this work task is limited to the northbound interface of a common network exposure framework in 6G and needs to be coordinated with WT#3. AI aspects will be covered under WT#3 and will not be covered under this work task.

### A.1.2.7	Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

-	How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

### A.1.2.8	Localized service access

The scope of localized service access is to study the overall architecture design and enablers for localized service access in 6G, which consists of the following:

1.	Whether and how to support localized service access provided via PLMN, and study how to perform authorization and authentication of UE for localized service access, and minimize service interruption during UE mobility, including

a.	Whether, how and which aspects (e.g. provisioning, updating, retrieval) for the part of subscription (that is needed for localized service access) can be performed/managed via the NFs present locally in the network providing localized service access.

NOTE 1:	Network selection and access control are not to be studied for bullet 1 and bullet 2.

NOTE 2:	For the case when localized service access is provided via PLMN, solutions are expected not to introduce additional UE functionality that is specific for localized service access (i.e. anything that requires UE dependency beyond what is needed for general PLMN access).

NOTE 3:	Authentication aspects need to be coordinated with SA WG3.

2.	Study whether and how to make use of 5G NPN (SNPN and PNI-NPN) for localized service access in 6G.

NOTE 4:	Which of the existing features will be supported will be determined during the study.

NOTE 5:	Access control principles defined for NPN (SNPN and PNI-NPN) in 5G are assumed to be reused. Network selection principles defined for SNPN are assumed to be re-used, but remit stays within CT WG1.

NOTE 6:	Session and service continuity between PLMN and SNPN using direct interface between PLMN and SNPN is out of scope of this work task.

### A.1.2.9	FWA

Study support for Fixed Wireless Access in 6G:

1)	Analyse issues encountered in 5G deployments to efficiently support FWA and determine requirements to be taken by other WT for 6G. The result of this analysis will serve as the basis for architectural requirements to relevant Key Issues related with WT 1.1 and WT 1.2.

NOTE:	As the result of this analysis will serve as the basis for architectural requirements to Key Issues related with other Work Tasks, this analysis needs to conclude for June 2026.

## A.1.3	Work Task 1.3: Non-3GPP access

Study whether and how to support and/or enhance non-3GPP access in 6G as well as session continuity between 3GPP access and non-3GPP access.

NOTE 1:	The UE can access the 6G CN via non-3GPP access even if there is no 3GPP access available.

NOTE 2:	Coordination with SA3 for authentication aspects will be needed for this WT.

NOTE 3:	The dependency/interaction of non-3GPP access and 3GPP access should be minimized.

WT#1.3.1. For non-3GPP access:

1.	Study how to support untrusted non-3GPP access in 6G System architecture.

2.	Study how to support service continuity between 3GPP access and non-3GPP access in the above bullet(s).

3.	Study how to support Interworking aspects between 5GS and 6GS related to non-3GPP access.

NOTE 4:	Coordination with other WTs is needed.

NOTE 5:	Solutions to above bullets may support connectivity via non-3GPP access only.

NOTE 6:	Solution discussion on Simultaneous connectivity via 3GPP and non-3GPP access with only per flow granularity can only start from SA2#175, but with low priority at SA2#175 if time allows.

WT#1.3.2. For Non-Seamless WLAN Offload:

1.	How to support UE policy necessary for Non-Seamless WLAN Offload (NSWO).

## A.1.4	Work Task 1.4: 6G Essential Services

Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

# A.2	Work Task 2: Migration and Interworking

Study migration and interworking, including

1.	How to support migration to 6GS.

2.	How to support interworking with 5GS.

3.	Whether and how to support interworking with EPS.

NOTE 1:	Whether to support interworking with EPS will depend on SA1 requirement.

4.	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is required to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:	The detailed migration study scope and migration options will be coordinated and aligned with RAN.

NOTE 4:	Additional migration options beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR, will be studied no earlier than June 2026 in alignment with TSG RAN (if needed).

NOTE 5:	It is assumed that interworking for roaming is within the scope of this work task.

NOTE 6:	This work task focuses on general interworking procedures. The specific interworking aspects studied in other WTs (if any) need to follow the general interworking procedures.

# A.3	Work Task 3: AI

## A.3.0	General

Study how to support and enable use of AI in 6G (e.g. AI agent, AI framework).

NOTE: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA WG1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

## A.3.1	WT#3.1:	AI for 6G architecture

The AI for the 6G architecture shall be multi-vendor interoperable, reliable and sustainable.

NOTE 1:	It is assumed that operators can decide whether and which AI capabilities and technologies to deploy in their network.

Study whether and how to provide an architecture for AI to fulfil the following:

1.	Enable the 6G CN to leverage AI capabilities and technologies in the 6G CN (e.g. AI agent), subject to operator policies and configuration and using 6G CN functionalities available in the network:

1.1	Determine how to fulfil requests from the UEs or AFs when intent is included and, in order to ensure interoperability of intents and supporting system test cases, determine the constraints on the use and expression of intents sent from the UEs and the AFs so that the intents can be processed and interpreted unambiguously by the AI capable entities in the 6G CN, and define the mechanisms required to support these constraints.

1.2	Determine how to fulfil requests when intent is not included.

1.3	In order to enable AI capable entities in 6G CN to dynamically compose parts of procedures to fulfil requests from UEs and AFs, determine the design principles and constraints for the modularisation of the procedures for the 6G CN that will be applicable to the system procedures defined in the normative work. How to enable the AI capable entities to compose these procedures in SA2 specifications will be determined by the study.

NOTE 2:	An intent refers to expectations including requirements, goals, conditions, guidelines, and constraints, without specifying how to achieve them. What intent means and how to specify intents in SA WG2 specifications will be determined by the study. Intent is to be specified as a result of this work task.

NOTE 3:	Whether to support intents in a 6G CN is up to operator choice. It is assumed it is not required for the MT stack of UE to produce nor understand the intent. The MT stack of UE is assumed to be agnostic to whether or not the network uses 6G CN AI capable entities (e.g. AI agent, AI-enabled NFs) to address UE requests not including intent.

NOTE 4:	This should not result in duplicated procedures in 3GPP specifications. It is assumed that the 6G study will still describe 6G system procedures for the 3GPP features, without necessarily requiring AI capabilities.

2.	Enable closed-loop operations and learning techniques such as reinforcement learning, in 6G CN.

3.	Enable entities in 6G CN to access network AI capabilities provided by 6G NFs.

4.	Enable AI capable entities in 6G CN to access trusted external capabilities provided by AF.

NOTE 5:	Whether and what external capabilities provided by AF is accessed by AI capable entities will be discussed as part of the study.

5.	Enable the monitoring of the performance of all AI capable entities in 6G CN.

6.	Enable the operator to control the network's use of AI capabilities in its 6G CN, i.e. support different operator-configurable levels of autonomy based on operational needs, including the option to use or not use any AI capabilities.

7.	Support roaming scenarios, e.g. how visited networks using or not using AI technologies can interwork with home network using or not using AI technologies.

8.	Enable NFs of the 6G CN to have AI/ML capabilities, ML model provisioning, inferencing, training and monitoring.

9.	Enable the 6G CN AI architecture to ensure interoperability with 5G CN AI architecture if needed, for the purpose of maintaining a consistent service for UEs across 5G and 6G.

NOTE 6:	How to process the intent when AI capabilities are not being used is to be studied as part of this work task.

NOTE 7:	Aspects related to SA3, SA5, SA6 will be coordinated with each group respectively during the study.

NOTE 8:	Further alignments with SA1 consolidated requirements may be needed.

NOTE 9:	This sub work task does not cover AI agents in UE, which is covered by WT3.2.

NOTE 10:	Coordination across the WTs in the 6G study is needed.

NOTE 11:	This work task will not duplicate the work in the scope of WT#1.1.

## A.3.2	WT#3.2 6G Network for AI:

1.	Study whether and how to support an AI agent on a UE to discover another AI agent on a different UE via the 6G network(s).

2.	Study whether and how to enable communication for AI agents on different UEs via the 6G network(s) e.g. identification and authorization of an AI agent on a UE.

3.	Study whether and how to enhance network capability exposure functionalities to AI agent on AF(s).

NOTE 1:	Coordination with WT#1.2 for Network Exposure is required.

NOTE 2:	During the study, which aspects will belong to SA WG2 or SA WG6, or required coordination with SA WG6 on the application layer aspects will be discussed, if needed. And coordination with SA WG3 on security aspects may be needed.

NOTE 3:	When AI agent is on the UE, this AI agent is assumed not part of the MT. It is also assumed that this WT is not replacing UE to Core Network interaction, NAS.

NOTE 4:	The AI agent in WT#3.2 is outside of RAN and Core Network for 6G.

4.	Study whether and how the 6G CN can provide AI services (i.e. AI inferencing and AI training) to applications (AF or in UE).

5.	Study the potential system impacts based on the characteristics of AI traffic.

NOTE 5:	Coordination with WT1.2 QoS may be needed.

NOTE 6:	System enhancement related to QoS for AI traffic will be handled by WT1.2 QoS,

NOTE 7:	Which working groups will determine the characteristics of AI traffic is subject to TSG SA plenary (SA#110) decision.

NOTE 8:	Coordination with SA WG3, SA WG5 and SA WG6 may be needed on AI inferencing and AI training.

NOTE 9:	Coordination with WT#5 and WT#6 may be needed.

NOTE 10:	New service charging part may be coordinated with SA5 Charging group.

# A.4	Work Task 4: Integrated Sensing and Communication

Study the integration of Sensing and Communication over 3GPP access, covering RAN node and UE operating as 3GPP Sensing Entities in the different sensing modes in alignment with TSG RAN, and considering multiple sources of sensing data.

The following aspects will be studied:

1.	Architecture and functional support for Sensing Service, including:

a.	Sensing Service authorization and revocation.

b.	Discovery and (re-)selection of Sensing function and Sensing Entities (i.e. RAN nodes, UEs), including sensing modes selection.

c.	Configuration parameters and policy provisioning to support sensing service, e.g. to the selected Sensing Entities, Sensing function, etc.

d.	Collection and transport of 3GPP sensing related data for sensing result generation.

e.	Exposure framework of sensing result to the Sensing service consumer (i.e. AFs, UEs, Core NFs).

NOTE 1:	How the Sensing service consumer makes use of the sensing results is not part of this work task.

2.	Whether and how to support Sensing Service in consideration of the mobility of the UE as sensing entity.

3.	Whether and how to support providing sensing data, which is not obtained from 3GPP radio signals, to the Sensing Function for sensing result generation.

NOTE 2:	This includes sensing data obtained using technologies that are different from 3GPP defined ISAC mechanisms, such as sensing based on IEEE 802.11, camera, sonar, lidar, etc.

NOTE 3:	The scope of WT#4 assumes RAN node and UE operating as Sensing Entities in different sensing modes in alignment with TSG RAN.

NOTE 4:	Coordination with other WTs will be needed if identified, e.g. Sensing data aspects with WT#5, architectural aspect with WT#1, etc.

NOTE 5:	Coordination with RAN WGs for RAN aspects, SA WG3 for privacy and user consent aspects, and SA WG5 for OAM and charging aspects will be needed. Coordination with SA6 may be needed on aspects to be identified during the study.

NOTE 6:	The scope of WT#4 will be aligned with RAN for 6G in Q1 2026. A checkpoint is set for Q1 2026 to revisit WT/KI descriptions and determine if solutions can be discussed in Q2 2026.

# A.5	Work Task 5: 6G data framework in SA WG2

NOTE 1:	Coordination (as requested by SA plenary LS SP-251261) with SA WG5 is required. Common functionalities for data handling may be identified between SA WG2 and SA WG5 to be coordinated with each other to avoid incompatible or duplicated solutions for data framework.

The WT includes the following aspects:

1.	Identify high level use cases (e.g. AIML in the core, UE data collection for UE-sided model training, Sensing) potentially subject to the data framework.

NOTE 2:	This identification does not predetermine whether the use cases will ultimately adopt the data framework solutions. The coordination with other WTs for specific use cases is needed.

NOTE 3:	The use of the term “data framework” does not imply that all use cases that will be identified need to be supported by the same set of functionalities, procedures, NFs, services and/or use the same data structure/format.

2.	Study required data framework functionalities to support the identified use cases, e.g.:

a.	Data discovery and data registration.

b.	Data collection and transfer (including configuration of the data source (e.g. about collected data, reporting period and mechanisms to use), transfer of the collected data (including data distribution)).

c.	Data labelling/metadata handling.

d.	Data processing (e.g. data anonymization, data analysis, data generation, etc.).

e.	Data storage and retrieval.

f.	Data exposure.

The functionalities above may need to consider the quality of data, latency and data volume.

NOTE 4:	The bullet 1 and bullet 2 can be studied in parallel, e.g. each solution is expected to include or refer to the corresponding use case(s).

NOTE 5:	Coordination with SA WG5 may be required for charging aspects.

NOTE 6:	Consideration on MNO’s requirements of visibility and controllability during data handling processes is required.

NOTE 7:	The outcome of 5GS studies in R20 and normative work in previous releases on data handling will be considered.

3.	Study potential system impacts of user consent, privacy, security and data governance from SA WG2 architectural and system level perspective for data generated in 6G system corresponding to the use cases and functionalities in aspects 1 and 2.

NOTE 8:	Coordination with SA WG3 is required for user consent, privacy, security and data governance related topics to avoid duplicated or incompatible design. SA WG2 should study first which use cases and functionalities in aspects 1 and 2 above apply to aspect 3 and then coordinate with SA WG3. SA WG2 should consider the output of SA WG3 before specifying any system procedure on this topic.

# A.6	Work Task 6: 6G Computing Support

Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination among UE, core network and application server, exposure of computing service in the core network, etc.).

In order to support computing, the following aspects need to be studied:

WT#6.1: Identification of the architectural requirements and computing resource(s):

a.	Derive architectural requirements for scenario(s) and service requirements defined by SA WG1 to be enabled by the Computing Service.

b.	Whether and how to define the computing resource (e.g. computing resource type and/or status, location of the computing resource, etc.).

NOTE 1:	The use cases and the terms of Computing Service and the Service Hosting Environment defined in TR 22.870 [11] can be used as starting point for further refinement during solution development phase. The detailed computing resource type (e.g. software component, hardware component, etc.) can be identified during study phase.

WT#6.2: Enablement/authorization of computing service to UE or AF.

a.	Whether and how to enable Computing Service.

b.	Whether and how the operator network may expose the computing resource related information (e.g. its hosting capability (e.g. using compute resources at Service Hosting Environment)) and/or network metrics to AF.

WT#6.3: Coordination of communication and computing, service continuity and QoS aspects:

a.	Whether and how to coordinate (e.g. within or outside the core network) communication (for the traffic transferred over user plane) and computing resource(s) in the Computing Site.

b.	Whether and how to identify, collect/monitor, provision the computing resource related information.

c.	Whether and how to improve the service experience.

d.	Whether there is a need to improve QoS to satisfy Computing Service requirements.

e.	Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g. due to UE mobility, computing load balancing, AF influence etc.).

WT#6.4: Discovery and (re-)selection of compute site(s) for the Computing Service:

a.	Whether and how to discover and (re-)select Computing Site(s) for the Computing Service.

NOTE 2:	Whether and how to utilise and/or enhance Edge Computing mechanisms specified in 5G to address the aspects in the scope of WT#6 will be discussed in the study phase.

NOTE 3:	Which of above aspects should fall into SA WG5 and/or SA WG6 scope will be further identified in study phase, coordination with SA WG5 and SA WG6 may be needed.

NOTE 4:	Potential coordination with other WTs (e.g. WT#1.1 on service enablement and UE-CN interaction, WT#1.2 on QoS, exposure framework and UP aspects, etc.) is needed.

NOTE 5:	Computing Site represents the computing resources in a specific location of network topology. The computing site can offer computing services by allocating its computing resources to run application workloads upon demand of a Computing Service consumer. From networking perspective, the Computing Site is located within 6G CN (via e.g. user plane function in 6G) or Data Network (either owned and controlled by the operator or 3rd party).

# A.7	Work Task 7: Support of 6G NTN

Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 1:	The detailed scope for WT#7 will be coordinated and aligned with RAN in Q1 2026. A checkpoint is set to Q1 2026 to revisit WT/KI descriptions if needed before discussing solutions.

NOTE 2:	This WT has IWK aspects that are scoped under WT#2.

NOTE 3:	This WT has dependency on mobility aspects that are scoped under WT#1.1.

NOTE 4:	This WT has voice aspects that are scoped under WT#1.4.

# A.8	Work Task 8: Cellular IoT enablers in 6G

Study how to support cellular IoT enablers in 6G including:

1	Analyse whether and which 5GS cellular IoT features are applicable to 6G and identify which of these features that should be further studied.

NOTE 1:	Enhancements in 6G to the selected 5GS IoT features are not precluded.

NOTE 2:	Features driven by this WT are expected to be generic and may apply to any UEs in 6G.

NOTE 3:	The detailed scope for WT#8 will be coordinated and aligned with Massive Communication (IoT) requirements from RAN and SA WG1. Ambient IoT, NB-IoT and non-3GPP access are not in the scope of the WT.

Annex B:
Topics for further resolution

# B.X	Topics for further resolution for KI#X

Editor's note:	This clause will include the topics for further resolution as work progresses for the specific KI#X. Eventually this clause should only contain topics for further resolution that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.X) and can either be then marked as not pursued or postponed to a future Release.

Annex C:
Change History

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA2#170 | S2-2507937 | - | - | - | TR skeleton for FS_6G_ARC | 0.0.0 |
| 2025-08 | SA2#170 | - | - | - | - | Documented approved p-CRs at S2#170: S2-2508022, S2-2508034 | 0.1.0 |
| 2025-10 | SA2#171 | - | - | - | - | Documented approved p-CRs at S2#171: S2-2509788, S2-2509821, S2-2509835 | 0.2.0 |
| 2025-11 | SA2#172 | - | - | - | - | Documented approved p-CRs at S2#172: S2-2511123, S2-2511142, S2-2511144, S2-2511180, S2-2511196, S2-2511197, S2-2511201, S2-2511212, S2-2511213, S2-2511214, S2-2511215, S2-2511217, S2-2511218, S2-2511240, S2-2511241, S2-2511244, S2-2511245, S2-2511252, S2-2511253, S2-2511254, S2-2511261, S2-2511297 | 0.3.0 |