TSG SA WG3 #4, London, 16-18 June 1999													S3-99153

| Tdoc SMG10 AP99-070 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST No : |  |  |  |  |  | A005 | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| Technical Specification GSM / UMTS: |  |  |  | 02.33 |  | Version |  | 7.2.0 |  |  |  |
| Submitted to SMG | 29 |  | for approval |  | x | Without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  | X |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME |  | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | Lawful Interception |
| --- | --- |

| Source: | SMG10-WPD | Date: | 28 April 1999 |
| --- | --- | --- | --- |

| Subject: | Addition of intercept events for lawful interception for GPRS. Correction to target identities and intercept related data are also included. |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | The change is required to add interception related events. These events are required to provide regulatory agencies with needed information regarding target activity. Internet Protocol Address is removed as an target identity used for interception as this does not provide a reliable identity for targets. Access point name is added to, and reason for PDP deactivation reason is deleted from, the intercept related data. |
| --- | --- |

| Clauses affected: | Paragraphs 4.1, A.2.1, A2.2, A2.3 and A.3 are modified. |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: | 03.33-A001 |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

# 4	Description

## 4.1	General principle

GSM Public Lands Mobile Network (PLMN) shall provide access to the intercept product and the intercept related information of the mobile target on behalf of Law Enforcement Agencies (LEAs).

A mobile target in a given PLMN can be a subscriber of that PLMN, or a subscriber of another PLMN. The intercept product and the related information can only be delivered for activities on that given PLMN.

An unambiguous correlation shall be established between the intercept related information and the intercept product. The intercept related information and the intercept product shall be delivered in as near real time as possible.

Location Dependent Interception allows a PLMN to service multiple interception jurisdictions within its service area. Multiple law agencies with their own interception areas can be served by the PLMN. All the information or rules given for interception within a PLMN apply to interception within an IA when Location Dependent Interception is invoked. A target may be marked in one or more different IAs within the same PLMN. Interception is not required nor prohibited by this standard when Location Dependent Interception is active and the location of the target subscriber is not known or available.

Additional GPRS interception requirements are contained in annex A.

Annex A (Normative):
GPRS and Lawful Interception

## A.1	Introduction

In GPRS interception is applicable to Point-To-Point (PTP) services, Point-To-Multipoint Group call (PTM-G) and Short Message Service (SMS). Interception is not applicable to Point-To-Multipoint Multicast (PTM-M).

## A.2	Intercept Invocation

Lawful interception is invoked when a GPRS PDP mobile is attached.. If lawful interception is activated when a PDP context is already activated, the next packet transmitted shall be intercepted. If lawful interception is deactivated, the next packets transmitted shall not be intercepted.

### A.2.1	Intercept related information events

Intercept related information concerning call progress may be provided to the LEA when the target is using the GPRS service. The following intercept related information events may be provided to the LEA:

- GPRS attach;

- GPRS detach;

the PDP context activation (GPRS);

-	the start of interception with PDP context active;

- the PDP context deactivation (GPRS);

- Cell and/or RA update;

- SMS

For the case of an SMS transaction the intercept related information is provided in conjunction with the SMS intercepted product to the LEA.

### A.2.2	Intercept related information provided to the LEA

The specific information provided by the intercept related information events will vary depending upon: the specific event, and the availability of information in a particular scenario.

For GPRS, a list of the type of information that may be provided to the LEA by intercept related information events, includes:

-	an activation reference identity;

-	the target identity which has been intercepted (e.g. MSISDN, IMSI, IMEI if applicable);

-	type of protocol activated (e.g. Internet Protocol or X.25);

- 	PDP address used by the target;

-

-	location information of target (Cell Global Identity);

-	time of event;

-	Access Point Name.

### A.2.3	GPRS intercept product

GPRS is capable of carrying many different protocols at the application level. The intercept product shall be the packet as sent by the application without interpretation or modification except for possible reframing, segmentation, or enveloping required to transport the packet to the LEA. No PLMN encryption, compression or encoding shall be present in the intercepted product. The intercepted product should be delivered to the LEA using a standard or widely used protocol.

### A.3	Interworking considerations

For GPRS, the PLMN, visited or roaming, shall not be responsible to interpret the protocol used by the target, or to remove user level compression or encryption.