TSG SA WG3 #6, Sophia Antipolis, 29 Sep – 1 Oct, 1999-10-01							S3-99345

| 3G CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TS 33.102 |  |  |  |  |  | CR |  |  |  | 19 |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | V3.1.0 |  |  |  |
| 3G specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by 3G support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to TSG |  |  |  |  | SA#5 |  |  | for approval |  |  |  |  |  | x |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| list TSG meeting no. here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  | be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  | x | ME |  |  |  |  |  |  |  |  | UTRAN |  |  |  |  |  |  | Core Network |  |  |  |  | x |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Siemens AG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  | 01-10-99 |  |  |  |  |
| Subject: |  |  | A generalised scheme for sequence number management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  | Security |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A Corresponds to a correction in a 2G specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | Sequence number management as described in the current version of TS 33.102 has drawbacks which the generalised scheme avoids. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | Annex C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

# Annex C: Management of sequence numbers

This annex is devoted to the management of sequence numbers for the authentication and key agreement protocol.

# C.6	A generalised scheme for sequence number management

This section describes the use of generalised sequence numbers which have an individual and a global component.

- The sequence number consists of two concatenated parts SQN = SQN1 || SQN2. SQN1 represents the most significant bits of SQN, and SQN2 represents the least significant bits of SQN.

- There are counters SQNMS and SQNHE in the MS and the HE respectively. Both parts of SQN are stored by these counters. SQNHE is an individual counter, i.e. there is one per user.

- There is a global counter, e.g. a universal clock with an appropriate time granularity (e.g. seconds elapsed since the start of the system). For short we call the value of this global counter at any one time GLC. If GLC is taken from a universal clock it is computed mod 2n where n is the length of GLC and of SQN2 in bits.

- When the HE needs a new sequence number SQN to create a new authentication vector, HE retrieves the (user-specific) value of SQNHE = SQN1HE || SQN2HE from the database. If SQN2HE < GLC then HE sets SQN = SQN1HE || GLC. If SQN2HE  GLC then HE sets SQN = (SQN1HE +1) || GLC.
Then SQNHE  is reset to SQN.

- The sequence number SQN is accepted by the USIM if and only if SQN >SQNMS holds.

- If the mechanism described in Annex C.4 (lists of sequence numbers in the USIM) is used and if SQNLO denotes the lowest sequence number in the list then (5) becomes:

The sequence number SQN is now accepted by the USIM if and only if SQN >SQNLO holds and SQN is not in the list.

- If the mechanism described in Annex C.5 (protection against counter wrap-around) is employed then (5) becomes:

The sequence number SQN is now accepted by the USIM if and only if SQN>SQNMS and 
SQN - SQNMS<  hold.

- If both the mechanisms described in Annexes C.4 and C.5 are employed and if SQNHI denotes the highest sequence number in the list then (5) becomes:

The sequence number SQN is now accepted by the USIM if and only if SQN>SQNLO and 
SQN – SQNHI <  hold and SQN is not in the list.

When parameters are appropriately chosen then this use of sequence numbers is compatible with the re-synchronisation procedure described in section 6.3.5 and the protection against wrap around of counters described in Annex C.5, and it is not required to conceal this type of sequence numbers.