3GPP TSG-SA3 Meeting #100e 	S3-201552

- e-meeting, 17 -28 August 2020

- Source:	KPN

- Title:	New Solution for KI3: Encrypted transfer of LTK between UDR and UDM/ARPF

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

It is requested that SA3 approves this solution based on encryption for protecting the long term key during transfer between the UDR and the UDM/ARPF

# 2	References

[1]	3GPP TS 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.3.0

[2]	S3-201551, New solution for KI2: Encrypted storage of LTK in UDR, 3GPP SA3#100-e

# 3	Rationale

A new solution is proposed for the protection of the long term key during transfer from the UDR to the UDM/ARPF. The proposal is to use a an approved symmetric-key algorithm where the secret encryption/decryption key is shared between the UDM/ARPF and the source of the long term key (e.g. the party provisioning USIMs), based on the assumption that the long term key is already stored in encrypted form in the UDR (see new solution described in [2]). This would result a strong enough protection mechanism that does not require the storage of a decryption/encryption key in the UDR.

The solution allows the possibility of having multiple decryption/encryption keys by using a key identifier for identifying the appropriate key.

# 4	Detailed proposal

START OF CHANGE 1

## 7.x	Solution #<x>: Encrypted transfer of the LTK between UDR and UDM/ARPF

### 7.x.1	Introduction

The long term key is transported in encrypted form during its transfer from UDR to UDM/APRF, using a symmetric-key algorithm, where the decryption key is stored securely in the UDM/APRF. In order to enable the use of multiple decription keys, also a key identifier is transferred in association with the long term key. It is assumed that the long term key is provisioned and stored in encrypted form in the UDR (see Solution #x), so there is no need to store the encryption key in the UDR.

### 7.x.2	Solution details

The long term key shall be transferred in encrypted form between UDR and UDM/ARPF. This solution recommends the use of a NIST approved symmetric-key algorithm where the secret shared key (for encryption) is generated (and securely stored) at the source of the generation and encryption of the long term-key. The secret shared key (for decryption) shall also be present in secure storage inside the UDM/ARPF.

There may be multiple encryption/decryption keys. In this case, during transfer, a key identifier shall be associated to the encrypted long term key. The key identifier shall enable the retrieval of the associated decryption key in the UDM/ARPF.

Additionally, the protocol used to transfer the encrypted long term key between UDR and UDM/ARPF shall be based on secure encrypted transport mechanisms (such as SSH, HTTPS, or FTPS).

NOTE: In 5GC, Nudr is protected at transport level using HTTPS as any other SBA interface.

### 7.x.3	Evaluation

Editor's note: to be provided.

END OF CHANGE 1