3GPP TSG-SA3 Meeting #100e 	S3-202029-r3

- e-meeting, 17 -28 August 2020							Merger of S3-202029, S3-201751, S3-201752

- Source:	Ericsson, Huawei, HiSilicon

- Title:	pCR to living document for TS 33.220: SBA support for Zh and Zn interfaces

- Document for:	Approval

Agenda Item:	4.14

# 1	Decision/action requested

It is proposed to approve the description of SBA support for Zh and Zn interfaces.

# 2	References

[1]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2

[2]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

# 3	Rationale

This contribution proposes a detailed description of SBA support for the Zh and Zn interfaces. The contribution contains updates to the existing clauses a new clause x.2.3 and a new clause X.X.

The proposed text refers to the TS 23.228 [1] on IMS and the reference already exists in TS 33.220 [2] as reference [8]. However, the reference includes release information ("release 6") and the reference is not used anywhere in the existing TS 33.220 [2] text. Therefore, the reference [8] to the TS 23.228 [1] on IMS was corrected to remove the release information and the proposed text used reference [8].

A new reference to TS 23.502 [3] is added.

# 4	Detailed proposal

**** START OF CHANGES ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 31.102: "Characteristics of the USIM application".

[2]	3GPP TS 33.102: "3G Security; Security architecture".

[3]	IETF RFC 2617: "HTTP Authentication: Basic and Digest Access Authentication".

[4]	IETF RFC 3310: "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[5]	3GPP TS 33.221: "Generic Authentication Architecture (GAA); Support for Subscriber Certificates".

[6]	Void

[7]	Void

[8]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[9]	Void.

[10]	3GPP TS 31.103: "Characteristics of the IP Multimedia Services Identity Module (ISIM) application".

[11]	3GPP TS 23.003: "Numbering, addressing and identification".

[12]	IETF RFC 3548: "The Base16, Base32, and Base64 Data Encodings".

[13]	3GPP TS 33.210: "3G Security; Network domain security; IP network layer security".

[14]	Void.

[15]	3GPP TS 31.101: "UICC-terminal interface; Physical and logical characteristics".

[16]	3GPP TS 33.203: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G security; Access security for IP-based services".

[17]	Void.

[18]	IETF RFC 2818: "HTTP over TLS".

[19]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[20]	Void.

[21]	Void.

[22]	IETF RFC 2104: "HMAC: Keyed-Hashing for Message Authentication".

[23]	ISO/IEC 10118-3:2004: "Information Technology – Security techniques – Hash-functions – Part 3: Dedicated hash-functions".

[24]	IETF RFC 3629: "UTF-8, a transformation format of ISO 10646".

[25]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[26]	3GPP TS 33.246: "3G Security; Security of Multimedia Broadcast/Multicast Service (MBMS)".

[27]	Void.

[28]	IETF RFC 2246: "The TLS Protocol Version 1".

[29]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[30]	(void)

[31]	(void)

[32]	3GPP TS 29.109: "Generic Authentication Architecture (GAA); Zh and Zn Interfaces based on the Diameter protocol; Stage 3".

[33]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[34]	3GPP TS 23.002: “Network architecture “.

[35]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security Architecture".

[36]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[37]	"Unicode Standard Annex #15; Unicode Normalization Forms", Unicode 5.1.0, March 2008. http://www.unicode.org

[38]	3GPP TS 26.237: "IP Multimedia Subsystem (IMS) based Packet Switch Streaming (PSS) and Multimedia Broadcast/Multicast Service (MBMS) User Service; Protocols".

[39]	3GPP TS 33.224: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push Layer".

[40]	3GPP TS 33.328: "IMS Media plane security".

[41]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[42]	(void)

[43]	Void.

[44]	IETF RFC 5705: "Keying Material Exporters for Transport Layer Security (TLS)".

[45]	3GPP TS 33.223: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function".

[46]	3GPP TS 44.006 "Technical Specification Group GSM/EDGE Radio Access Network; Mobile Station - Base Station System (MS - BSS) interface; Data Link (DL) layer specification".

[47]	3GPP TS 43.020 "Technical Specification Group Services and system Aspects; Security related network functions".

[48]	IETF RFC 5929 "Channel Bindings for TLS".

[49]	3GPP TS 33.303: "Proximity-based Services; Security Aspects".

[50]	3GPP TS 33.179: "Security of Mission Critical Push-To-Talk (MCPTT)".

[51]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[52]	3GPP TS 33.163: " Battery Efficient Security for very low Throughput Machine Type Communication (MTC) devices (BEST)".

[53]	3GPP TS 33.501: " Security architecture and procedures for 5G system".

[54]	3GPP TS 33.180: "Technical Specification Group Services and System Aspects; Security of the mission critical service".

[55]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[XY]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

**** Next CHANGE ****

# Annex X (normative): Support of SBA in GBA

## X.1	General

Editor’s Note: More details to be specified.

### X.1.0	 Overview

This Annex X describes support for SBA for GBA.

### X.1.1 Architectural Support

Figure X.1.1-1 shows the non-roaming architecture to support SBA interactions in GBA. An SBI capable BSF, HSS and NAF shall implement the SBA interfaces specified in this Annex. An SBI capable NF can invoke SBA services provided by SBI capable NFs and may expose services itself. For this Annex an SBI capable BSF uses and provides SBA services, an SBI capable HSS provides SBA services while an SBI capable NAS only uses SBA services. The BSF, HSS and NAF reside in the home network.

Figure X.1.1-1: System Architecture to support SBA in GBA

Figure X.1.1-2 shows the architecture using the reference point representation. It should be observed that this annex addresses only the specification of the NG1 (between the BSF and HSS) and NG2 (between the NAF and BSF) reference point interfaces as SBA interfaces. The specification of Ua and Ub is not impacted by the introduction of the SBA interfaces between the NAF, BSF and HSS. Therefore, the UE interacts with the BSF and NAF as defined in the main body of this specification.

Figure X.1.1-2: System Architecture to support SBA in reference point representation

Editor’s Note: Roaming considerations are FFS.

### X.1.2 Reference point to support SBA in GBA

The following reference points are realized by service-based interfaces in GBA:

NG1: Reference point between an SBI capable BSF and an SBI capable HSS.

NG2: Reference point between an SBI capable BSF and an SBI capable NAF.

Editor’s Note: New reference point names are to be confirmed with SA2.

### X.1.3 Service based interface to support SBA in GBA

The following service-based interfaces are defined:

Nhss: Service-based interface exhibited by an SBI capable HSS.

Nbsp: Service-based interface exhibited by an SBI capable BSF.

Editor’s Note: New SBA interface name for BSF is to be confirmed with SA2.

These SBI services provide equivalent functionality to the Diameter Zh and Zn reference points.

To support co-existence of GBA nodes supporting SBA services and GBA nodes not supporting SBA services SBI capable GBA nodes may support both SBI and non-SBI interfaces.

## X.2 GAA/GBA SBA Services

Editor’s Note: More details to be specified.

Editor’s Note: For 5G, we only have the AUSF, UDM. Who will play the role of HSS is FFS.

Editor’s Note: Whether GBA AKA is still application in 5G-GBA is FFS.

### X.2.1	HSS Services

#### X.2.1.1 General

An SBI capable HSS supports providing the authentication vectors and the subscription profile, i.e. GUSS, to an SBI capable BSF via service-based interfaces.

The following table shows the GBA services exposed by an SBI capable HSS.

Table X.2.1.1-1: GBA Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nhss_GbaSubscriber Data | Get | Request/Response | BSF |
| Management (_GbaSDM) | Subscribe | Subscribe/Notify | BSF |
|  | Unsubscribe | Subscribe/Notify | BSF |
|  | Notification | Subscribe/Notify | BSF |
| Nhss_GbaUE Authentication | Get | Request/Response | BSF |

#### X.2.1.2	Nhss_GbaSubscriberDataManagement (GbaSDM) service

##### X.2.1.2.1	General

GBA Subscriber data types e.g. GUSS used in the Nhss_GbaSDM Service are defined in Table X.2.1.2.1-1 below.

Table X.2.1.2.1-1: GBA Subscriber data types

| GBA Subscriber data | Description |
| --- | --- |
| GUSS | This includes GBA User Security Settings. GUSS is consumed by BSF. |

At least a mandatory data key is required for each GBA Subscriber Data Type to identify the corresponding data as defined in Table X.2.1.2.1-2 below.

Table X.2.1.2.1-2: GBA Subscriber data types keys

| GBA Subscriber Data Types | Data Key | Data Sub Key |
| --- | --- | --- |
| GUSS | User Identity |  |
| NOTE: User Identity shall be one of IMSI, MSISDN, IMPI, IMPU. |  |  |

##### X.2.1.2.2	Nhss_GbaSDM_Get service operation

Service operation name: Nhss_GbaSDM_Get

Description: This service operation enables the NF consumer to fetch the GBA User Security for the end user.

The HSS shall check that the requested NF consumer is authorized to fetch the requested data.

Inputs, Required: NF Type, GBA Subscriber data type(s), Key for GBA Subscriber data type(s).

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: Requested Data.

##### X.2.1.2.3	Nhss_GbaSDM_Subscribe service operation

Service operation name: Nhss_GbaSDM_Subscribe

Description: The NF consumer subscribes for updates to requested data. HSS shall check that the requested NF consumer is authorized to subscribe to requested updates.

Inputs, Required: NF Type, GBA Subscriber data type(s), Key for GBA Subscriber data type(s).

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: Subscription Data.

##### X.2.1.2.4	Nhss_GbaSDM_Unsubscribe service operation

Service operation name: Nhss_GbaSDM_Unsubscribe

Description: The NF consumer unsubscribes for updates to Requested data.

Inputs, Required: GBA Subscriber data type(s), Key for GBA Subscriber data type(s).

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

##### X.2.1.2.5	Nhss_GbaSDM_Notification service operation

Service operation name: Nhss_GbaSDM_Notification

Description: This service operation enables HSS to notify a NF of any changes to what the NF subscribed to.

Inputs, Required: GBA Subscriber data type(s), Key for each GBA Subscriber data type(s).

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

#### X.2.1.3	Nhss_GbaUEAuthentication service

##### X.2.1.3.1	Nhss_GbaUEAuthenticate_Get service operation

Service operation name: Nhss_GbaUEAuthenticate_Get

Description: This service operation is used between the BSF and the HSS to request the authentication data of the end user.

Inputs, Required: User Identity(-ies), Authentication Data (Authentication Scheme).

User Identity shall be one of IMSI, MSISDN, IMPI, IMPU. At least one of User Identities shall be presented.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: User Identity, Authentication Data (e.g. AV).

In case only MSISDN or IMPU is present in the request, the HSS returns IMSI or IMPI in the response.

### X.2.2 BSF Services

#### X.2.2.1 General

The following table shows the services exposed by an SBI capable BSF.

Table X.2.2.1-1: GBA Services provided by an SBI capable BSF

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nbsp_Gba | BootStrapInfo | Request/Response | NAF |

#### X.2.2.2	Nbsp_Gba service

##### X.2.2.2.1	General

This clause describes the SBA interfaces exposed by the BSF for the purpose of providing the bootstrap information to the NAF for the derivation of the application key material (e.g. Ks_(ext/int)_NAF).

##### X.2.2.2.2	Nbsp_Gba_BootstrapInfo service operation

Service operation name: Nbsp_Gba_BootstrapInfo

Description: This service operation is used between the BSF and the NAF to request the key material key material agreed during bootstrapping from the UE to the BSF. It is also used to fetch application-specific user security settings from the BSF, if requested by the NAF.

Inputs, Required: B-TID, NAF-Id.

Inputs, Optional: Flag to indicate that the NAF is GBA_U aware, identifier of the application-specific USS.

Outputs, Required: Key material, bootstrapping time, key lifetime. The key material consists of Ks_NAF in case of GBA_ME and Ks_ext_NAF in case of GBA_U. The key lifetime is the lifetime associated to the key material.

Outputs, Optional: Key material, Application-specific USS, Private Identity.

NOTE X: Depending on the value of the GBA_U aware flag, more key material (i.e. Ks_int_NAF) may be returned as optional output.

### X.2.3	Mapping of Zh, Zn operations and terminology to SBI services

### X.2.3.1	General

This clause gives mappings from Zh, Zn operations to SBI services and service operations.

### X.2.3.2	Mapping of Zh messages to HSS SBI services

The following table defines the mapping between Zh messages and HSS SBI services and service operations:

Table X.2.3.2-1: Zh messages to HSS SBI services and service operations mapping

| Zh message | Source | Destination | HSS SBI service operation name |
| --- | --- | --- | --- |
| Zh interface: BSF retrieves AV and user profile | BSF | HSS | Nhss_GbaUEAuthentication_Get Nhss_GbaSDM_Get (see NOTE 1) Nhss_GbaSDM_Subscribe (see NOTE 1) Nhss_GbaSDM_Unsubscribe (see NOTE 1) |
|  | HSS | BSF | Nhss_GbaSDM_Notification (see NOTE 1) |
| NOTE 1: Corresponds to the GUSS retrieval during execution of the authentication of the end user. |  |  |  |

### X.2.3.3	Mapping of Zn messages to BSF SBI services

The following table defines the mapping between Zn messages and BSF SBI services and service operations:

Table X.2.3.3-1: Zn messages to BSF SBI services and service operations mapping

| Zn message | Source | Destination | BSF SBI service operation name |
| --- | --- | --- | --- |
| Zn interface: NAF requests the bootstrapping information from the BSF | NAF | BSF | Nbsp_Gba_BootstrapInfo |

## X.X	SBI Capable NF Discovery and Selection

Editor’s Note: The discovery and selection needs revisit after check with SA2

## X.X.1	General

During the GBA procedures SBI capable network functions such as the BSF and NAF need to discover and select other SBI capable network functions such as the HSS and the BSF respectively.

## X.X.2	SBI Capable HSS Discovery and Selection

An SBI capable BSF performs discovery and selection of an SBI capable HSS. The SBI capable BSF shall utilize the NRF to discover an SBI capable HSS unless the information about SBI capable HSS instance(s) is available by other means, e.g. locally configured on the SBI capable BSF. The HSS selection function in SBI capable BSF entities selects an SBI capable HSS instance based on the available SBI capable HSS instances (obtained from the NRF or locally configured).

An SBI capable BSF always selects an SBI capable HSS within its own PLMN. The HSS selection should consider one of the following factors when available to the SBI capable BSF:

1.	HSS Group ID of the UE's user identity (IMSI/IMPI or MSISDN/IMPU).

2.	IMSI/IMPI; e.g. the SBI capable BSF selects an SBI capable HSS instance based on the IMSI/IMPI range the UE's IMSI/IMPI belongs to, configured locally or based on the results of a discovery procedure with NRF using the UE's IMSI/IMPI as input for HSS discovery.

3.	MSISDN/IMPU; e.g. the SBI capable BSF selects an SBI capable HSS instance based on the MSISDN/IMPU range the UE's IMSI/IMPU belongs to, configured locally or based on the results of a discovery procedure with NRF using the UE's MSISDN/IMPU as input for HSS discovery.

Unless the information about the interface type to be used towards HSS is locally configured on the SBI capable BSF, an SBI capable BSF can also use the NRF to decide the type of interface (SBI vs diameter) to be used towards HSS similarly as defined for SBI capable IMS entities in TS 23.228 [8]. For this purpose, an SBI capable BSF can send a Nnrf_NFDiscovery_Request to NRF as defined in TS 23.502 [XY] to discover SBI capable HSS instances within a given PLMN. The SBI capable BSF may store all returned SBI capable HSS instances and their NF profiles for subsequent use, including, if applicable, supported IMSI/IMPI and/or MSISDN/IMPU ranges, and/or HSS Group IDs. If no SBI capable HSS instance is available in the PLMN, then the NRF replies to the SBI capable BSF with no information. In this case, the SBI capable BSF may then attempt to communicate with the HSS using legacy GBA protocols.

## X.X.3	SBI Capable BSF Discovery and Selection

An SBI capable NAF performs discovery and selection of an SBI capable BSF. The SBI capable NAF shall utilize the NRF to discover an SBI capable BSF unless the information about SBI capable BSF instance(s) is available by other means, e.g. locally configured on the SBI capable NAF. The BSF selection function in SBI capable NAF entities selects an SBI capable BSF instance based on the available SBI capable BSF instances (obtained from the NRF or locally configured).

The BSF selection in an SBI capable NAF shall consider the BSF server name included in the B-TID provided by the UE.

Unless the information about the interface type to be used towards the BSF is locally configured on the SBI capable NAF, an SBI capable NAF can also use the NRF to decide the type of interface (SBI vs diameter) to be used towards BSF. For this purpose, an SBI capable NAF can send a Nnrf_NFDiscovery_Request to NRF as defined in TS 23.502 [XY] to discover SBI capable BSF instances within a given PLMN. The SBI capable NAF may store all returned SBI capable BSF instances and their NF profiles for subsequent use. If no SBI capable BSF instance is available in the PLMN, then the NRF replies to the SBI capable NAF with no information. In this case, the SBI capable NAF may then attempt to communicate with the BSF using legacy GBA protocols.

Editor’s Note: Roaming considerations are FFS.

**** END OF CHANGES ****