- 3GPP TSG-SA3 Meeting #101e	draft_S3-203017-r1

- e-meeting, 9 – 20 November 2020	revision of S3-203017

- Source:	CATT

- Title:	pCR to TR33.847-Update Solution#3 for removing some ENs

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

Approve the pCR below

# 2		Rationale

(1) In order to support out of coverage scenario (MIC is checked in UE) and more security flexibility (MIC is checked in different entities), Discovery Key can be provided to the DDNMF in the HPLMN of the monitoring UE and/or the Monitoring UE.

(2) Remove following two ENs through adding "This solution does not address the discovery key generation and key delivery protocol used in the discovery procedure, which is up to the conclusion of key issue #2."

Editor’s note: How to derive the discovery key is FFS.

Editor’s note: Whether this solution based on CP or UP is FFS.

(3) Correct some errors in step 15.

# 3	Detailed proposal

#### *** BEGIN CHANGES ***

## 6.3	Solution #3: Reuse LTE security mechanism for 5G ProSe open discovery

### 6.3.1	Introduction

This solution addresses Key Issue #1(Discovery message protection). It proposes to reuse the open discovery security mechanism specified in TS 33.303 [6] for 5G ProSe open discovery. This solution does not address UE-to-Network and UE-to-UE relay discovery.

In LTE ProSe, the ProSe function is used to provide the UE with the necessary security material in order to protect discovery messages transmitted over the air. In 5G ProSe, the 5G Direct Discovery Name Management Function (DDNMF) is used to replace the ProSe function in the open discovery.

This solution allows the discovery key to be provided to the DDNMF in the HPLMN of the monitoring UE and the monitoring UE in order to support out of coverage scenario and more security flexibility.

Editor’s note: It’s FFS about new security parameters for 5G that is different from LTE ProSe.

Editor’s note: The detailed security-related parameters in the announcing message are FFS.

Editor’s note: It’s FFS how to support the security flexibility.

### 6.3.2	Solution details

This solution does not address the discovery key generation and key delivery protocol used in the discovery procedure, which is up to the conclusion of key issue #2.

The open discovery security procedure is described as follows:

- The announcing UE sends a Discovery Request message containing the ProSe Application ID to the DDNMF in its HPLMN in order to be allowed to announce a code on its serving PLMN (either VPLMN or HPLMN).

- If the announcing UE wants to send announcements in the VPLMN, the DDNMF in the HPLMN requests authorization from the VPLMN DDNMF by sending Announce Auth.() message.

- VPLMN DDNMF responds with an Announce Auth. Ack () message.

- The DDNMF in HPLMN of the announcing UE returns the ProSe App Code that the announcing UE can announce and a Discovery Key associated with it. The DDNMF stores the Discovery Key with the ProSe App Code. In addition, the DDNMF provides the UE with a CURRENT_TIME parameter, which contains the current UTC-based time at the DDNMF, a MAX_OFFSET parameter, and a Validity Timer. The UE sets a clock which is used for ProSe authentication (i.e. ProSe clock) to the value of CURRENT_TIME and the UE stores the MAX_OFFSET parameter, overwriting any previous values. The announcing UE obtains a value for a UTC-based counter associated with a discovery slot based on UTC time. The counter is set to a value of UTC time in a granularity of seconds. The UE may obtain UTC time from any sources available, e.g. the RAN via SIB16, NITZ, NTP, GPS, via Ub interface (in GBA) (depending on which is available).

- The UE starts announcing, if the difference between UTC-based counter provided by the system associated with the discovery slot and the UE’s ProSe clock is not greater than the MAX_OFFSET and if the Validity Timer has not expired. For each discovery slot it uses to announce, the announcing UE calculates a 32-bit Message Integrity Check (MIC) to include with the ProSe App Code in the discovery message. Four least significant bits of UTC-based counter are transmitted along with the discovery message. The MIC is calculated as described in clause A.2 of TS 33.303 [6] using the Discovery Key and the UTC-based counter associated with the discovery slot.

- The Monitoring UE sends a Discovery Request message containing the ProSe Application ID to the DDNMF in its HPLMN in order to get the Discovery Filters that it wants to listen for.

- The DDNMF in the HPLMN of the monitoring UE sends Monitor Req. message to the DDNMF in the HPLMN of the announcing.

- The DDNMF in the HPLMN of the announcing UE sends Monitor Resp. message to the DDNMF in the HPLMN of the monitoring. If MIC needs to be checked by the DDNMF in the HPLMN of the monitoring UE or the monitoring UE, the Discovery Key should be contained in the response message.

- The DDNMF returns the Discovery Filter containing either the ProSe App Code(s), the ProSe App Mask(s) or both along with the CURRENT_TIME and the MAX_OFFSET parameters. The UE sets its ProSe clock to CURRENT_TIME and stores the MAX_OFFSET parameter, overwriting any previous values. The monitoring UE obtains a value for a UTC-based counter associated with a discovery slot based on UTC time. The counter is set to a value of UTC time in a granularity of seconds. The UE may obtain UTC time from any sources available, e.g. the RAN via SIB16, NITZ, NTP, GPS (depending on which is available). If MIC needs to be checked by the monitoring UE, the Discovery Key should be contained in the response message.

- The Monitoring UE listens for a discovery message that satisfies its Discovery Filter, if the difference between UTC-based counter associated with that discovery slot and UE’s ProSe clock is not greater than the MAX_OFFSET of the monitoring UE's ProSe clock. If the monitoring UE has the Discovery Key, the MIC check is performed locally, and steps 11 to 15 are omitted.

- On hearing such a discovery message, and if the UE needs to check the MIC for the discovered ProSe App Code, the Monitoring UE sends a Match Report message to the DDNMF in the HPLMN of the monitoring UE. The Match Report contains the UTC-based counter value with four least significant bits equal to four least significant bits received along with discovery message and nearest to the monitoring UE’s UTC-based counter associated with the discovery slot where it heard the announcement, and other discovery message parameters including the ProSe App Code and MIC. If the DDNMF in the HPLMN of the monitoring UE has the Discovery Key, the MIC check is performed locally, and steps 12 to 14 are omitted.

- The DDNMF in the HPLMN of the monitoring UE passes the discovery message parameters including the ProSe App Code and MIC and associated counter parameter to the DDNMF in the HPLMN of the announcing UE in the Match Report message.

- The DDNMF in the HPLMN of the announcing UE shall check the MIC is valid.

- The DDNMF in the HPLMN of the announcing UE shall acknowledge a successful check of the MIC to the DDNMF in the HPLMN of the monitoring UE in the Match Report Ack message.

- The DDNMF in the HPLMN of the monitoring UE acknowledges the check result to the monitoring UE.

Figure 6.3.2-1: Open discovery security procedure

### 6.3.3	Evaluation

TBD

#### *** END OF CHANGES ***