- 3GPP TSG SA WG3 Meeting #102bis-e                            	S3-210910

- e-meeting, 1 – 5 March 2021	                         revision S3-21abcd

- Source:	Huawei, Hisilicon

- Title:	Solution Update for Solution #12

- Document for:	Approval

Agenda Item:	2.12

# Decision/action requested

This contribution proposes to update current solution#12 in TR 33.857

# References

# Rational

This contribution proposes to update current solution to address existing ENs and add evaluation for solution #12 in TR 33.857.

# Detailed proposal

*************** Start of 1st Change ****************

- 6.12	Solution #12: Authentication for UE Onboarding for SNPN

- 6.12.1	Introduction

This solution addresses key issue #4.

UE

(R)AN

UPF

N2

N4

PS

N1

N3

N6

N12

N13

AMF

SMF

AUSF

UDM

N11

N8

O-SNPN

SO-SNPN

DCS

Nxx

Trusted 3rd party

PAF

TBD

Nxy

The authentication architecture is shown in figure 6.12.1-1.

Figure 6.12.1-1: Architecture for authentication for UE onboarding for SNPN

The authentication related functions in the architecture are:

- Default Credential Server (DCS), stores UE’s default credential. The solution assumes that the owner of DCS belongs to 3rd party, who is different from SO. If the SO decides to use DCS to authenticate the UE, there is an agreement between 3rd party and SO, and the SO trusts 3rd party.

- Primary Authentication Function (PAF) is introduced in SNPN for translation of SBI protocol and AAA protocol. The function can be collocated with NSSAAF, or AUSF.

- 6.12.2	Solution details

- 6.12.2.1	Authentication for onboarding with default credentials is provisioned in UDM

UE

RAN

AMF

AUSF

UDM

2. Construct SUCI

8. Primary Authentication

a.Provisioning unique ID and default credential

10. AS SMC

9. NAS SMC

b.Provisioning unique ID, authentication method and default credential

1. Broadcast

(on-boarding information)

3. Registration Request

(SUCI)

4. Nausf_UEAuthentication_

Authenticate Request (SUCI)

5. Nudm_UEAuthentication

_Get Request (SUCI)

6. Retrieve default credential

7. Nudm_UEAuthentication_

Get Response (SUPI, AV,

authentication method)

O-SNPN

SO-SNPN

Figure 6.12.2-1: Authentication for onboarding with default credentials is provisioned in UDM

Preconditions:

- UE has been provisioned with unique ID, and default credential. In case that the UE is in automatic selection model, the UE may be provisioned with a priority list of O-SNPNs.

- UDM has been provisioned with unique ID, default credential and authentication method at onboarding phase.

Procedures:

1. 	RAN broadcasts onboarding information. The onboarding information includes indication for onboarding support, and SNPN identity of the O-SNPN and SNPN identity of the connected SO-SNPN.

2.	The UE may manually get an SNPN ID of SO-SNPN. The UE may output list of SO-SNPN in the onboarding information in the scream, and the user can select one of an expected SO-SNPN.

The UE may automatically get an SNPN ID of SO-SNPN. The UE may be provisioned with a priority list of O-SNPNs. The UE can select O-SNPN using priority list and O-SNPN in the onboarding information, and get the related SNPN ID of SO-SNPN in the onboarding information.

The UE constructs SUCI using the schemes described in TS 33.501 [2] according to SNPN ID of SO-SNPN and UE’s unique ID, in the following manner. The Home Network Identifier is set as SNPN identity of SO-SNPN so that the AMF can select the right AUSF. The Scheme Output is set as unique ID.

3.	The UE sends the Registration Request message to the SEAF, containing SUCI.

4.	The SEAF sends Nausf_UEAuthentication_Authenticate Request message to AUSF. The message includes SUCI.

5.	The AUSF sends the Nudm_UEAuthentication_Get Request message to UDM. The message includes SUCI.

6.	The UDM de-conceals the SUCI to SUPI, and gets the unique ID. The UDM shall check whether the unique ID is provisioned, if not, the UDM shall reject the registration. If the UDM has been provisioned with the related default credential, the UDM retrieves default credential and authentication method according to unique ID. The UDM may generate AV according to the default credential and authentication method.

7. The UDM sends the SUPI, authentication method and AV to the AUSF in the Nudm_UEAuthentication_Get Response.

8.	The UE and AUSF perform the authentication procedure. After successful authentication, the UE and SEAF derive KSEAF and KAMF. If the authentication is failed, the UE may re-select the O-SNPN or SO-SNPN as depicted in step 2.

9.	The UE and the AMF perform NAS SMC procedure to activate NAS security.

10. The UE and the RAN perform AS SMC procedure to activate AS security.

NOTE:	If the SO does not want to continue the use of the default credential in the UDM except for the initial access for onboading (e.g., default credential received from an external entity), the procedure depicted above can be used to replace the default credential with a different credential.

- 6.12.2.2 Authentication for onboarding with default credentials is provisioned in DCS

PAF

UE

RAN

SEAF

AUSF

UDM

2. Construct SUCI

a.Provisioning unique ID and default credential

16. AS SMC

15. NAS SMC

b.Provisioning unique ID

1. Broadcast

(on-boarding information)

3. Registration Request

(SUCI)

4. Nausf_UEAuthentication_

Authenticate Request (SUCI)

5. Nudm_UEAuthentication

_Get Request (SUCI)

6. Cannot retrieve default credential

7. Nudm_UEAuthentication

_Get Response (SUPI)

O-SNPN

SO-SNPN

DCS

b.Provisioning unique ID,  authentication method and default credential

8. SBI (EAP trigger,

AAA address)

9. AAA (EAP trigger)

10. EAP (e.g. EAP-TLS)

11. AAA(EAP success, MSK)

12. SBI (MSK,

EAP success)

13. Nausf_UEAuthentication_

Authenticate Response (EAP success, Kseaf)

14. Auth-Req. (EAP success, ngKSI, ABBA)

Trusted 3rd party

Figure 6.12.2-2: Authentication for onboarding with default credentials is provisioned in DCS

Preconditions:

- UE has been provisioned with unique ID, and default credential. In case that the UE is in automatic selection model, the UE may be provisioned with a priority list of O-SNPNs.

- UDM has been provisioned with unique ID at onboarding phase

- DCS has been provisioned with unique ID, authentication method and default credential.

Procedures:

1 - 5. 	The same steps with step 1-5 in clause 6.12.2.1.

6.	The UDM de-conceals the SUCI to SUPI, and gets the unique ID. The UDM shall check whether the unique ID is provisioned, if not, the UDM shall reject the registration. If the UDM has been provisioned with the unique ID, but has not been provisioned the related default credential, the UDM sends the SUPI, and DCS address derived from SUPI to the AUSF directly.

7. The UDM sends the SUPI and DCS address to AUSF in the Nudm_UEAuthentication_Get Response.

8.	The AUSF invokes external primary authentication service provided by PAF. The AUSF sends SBI message containing the DCS address and EAP trigger (e.g. EAP-TLS start) message to the PAF.

9. The PAF finds DCS according to the DCS address, translates SBI message to AAA protocol, and sends the EAP trigger message to the DCS.

10. - 14. Since DCS can be regarded as an external AAA, those steps can reuse authentication procedure steps similar with authentication solutions with credentials owned by an external AAA in key issue #1, e.g. solution 1, 3, 4, 5, etc. After successful authentication, the UE and AMF derives the KAMF. If the authentication is failed, the UE may re-select the O-SNPN or SO-SNPN as depicted in step 2.

15. The UE and the AMF perform NAS SMC procedure to activate NAS security.

16. The UE and the RAN perform AS SMC procedure to activate AS security.

- 6.12.3	System impact

In case that authentication for onboarding with default credentials is provisioned in UDM, UE, RAN, UDM may be impacted.

In case that authentication for onboarding with default credentials is provisioned in DCS, UE, RAN, AUSF, PAF (which may be collocated with AUSF), and UDM may be impacted. DCS can be legacy AAA.

- 6.12.4	Evaluation

TBA.

*************** End of 1st Change ****************