- 3GPP TSG SA WG3 Meeting #102bis-e                            	S3-210912

- e-meeting, 1 – 5 March 2021	                         revision S3-21abcd

- Source:	Huawei, Hisilicon

- Title:	Solution Update for Solution #2

- Document for:	Approval

Agenda Item:	2.16

# Decision/action requested

This contribution proposes to update current solution#2 in TR 33.866

# References

# Rational

This contribution proposes to update current solution for solution #2 in TR 33.866.

# Detailed proposal

*************** Start of 1st Change ****************

- 6.2	Solution #2: Network Analysis Framework for DDoS Attack

- 6.2.1	Introduction

The solution addresses key issue #2.1: Cyber-attacks Detection supported by NWDAF.

- 6.2.2	Solution details

- 6.2.2.1 Introduction

As depicted in clause 6.7.5 in TS 23.288 [4], the NWDAF could collect the following input data:

- Exceptions information from AF, including: IP address 5-tuple, exception ID, exception level, and exception trend.

- UE mobility information from OAM is UE location carried in MDT data.

- Network data related to UE mobility from AMF, including: UE ID, UE location, Timestamp, TAC, frequent mobility registration update.

- Service data related to UE mobility provided by AF, including: UE ID, Application ID, UE location, Timestamp.

- Service data related to UE communication provided by SMF, AF, UPF, including: UE ID, group ID, S-NSSAI, DNN, Application ID, Expected UE behaviour parameters, communication description per application (e.g. communication start, communication stop, UL data rate, DL data rate, traffic volume), TAC.

The NWDAF could output the following: Exception ID, Exception Level, Exception trend, UE characteristics, SUPI list (1..SUPImax), Ratio, Amount, Additional measurement, Confidence.

Specifically, exception ID can be “Suspicion of DDoS attack” means that the UE may trigger a DDoS attack. In this case, Additional measurement is “Victim's address (target IP address list)”. And the mitigation can be “PCF may request SMF to release the PDU session. SMF may release the PDU session and apply SM back-off timer.”

However, the analysis is just for DDoS attack to external AF.

DDoS attack to internal NF, e.g. RAN, Core Network should also be investigated. In order to make it more clear to capture the DDoS analysis, it is proposed a network analysis framework for DDoS attack.

- 6.2.2.2 Network Analysis Framework for DDoS attack

The framework is depicted in table 6.2.2.2-1. In column DDoS attack, target network entity and attack method shall be clarified. In column analysis, input, output and mitigation are listed as the same way as TS 23.288 [4]. With the framework, it will be more clear how to capture attack and how to detect the DDoS attack.

Table 6.2.2.2-1	Network Analysis Framework for DDoS attack.

| DDoS Attack |  | Analysis |  |  |
| --- | --- | --- | --- | --- |
| Target | Method | Input | Output | Mitigation |
| AF | DDoS using heavy UP traffic | AF: GPSI, external group ID, Exception information (IP address 5-tuple, exception ID, exception level, and exception trend), Application ID, communication description per application (communication start, communication stop, UL data rate, DL data rate, traffic volume), Expected UE Behaviour parameters SMF: SUPI, internal group ID, Application ID UPF: UE communication description per application (communication start, communication stop, UL data rate, DL data rate, traffic volume) AMF: TAC | DDoS to AF | PCF may request SMF to release the PDU session. SMF may release the PDU session and apply SM back-off timer. |
| RAN | DDoS using heavy RRC signaling | OAM: Global RAN Node ID, time stamp, SUPI, initial RRC message number AMF: Global RAN Node ID, time stamp, SUPI, initial NAS message number | DDoS to RAN Victim RAN Node ID Malicious SUPI | AMF may provide AMF UE N2AP ID and RAN UE N2AP ID to RAN of malicious SUPI. RAN may treat the malicious UEs based on local policy, e.g. release its resource. |
| AMF | DDoS using heavy NAS signaling | OAM: Global RAN Node ID, time stamp, SUPI, initial RRC message number AMF: AMF instance ID, Global RAN Node ID, time stamp, SUPI, initial NAS message number, initial SM message number | DDoS to AMF Victim AMF instance ID Malicious SUPI | AMF may treat the malicious UEs based on local policy, e.g. release its resource. |

Editor’s Note: Rational for input parameters that how to address DDoS are FFS.

Editor’s Note: Details description for input parameters are FFS.

- 6.2.3	Evaluation

TBA

*************** End of 1st Change ****************