3GPP TSG-SA3 Meeting #102bis-e 	S3-210964-r1

- e-meeting, 1 - 5 March 2021											Revision of S3-20xxxx

- Source:	Huawei, HiSilicon

- Title:	Add evalution to solution #4

- Document for:	Approval

Agenda Item:	2.11

# 1	Decision/action requested

SA3 is kindly requested to approve the proposed evaluation.

# 2	References

# 3	Rationale

Solution #4 in TR 33.850 was proposed to address the authentication and authorization apsects of multicast communication server. This contribution proposes to add the evaluation to solution #4.

# 4	Detailed proposal

******************* Start of Change 1 *******************

## 6.4	Solution #4: Authentication and authorization for multicast communication service

### 6.4.1	Solution overview

This solution, which is based on existing EAP based secondary authentication, addresses the key issue #1 Security of authentication and authorization for multicast communication service.

### 6.4.2	Solution details

In the solution below, the (MB-)SMF is either the SMF for managing in the MBS session and controlling of MBS transport (e.g. MB-UPF configuration, RAN configuration) or the SMF for managing the per-UE PDU session. The two may be the same network function.

Figure 6.4.2-1: Authentication and authorization procedure

- UE sends a request to join a multicast service. The request is forwarded to the (MB-)SMF. According to the TR 23.757 [2], there are two ways for step 1, the control plane join and the user plane join. For the control plane join, the UE initiates the request for a PDU session establishment/modification and includes the identifier of the multicast service that the UE wishes to join. The request is forwarded to the (MB-)SMF through the control plane. For the user plane join, the UE sends an IGMP/MLR message including the identifier of the multicast service/application to a UPF. The UPF forwards the IGMP/MLR message the (MB)-SMF. The identifier of the multicast service may be a TMGI (Temporary Mobile Group Identifier) or a source specific IP multicast address.

If not available locally, the (MB-)SMF retrieves the subscription data from the UDM.

- The (MB-)SMF determines that authentication and authorization is needed for the MBS communication service based on the subscription data.

- The (MB-)SMF sends an EAP request and the identifier of the multicast service/application to the UE to request the EAP identity used for the multicast service/application.

- The UE responds with an EAP response with the EAP identity and the identifier of the multicast service/application.

To avoid the round-trip in step 3 and 4, the UE may also send the EAP identity in step 1 if the control plane join is used, similar to the EAP based secondary authentication by an external DN-AAA server in 33.501.

6-7.  The (MB-)SMF sends the received EAP identity and the identifier of the multicast service/application to the AAA server through a (MB-)UPF.

- EAP messages are exchanged between the AAA server and the UE.

- After the successful completion of the authentication procedure, DN AAA server shall send EAP Success message to the (MB-)SMF.

- If the UE is authorized to join the multicast service/application based on the subscription, then the (MB-SMF) will proceed with PDU session establishment/modification. The (MB-)SMF sends the EAP access and the identifier of the multicast service/application to the UE.

Editor's Note: Whether SMF and MB-SMF are separated needs to be revisited once SA2 has conclusion.

### 6.4.3	Solution evaluation

This solution addresses KI#1 “Security of authentication and authorization for multicast communication service”. It re-uses the EAP based secondary authentication and authorization in TS 33.501[6] with the adaptation that a TMGI or a source specific IP multicast address is used to identify a multicast service.

Editor's Note: Further evaluation is FFS.

*************************** End of Change 1 **************************