3GPP TSG-SA3 Meeting #102-e 	draft_S3-210371-r1

- e-meeting, 18 - 29 January 2021											Revision of S3-20xxxx

- Source:	Huawei, HiSilicon

- Title:	Updates to solution #5

- Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

SA3 is kindly requested to approve the proposed updates to solution #5 in TR 33.850.

# 2	References

# 3	Rationale

In the last meeting of SA2, conclusions on the architecture for 5G multicast-broadcast services (5MBS) have been reached in TR 23.757. A converged architecture based on both architecture 1 and 2 is decided to be adopted, as shown below. Therefore, this contribution proposes to update the solution #5 in TR 33.850 to align with SA2’s architectural conclusions. This contribution also proposes to solve the editor’s note.

# 4	Detailed proposal

************************* Start of Change 1 **************************

## 6.5	Solution #5: Authorization revocation

### 6.5.1	Solution overview

This solution proposes how the authorization revocation is performed, for KI#1. When the content provider decides that the user authorization for a multicast service needs to be revoked, the content provider will inform the UDM/UDR about the revocation. The UDM/UDR will accordingly instructs the SMF to release the corresponding resources established for the user for the multicast service.

### 6.5.2	Solution details

In the solution below, the (MB-)SMF is either the SMF for managing in the MBS session and controlling of MBS transport (e.g. MB-UPF configuration, RAN configuration) or the SMF for managing the per-UE PDU session. The two may be the same network function

.

Figure 6.5.2-1: Authorization revocation

- The AF of the content provider provisions the information on the multicast service/application including the authorization information to the UDM/UDR.  The NEF is involved in the provisioning if the content provider belongs to a 3rd party.

- The UE has successfully joined a multicast service/application and the PDU session for the multicast service/application has been established.

- The (MB-)SMF subscribes to the UDM/UDR on the changes of the multicast information including the authorization information.  Step 2 may also be performed during step 1. SMF may get data from UDR via UDM/PCF/NEF.

- The content provider updates the multicast information.  The NEF is involved in the provisioning if the content provider belongs to a 3rd party.

- The UDM/UDR notifies the (MB-)SMF when the authorization for a UE to join the multicast service/application is revoked. The identifier of the multicast service/application and UE identifier (i.e. SUPI) is included in the notification. The identifier of the multicast service/application may be a Temporary Mobile Group Identifier (TMGI) or a multicast address.

- The (MB-)SMF may release the PDU session for the multicast service/application identified by the received  identifier if no UE is left using the multicast service.

When a UE decides to revoke the authorization, the UE may send a request to the content provider in the application layer, then the step 3, 4, and 5 of the solution apply.

Editor's note:	Whether SMF and MB-SMF are separated needs to be revisited once SA2 has conclusion.

### 6.5.3	Solution evaluation

TBC

*************************** End of Change 1 **************************