3GPP TSG-SA3 Meeting #104-e 	S3-212788

# e-meeting, 16 – 27 August 2021

# Source:	Apple

# Title:	User consent update

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is proposed to revise the solution#1 for user consent update in TR 33.867.

# 2	References

[1]	3GPP TR 33.867, v0.3.0

# 3	Rationale

As key issue 6.3 in TR 33.867 mentioned:

“UDR (via UDM services) holds the user consent for user related data which is provisioned by MNO as a user subscription information. A service provider (external to MNO domain) can use Nnef_parameterProvision_Update service to update or to revoke the user consent to the UDM/UDR (when applicable).”

Following the key issue 6.3, this contribution proposes that, when the external service provider needs to update the user consent to the UDM/UDR, the exsiting service “Nnef_parameterProvision_Update” shall be reused.

# 4	Detailed proposal

******	START OF CHANGE *********

## 7.1	Solution #1: User Consent for Exposure of information to Edge Applications in Real Time

### 7.1.1	Solution overview

The solution addresses key issue #1 “User Consent for Exposure of information to Edge Applications”.

The solution introduces a new function user consent function (UCF), the UCF can collect and maintain user consent from user in real time. The UCF is operator’s internal AF which can be merged with other internal AF, and can communicate with user.

Editor’s Note: how would the UCF know from what user it collected the user consent form is ffs.

Editor’s Note: It is FFS if UCF should communicate with user.

Editor’s Note: If UCF communicates with user, it is FFS how. It is also FFS how UCF can determine the user.

### 7.1.2	Solution details

Figure 7.1.2.1	Authorization based on User Consent

- AS requests for/update user’s sensitive information through “Nnef_parameterProvision_Update” , e.g. location or consent revocation. The invocation includes user consent information. The user consent information could be:  AS ID and user ID, or user consent update, etc. The user ID is associated to specific user.

- The NEF sends the API invocation with the AS ID and the user ID to NF provider to retrieve the information.

- The NF provider checks whether authorization of user consent is needed or not based on the invocated API according to local policy, e.g. regulation, if the invocation requests the user’s sensitive data, the NF provider may check whether user is allowed based on local policy. Otherwise, if the invocation requests non-user information or policy is not needed, the NF provider may not check the consent.

- If check of consent is needed, the NF provider sends Consent Request message with the API ID, the AS ID and the user ID to the UCF.

- The UCF checks whether consent is allowed. The UCF may push application request to the user for consent via application layer, the UCF may also push SMS to the user assocated with the MISDN. The request shows that user consent is needed for sharing your sensitive information to 3rd party. If consent is received or rejected, the UCF replies result to the NF provider. The UCF may store the consent for future use.

- The UCF sends Consent Response to the NF provider with the result.

- If the result shows that consent is allowed, the NF provider response to the API invocation, otherwise, the invocation is cancelled.

Editor’s Note: How AS’s purpose for data processing is determined is FFS.

Editor’s Note: How to track where data has been communicated to in case of a requirement for later deletion is FFS.

### 7.1.3	Solution evaluation

TBA

*********END OF CHANGE*********