3GPP TSG-SA3 Meeting #108e-AdHoc 	S3-222792

# e-meeting, 10 - 14 October 2022

# Source:	Ericsson

# Title:	New KI: Support for Emergency service over UE-to-Network Relaying

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This document proposes to add a new key issue for emergency service over UE-to-Network Relaying.

# 2	References

[1]	3GPP TR 23.700-33

# 3	Rationale

This contribution proposes a new key issue for emergency call.

# 4	Detailed proposal

**** START OF CHANGE ****

## 5.X	Key issue #X: Support for Emergency service over UE-to-Network Relaying

### 5.X.1	Key issue details

In TR 23.700-33 [1], key issue #7 is about support of Emergency for UE-to-Network Relaying. According to TS 22.101 [9], emergency service is defined as citizen to authority services, and it is left to the national authorities to decide whether the network accepts emergency calls e.g. for valid UE only, or for UEs without the SIM/USIM/ISIM.

In the 5G ProSe UE-to-Network relaying, if there is an emergency request from the remote UE, it implies that the Relay UE needs to be responsible for remote UE's emergency service.

A Remote UE which has no USIM inserted may need to initiate an emergency service with a network over PC5 interface via a Layer 3 UE-to-network relay. This Remote UE can only discover UE-to-network relays which indicates a RSC for emergency in clear text. Also, this Remote UE is not able to establish a secure PC5 link with a UE-to-network relay.

Based on the local regulation and the operator policy, the Relay UE needs to support to establish PC5 communication for emergency service, for either an authenticated Remote UE (e.g. Remote UE with USIM) or an unauthenticated Remote UE (e.g. Remote UE without USIM or authentication can’t complete for any reason).

### 5.X.2	Security threats

Local regulation can’t be fulfilled if a Remote UE without a USIM is not allowed to establish a PC5 communication with a UE-to-network Relay for emergency services.

### 5.X.3	Potential security requirement

The 5G system shall support the establishment of PC5 communication for emergency service over UE-to-network relay, for either an authenticated Remote UE (e.g. Remote UE with USIM) or an unauthenticated Remote UE (e.g. Remote UE without USIM or authentication can’t complete for any reason).

**** END OF CHANGE ****