- 3GPP TSG-SA3 Meeting #108Adhoc-e	S3-222510

- e-meeting, 10th – 14th October, 2022

- Source:	Huawei, HiSilicon

- Title:	Update Key Issue 1

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

Approve the changes in clause 4 for inclusion in 33.741.

# 2	References

NA

# 3	Rationale

The Editor’s Note is deleted by adding threat to each use case.

# 4	Detailed proposal

********* Begin 1st change*********

## 4.1	Key Issue #1: Ability of the home network to trigger primary authentication

### 4.1.1	Key issue details

In the 5G System, the home network control over the security of the UE has been strengthened compared to previous generations by many new mechanisms such as SUPI privacy, termination of the authentication procedure in the home network and the provisions for increased home network control and linkage to subsequent procedures. However, when it comes to triggering the authentication, then this is still under the control of the serving network.

The home network uses KAUSF or keys derived from K_AUSF to provide protection for various services, (e.g. interworking from 4G to 5G, SoR/UPU and AKMA services) and hence the home network would benefit from having the ability to be able to ensure a fresh KAUSF is available by triggeringan authentication, in particular to prevent counter wrap in SoR/UPU or after interworking from 4G when there might be no KAUSF available.

### 4.1.2	Security threats

.

Regarding AKMA, when KAF is expired, and the Ua* protocol does not support the key refresh, the behaviour of AF and the UE cannot be predicted, for example, the UE may delete the key, but the AF continue to use, and the service is broken, another example is the UE and the AF may continuous to use the key which lead to the timer is pointless, in this case, the longer the key is used, the weaker the protection is. .

Regarding interworking from 4G to 5G, if no primary authentication is triggered by the AMF or the guience in TS 33.501[3] is not implementated, the KAUSF may not be generated so that the home network cannot send the updated UPU date or SoR data to the UE when necessary.

Regarding SoR/UPU counter wrap around case, there is no discussion or agreement on how to handle this issue, but one example coud be the KAUSF is reused, and another example is the SoR/UPU cannot be used anymore.

### 4.1.3	Potential requirements

The home network may be able to trigger a primary authentication.

The messages in home network triggered primary authentication should be confidentiality protected, integrity protected, and anti-replay protected.

Note: All the solutions will be evaluated specific to the use case in the Annex A of this present document.

******** End of change*********