3GPP TSG-SA3 Meeting #112 	draft_S3‑234176-r2 was S3-234013

Goteborg, Sweden, 14 - 18 August 2023	(revision of xx-yyxxxx)

# Source:	Lenovo, Motorola Mobility, MITRE, Interdigital, Motorola Solutions, Charter Communications, Johns Hopkins University APL, Intel, US National Security Agency, Telefonica, NCSC, OTD_US, Deutsche Telekom, Keysight Technologies, Center for Internet Security, SDI Squared, Cablelabs, IIT Delhi, Philips International B.V., Nokia, Nokia Shanghai Bell, Samsung, NEC, Rakuten Mobile, Peraton Labs, CISA ECD, NTIA, Department of Telecom, British Telecom

# Title:	New SID on enablers for Zero Trust Security

# Document for:	Approval

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on enablers for Zero Trust Security

Acronym: FS_eZTS

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | x |  |
| No | x | x | x |  |  |
| Don't know |  |  |  |  | x |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_ZTS | SA3 | 960038 | Study on applicability of the Zero Trust Security principles in mobile networks |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification: N/A

# 3	Justification

The 5G system was designed with core security features to support authentication, communication security and authorization which form the basis of Zero Trust security [1]. Nevertheless, the ZT core principle also insists to ‘assume breach and verify explicitly’ to improve the security posture and if an attack happens to prevent lateral threat movement. An effective Zero Trust deployment is dependent on an effective set of security controls to assess, detect and report attacks on the 5G system.  In the ZT concept, these can be viewed as the inputs for zero trust security policy(ies).  Under the assumption of a breached 5G network when there is no sufficient mechanism(s) to detect the attacks and related abnormalities, further lateral movement will be unhindered. Furthermore, it may be possible for the malicious actor to gain unauthorized access and exfiltrate/manipulate network data. The 5G system supports mechanisms as described in TS 23.288 clause 6.7.5 [2] to identify risks (related to DDoS suspicion, mainly to identify a group of UEs or a specific UE with abnormal behaviour, e.g., being misused or hijacked), predict abnormal behaviours statistics/predictions, and improves access control (e.g., with NF actions such as PCF may request SMF to release the PDU session. SMF may release the PDU session and apply SM back-off timer). There is no corresponding 5G system security controls to identify risks to the NFs in the 5GC.

Due to the heterogeneity and varied NF deployment options, there is a chance that a NF(s) may experience configuration issues, modification in access privilege levels, encounter insider threats, or face cyberattacks. Thus, static trust of a NF and its behaviour should not be assumed to be intact and normal throughout its lifetime.  If a NF behaves abnormally, there should be a means to identify and improve the security controls as applicable in real-time. Real-time identification and security control changes have not yet been standardized in 3GPP (presuming each Network operator deployed their own choice for SIEM system). There were Rel.18 initiatives [1] undertaken to study the Zero Trust security principles. [1] provides a detailed security evaluation for Zero Trust Tenets 1,2,3, and 5, and suggests required data collection to enable continuous security monitoring and evaluation. However, evaluations for Tenets 4, 6 and 7 were incomplete (need to evaluate presuming SIEM system deployed externally). There is an ongoing global effort to adapt Zero Trust security principles [5][6][7][8][9][10][11][12] to overcome the emerging attack surface and lateral threat movement within the network. Meanwhile SA5 has also endorsed ‘Enablers for Security Monitoring’ as the Rel.19 study topic and cited SA3 as the collaboration group for this topic [3]. Therefore, it is proposed to analyse the following and identify necessary security adaptions as required [4]:

- For any NF that is compromised or exhibits abnormal behaviour, 3GPP security controls are needed to at least for example:

  - Identify the NF that has been compromised/behaving abnormally.

  - Suitable mechanisms are needed to prevent lateral attack movement and ensure continuous service availability. Simply terminating/isolating a compromised or misbehaving NF would impact all the ongoing services.

  - Mechanism(s) are needed to collect and report abnormal behavior related data (for external security analysis/monitoring) and to enable OAM security functions to use the results and in response provide appropriate actions to prevent threat of lateral movement and ensure service availability.

  - Mechanisms are needed to automatically identify, and correct compromise means to ensure the replacement NF will not be compromised in the same manner.

- Complete evaluations for Zero Trust Tenets 4, 6 and 7.

References:

[1] 3GPP TR 33.894, ‘Study on applicability of the Zero Trust Security principles in mobile networks’, Release 18.

[2] 3GPP TS 23.288, ‘Architecture enhancements for 5G System (5GS) to support network data analytics services’, Release 18.

[3] S5-234823, ‘Enablers for Security Monitoring’, SA5 Collection of Rel-19 potential topics for SA workshop presentation, Endorsed.

[4] S3-23aaa, ‘Discussion Paper on Rel-19 Study on Network based Zero Trust Security’.

[5] NIST Special Publication 800-207, ‘Zero Trust Architecture’, Zero Trust Architecture (nist.gov).

[6] National Security Agency, ‘Embracing a Zero Trust Security Model’, CSI_EMBRACING_ZT_SECURITY_MODEL_UOO115131-21.PDF (defense.gov).

[7] ITU SG 17, ‘Guidelines for zero trust based access control platform in telecommunication network’, https://www.itu.int/ITU-T/workprog/wp_item.aspx?isn=18032.

[8] ITU SG 13, ‘Assessing trust evaluation models for telecommunication networks’, https://www.itu.int/itu-t/workprog/wp_item.aspx?isn=18421.

[9] Department of Defense (DOD), ‘Zero Trust Reference Architecture’, Department of Defense Zero Trust Reference Architecture.

[10] National Cyber Security Centre (NCSC), ‘Zero Trust architecture design’, https://www.ncsc.gov.uk/collection/zero-trust-architecture/introduction-to-zero-trust.

[11] ATIS, ‘Enhanced Zero Trust and 5G’, https://www.atis.org/tops-council/enhanced-zero-trust-and-5g/

[12] MITRE, ‘Achieving Mission Assurance for Enterprises today and tomorrow – Zero Trust, The cloud, and other Tools’, https://apps.dtic.mil/sti/trecms/pdf/AD1172262.pdf.

# 4	Objective

The Objective of the study includes:

- Based on TR 33.894 KI#1 security requirement and Tenet 5 evaluation, analyze, and identify the data to be collected in the 5G core network to enable the external security evaluation and monitoring (if not completed in Rel.18).

NOTE: The external security evaluation and monitoring is up to operator’s implementation and outside the 3GPP domain.

- Analyse the impacts and threats related to compromised NF(s) and abnormal NF behaviors.

- For NFs that have been identified as compromised or misbehaving, provide support for employing appropriate security mitigations to prevent lateral movement and minimize impact to service availability.

  - Study how 3GPP security policies and controls can be enhanced to mitigate threat lateral movement and service availability issues.

- Provide recommendations for network based Zero Trust security adaptation, where the recommendations may include but are not limited to requirements, technical enhancements, and procedural fixes.

- Complete evaluations of Zero Trust Tenets 4, 6, and 7.

## TU estimates and dependencies

| TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | SA5 Dependency (Yes/No/Maybe) |
| --- | --- | --- | --- |
| 3.5 TUs (5 meetings) | 1.5 TUs (3 meeting cycles) | No | May be |

Total TU estimates for the study phase: 3.5 TUs (5 meeting cycles)

Total TU estimates for the normative phase: 1.5 TUs (3 meeting cycles)

Total TU estimates: 5 TUs Max

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 33.xxx | Study on enablers for Zero Trust Security | TSG#101 | TSG#104 | Sheeba Backia Mary B, Lenovo, Motorola Mobility, smary@lenovo.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| N/A | N/A | N/A | N/A |

# 6	Work item Rapporteur(s)

Sheeba Backia Mary B, Lenovo, smary@lenovo.com

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

SA5 for the management services (as applicable)

# 9	Supporting Individual Members

Supporting IM name

Lenovo

Motorola Mobility

MITRE

Interdigital

Motorola Solutions

Charter Communications

Johns Hopkins University APL

Intel

US National Security Agency

Telefonica

NCSC

OTD_US

Deutsche Telekom

Keysight Technologies

Center for Internet Security

SDI Squared

Cablelabs

IIT Delhi

Philips International B.V.

Nokia

Nokia Shanghai Bell

Samsung

NEC

Rakuten Mobile

Peraton Labs

CISA ECD

NTIA

Department of Telecom

British Telecom