3GPP TSG-SA3 Meeting #120	S3-250425

Athens, Greece, 17 - 21 February 2025

Title:	Reply LS on security questions related to NAS layer overhead reduction for data transfer using control plane CIoT

Response to:	LS (C1-247156/SA3-250316) on LS on security questions related to NAS layer overhead reduction for data transfer using control plane CIoT

Release:	Rel-19

Work Item:	NORDAT_CP

Source:	Qualcomm Incorporated (to be SA3)

To:	CT1

Cc:	-

Contact person:	Adrian Escott

aescott@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA3 thank CT1 for their LS (C1-247216) on security questions related to NAS layer overhead reduction for data transfer using control plane CIoT.

SA3 provides the following answers to CT1’s questions:

Question 1: Currently, SERVICE REQUEST message in EPS NAS uses short MAC and short sequence number. Would it be acceptable to use short MAC and/or short sequence number for the new NAS message format with minimized overhead for data transfer via NAS?

Answer: Short MAC is not acceptable as this new message format can contain user data. Short sequence number is acceptable and it is left to CT1 to decide between the risk of messages being discarded due to the extra possibility of sequence number mismatch due to the short sequence number and the gain of reducing overhead

Question 2: Currently, all initial NAS messages contain Key Sequence Indicator (KSI). Would it be acceptable not to include the KSI in the new initial NAS message format with minimized overhead for data transfer via NAS?

Answer: From a security perspective, it is acceptable to not include KSI in the new NAS message format.

Question 3: In the NAS message format with minimized overhead for data transfer via NAS, only a portion of the message will be ciphered. Is it acceptable for the ciphered portion not to be byte-aligned? Not having to byte-align the ciphered portion would allow for more efficient message encoding

Answer: From a security perspective, it is OK for the ciphered portion not to be byte-aligned. The encryption is applied by a simple XOR and hence does not require byte-alignment.

# 2	Actions

To CT1

ACTION: 	SA3 asks CT1 to take the above responses into account on their further work on the issue.

# 3	Dates of next TSG SA WG 3 meetings

SA3#121	7 – 11 April 2025		Goteborg, Sweden

SA3#122	19 - 23 May 2025		Fukuoka, Japan