3GPP TSG-SA3 Meeting #120	S3-250882

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Adding token based mechanism to draftCR of MEC ph3

Document for:	Approval

Agenda item:	4.17

Spec:	3GPP TS 33.558

Version:	18.2.0

Work Item:	EDGE_Ph3

Comments

This contribution proposes to add token based mechanism on the ID verification concluded in the clause 7.2 of TR 33.749.

* * * First Change * * * *

Z.x	Token based mechanism

Z.x.1	Introduction

This mechanism proposes usage of UE ID token, with which there will be no need to share private IP address with EEC and other entities in the EC architecture.

Z.x.2	Details

High-level overview of the mechanism is presented in Figure Z.1.2-1-1 and steps are explained in detail below.

Figure Z.1.2-1: UE ID token based mechanism

1. The EEC obtains a UE ID token from the UE ID server which is located inside the operator domain. UE ID server can identify the UE by using the private IP address of the PDU session. The UE ID Server can fetch the IP address from the IP packets received from the EEC. The UE ID Server can use the Nbsf_Management_Discovery service operation with UE address and IP domain to retrieve the session binding information of the UE. For the UE ID Server service consumption by the EEC, the UE where the EEC is running on needs to execute the primary authentication successfully and have a PDU session. The EEC can authenticate the UE ID Server by using server certificate. The EEC needs to be configured with the root certificate or certificate of the UE ID Server, or the Internet (public) PKI mechanism can be used. The EEC does not need to authenticate with the server since the server issues the UE ID token, which does not include cleartext sensitive information, to the EEC running on the UE who has already executed the primary authentication.

NOTE:	Since the issuer and resolver of the UE ID token is the UE ID Server, the details of the UE ID token (e.g., whether it is a signed token or a random number) can be left to implementation.

2. The EEC sends the token to the EES to learn edge specific UE ID.

3. The EES invokes the NEF IP by sending the UE ID token.

4. The NEF interacts with the UE ID server to identify the UE.

5. The NEF sends the UE ID to the EES.

6. The EES sends the response to the EEC.

The UE ID Server is an AF function located inside the HPLMN trust domain and before the execution of NAT. In step 1, the UE ID server uses the UE IP address to identify the UE. For the execution of step 4, the NEF needs to be able to consume the UE ID Server service to obtain the UE ID by providing the UE ID token.