3GPP TSG-SA3 Meeting #120	S3-250985

# Athens, Greece, 17 - 21 February 2025

Source:	ZTE Corporation,China Mobile, Nokia, Huawei

Title:	Pseudo-CR on Location Security

Document for:	Approval

Agenda item:	4.16

Spec:	3GPP TS 33.545

Version:	0.0.0

Work Item:	5G_Femto_Sec

Comments

Update location security enhancements of NR Femto according to TR conclusions.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

* * * Second Change * * * *

5.3	Location Security

5.3.1	General

The NR Femto MS and/or NR Femto GW (referred to in this section as the “verifying node”) shall perform location verification. Optionally, the SeGW can act as the verifying node to perform the location verification for the NR Femto node. The location verification is performed during or after the device authentication process. If the location verification fails, the connection from the NR Femto node to 5GC should be blocked as early as possible.

One or more types of following location information of NR Femto may be optionally stored in the verifying node by operators for location verification and the location security aspects in TS 33.320 [x] clause 8.1 shall be reused:

-	the public IP address of the broadband access device provided by the NR Femto;

-	the IP address and/or access line location identifier provided by broadband access provider;

-	information of the neighbouring cells surrounding the NR Femto;

-	geo-coordinates provided by a GNSS receiver embedded into the NR Femto.

The following information may also be used to perform location verification of NR Femto:

-	locations verification during the mutual authentication process by SeGW. Refer to clause 5.3.2 of the present document.

-	locations of UEs connected to the NR Femto. Refer to clause 5.3.3 of the present document.

5.3.2	Location verification during the mutual authentication process by SeGW

When the SeGW acts as the verifying node, the mutual authentication between NR femto and SeGW may followed by location verification for the Femto node. If the mutual authentication is IKEv2 certificate-based, an example call flow of the location verification during the authentication process is performed as shown in Figure x. As IKEv2 allows the inclusion of information data into Notify Payload, the location information of the NR Femto node may be carried in the Notify Payload (see Figure x) during IKEv2 procedures from the NR Femto to the SeGW.

Figure x: Location verification during the mutual authentication process by SeGW

0a.The location information and verification policy is pre-configured in SeGW.

0b.Optionally the location information and verification policy is dynamically provided by NR Femto management system or NF in 5GC.

1-3.The same as the steps 1-3 in TS 33.320[x] clause A.1.

4.	The same as the step 4 in TS 33.320[x] clause A.1. Additionally, the NR Femto sends its location information in the Notify Payload  with a Notification Type of LOCATION_INFO in the IKE_AUTH request. Notification Type of LOCATION_INFO is defined and configured in both NR Femto and SeGW.

5.	The same as the step 5 in TS 33.320[x] clause A.1.

6.	The SeGW processes the Notify payload of the IKE_AUTH request and verify the location of the NR Femto based on the policy of the operator.

7.	If the location verification is successful in step 6, the SeGW sends IKE_AUTH response to continue the authentication.

NOTE 1:	If the location verification fails,  the following procedure may not be executed based on the operator policy.

8-9.	The same as the steps 8-9 in TS 33.320[x] clause A.1.

5.3.3	Locations of UEs connected to the 5G NR Femto

NR Femto cells are expected to provide coverage over a small geographical area. The location of UEs connected to the NR Femto cells can only be within small distances, and this can be used by the verifying nodes to verify the 5G NR Femto location.

* * * End of Changes * * * *