3GPP TSG-SA3 Meeting #120	S3-250369

Athens, Greece 17 - 21 February 2025

# Source:	KDDI Corp.

# Title:	Use of AEAD in Next-Generation 3GPP System

# Document for:	Discussion

Agenda Item:	6

# 1	Decision/action requested

With this contribution, KDDI would like to start a discussion about changes that would allow a future 3GPP System to utilize AEAD algorithms for AS and NAS security. We believe it is necessary to consider implications of AEAD algorithms early on to ensure readiness of the next-generation system.

# 2	References

[1]	3GPP TS 33.501 Security architecture and procedures for 5G System

[2]	3GPP TS 35.240 Specification of the Snow 5G based 256-bits algorithm set

[3]	3GPP TS 35.243 Specification of the AES based 256-bits algorithm set

[4]	3GPP TS 35.246 Specification of the ZUC based 256-bits algorithm set

[5]	RFC 8452, “AES-GCM-SIV: Nonce Misuse-Resistant Authenticated Encryption”

[6]	Power Consumption of Common Symmetric Encryption Algorithms on Low-Cost Microchips; Marc Ohm, Lars Taufenbach, Karsten Weber, Timo Pohl

[7]	3GPP TS 38.323

[8]	3GPP TS 38.331

[9]	3GPP TS 24.301

[10]	NIST SP 800-38D

[11]	Authenticated Encryption: Relations among notions and analysis of the generic composition paradigm; Mihir Bellare, Chanathip Namprempre

[12]	The Order of Encryption and Authentication for Protecting Communications (Or: How Secure is SSL); Hugo Krawczyk

# 3	Rationale

## Motivation for AEAD

ETSI SAGE and 3GPP SA3 have recently completed specifications for 256-bit cryptographic algorithms. For the very first time in 3GPP, these specifications also include authenticated encryption with associated data (AEAD) algorithms [2][3][4]. With the industry’s increasing focus on higher throughput and data-intensive applications, SA3 should also consider adoption of these AEAD algorithms to be used for AS/NAS security in a next-generation 3GPP System.

Cryptographic algorithms operating in AEAD mode combine both encryption and integrity protection to a single operation. Generally speaking, this approach has two main benefits:

- Enhanced throughput and power consumption

  - Single-pass operation: Ideally, they provide data encryption and authentication in a single pass (in contrast to the two-pass “Encrypt, then MAC” scheme used today). This can also lead to reduced power consumption.

  - Reduced power consumption: There is data that suggests that, for smaller payloads, AEAD algorithms can exhibit reduced power consumption compared to other symmetric key schemes that generate the MAC / authentication tag separately [6]. Specific findings and data are described in detail below in [Section 4: Power Consumption Evaluation].

- Simplified system design and key management

  - Fewer keys to manage: With AEAD, only one key is needed for both encryption and integrity protection, simplifying key management.

  - Less Error-Prone: Using separate algorithms can introduce complexity and potential errors in their combination. AEAD reduces the risk of such errors by providing a single, well-defined process.

## Procedures impacted by AEAD

Considering the motivations outlined above, it is crucial to consider deployment for an AEAD mode compatible security architecture. To ensure a next-generation 3GPP System is able to utilize AEAD algorithms, the following key issues would need to be agreed.

### Algorithm negotiation

The current 5G system uses dedicated algorithms for encryption (128-NEAs) and integrity protection (128-NIAs). These algorithms are negotiated independently from each other. This means a given session may use the same or different algorithms for encryption and integrity protection, on both AS and NAS layer. When using AEAD algorithms, both operations are combined into one. Therefore, a next-generation system may also simplify the selection procedure and allow UE and network to negotiate a combined cipher suite. At the same time, AEAD algorithms still offer the flexibility of only providing either confidentiality or integrity protection, if required. What remains to be decided is how to reflect these choices (enc+int, enc-only, int-only) in AS/NAS SMC procedures. One potential solution may be as trivial as defining new code points for these options.

### AEAD algorithm inputs

#### Cryptographic keys

The current system uses two separate keys – one for encryption and another for integrity protection. A next-generation system utilizing AEAD algorithms would only require one key for both operations, allowing for a simplified key hierarchy. Similar to algorithm negotiation, a possible solution may be as trivial as omitting one part of the key derivation that is performed today in the AMF (KNAS à KNASenc / KNASint) and in the gNB (KgNB à KRRCenc / KRRCint, KgNB à KUPenc / KUPint).

#### Nonce

AEAD algorithms commonly require a nonce that is not to be reused as it ensures the uniqueness and integrity of the ciphertext. When a nonce is misused, in other words, a nonce is repeated for a given key, an attacker may be able to exploit the repetition to recover the encryption key or manipulate the ciphertext [5]. Without a mechanism in the 3GPP System to prevent such threat by providing a unique nonce, the algorithm may not be able to provide effective security.

Note: The Nonce may be constructed of multiple parameters yet to be defined, similar to the ciphering / integrity algorithms used today, e.g., COUNT, BEARER, DIRECTION [1].

Note: This key issue may require involvement of ETSI SAGE.

#### Associated Data

Authentication Encryption Algorithms also can have Associated Data the use of which is yet to be determined in the specifications. A next-generation system may require further study into how to utilize the Associated Data input for various possible use cases.

### Backwards compatibility with separate encryption and integrity protection algorithms

#### Mobility

Assuming that a next-generation 3GPP System would solely support AEAD algorithms to fully leverage the benefits mentioned earlier in this document, the intra-RAT mobility case would be trivial. In this scenario, all security termination points unanimously support AEAD. Other scenarios, such as inter-RAT handovers would fall under the “Interworking” section.

#### Interworking

It is fair to assume that existing generations of the 3GPP System (e.g., 4G/LTE, 5G) will not feature support for AEAD. In these scenarios, the question arises how to realize interworking between next-generation (AEAD-compatible) networks and legacy (AEAD-incompatible) networks. Specifically, one needs to consider differences in the security context maintained by the UE and the network as well as associated procedures, such as key derivation.

#### Multi-RAT Dual connectivity

Similar to mobility and interworking scenarios, the conditions remain the same with AEAD-compatible updated next generation networks, and previous networks without support. In these situations, there is a risk of inconsistent key sizes being used to protect different communication paths associated to a single subscriber session when using dual connectivity. Specifically, one can envisage a scenario in which the UE is connected to both a Master Node (MN) or Primary RAN and Secondary Node (SN) or Secondary RAN. Assuming a mixed deployment in which the MN already supports AEAD mode, but the SN only supports legacy algorithms (or vice versa), this can lead to a situation where the RRC and UP between the UE and the MN or SN is protected with algorithms using different modes resulting in inconsistent performance output.

### Order of Encryption and Integrity protection for NAS and AS security

In the 5G system, the order of encryption and integrity protection for AS security is MAC-then-Encrypt (MtE) [7]. For NAS security, is the opposite, i.e., Encrypt-then-MAC (EtM) [8]. The way that the ETSI SAGE has defined the new AEAD cryptographic algorithms aligns with the way NAS security works today, allowing for a single-pass operation to achieve encryption and integrity protection. To achieve the same in AS security, two passes of these AEAD algorithms are required. The MAC tag is generated in the first operation and the ciphertext is generated in the second.

While the procedure described above is not optimal from a performance standpoint, there are also publications that suggest EtM is preferrable from security perspective.  Bellare & Namprempre (2000) and Krawcyk (2001) analysed the security of authenticated encryption schemes and find EtM to be most secure in various cases. Specifically, EtM ensures integrity is independently verified before decryption, demonstrating to be secure when the MAC authenticates the ciphertext. This makes EtM resilient against attacks like padding oracle attacks or chosen-ciphertext attacks [11][12]. For MtE, even when the encryption function is secure and the MAC unforgeable, it may not be generically secure [12].

Considering these performance and security considerations, the order or encryption and integrity protection should also be studied in the context of the adoption of AEAD cryptographic algorithms.

# 4	Performance evaluation

## Throughput Evaluation

An analysis was conducted to compare the throughput of standalone encryption and integrity protection with that of the combined AEAD mode. For the purposes of this comparison, it was assumed that both encryption and integrity protection are required, i.e. encryption-only or integrity protection-only scenarios were considered out of scope.

We compared AES-256-CTR and GMAC to the AES-256-GCM algorithm, considering the message size distribution of NAS protocol messages as defined in TS 24.301[9] on both a PC and a mobile phone running an Intel® Core™ i7-9700 CPU at 3.00GHz and a Snapdragon 8 +Gen1 respectively. The NAS protocol is of particular interest because the size of Control Plane messages is more predictable than that of User Plane messages. For the purposes of this comparison, the assumed message size distribution is based on the maximum size of all messages defined in TS 24.301 [9]. While most NAS message have a defined maximum size below 16, 32, or 48 bytes, there are certain messages that include containers of variable length that can be up to 65kbyte long. That said, it is unlikely that NAS messages would regularly reach this maximum size in typical deployments. For our measurements, we therefore focus on message sizes of 16, 32, and 48 bytes as well as16,384 bytes as an example for larger NAS messages.

In order to compare the average throughput gain of AES-256-GCM against AES-256-CTR and GMAC the throughput of all algorithms is measured with the openssl speed command. We measure each algorithm’s throughput for block sizes of 16, 32, 48, and 16,384 bytes. To calculate the average throughput gain, we compare the time to process one block by AES-256-GCM against the combined time to process one block by AES-256-CTR and GMAC. Our measurements are summarized in the table below.

Note: We infer the performance of GMAC from results of the command openssl speed ghash. Since the openssl standard implementation performs initialise, update, and finalise processing steps, the ghash command is actually evaluating the performance of GMAC.

| A | B | C | D | E | F | G |
| --- | --- | --- | --- | --- | --- | --- |
| Block size (bytes) | NAS message size distribution (max. length) | Throughput AES-256-CTR (ns/block) | Throughput GMAC (ns/block) | Throughput AES-256-GCM (ns/block) | GCM efficiency gain over C+D (%) | B × F |  |
| 16 | 80.36% | 28.5149993 | 355.729594 | 188.477894 | 50.95 | 40.94 |
| 32 | 5.36% | 38.1020621 | 371.824229 | 202.981158 | 50.48 | 2.70 |  |
| 48 | 3.57% | 38.6969076 | 347.471754 | 206.838266 | 46.44 | 1.66 |  |
| 16384 | 10.71% | 4957.8747 | 2452.02811 | 5176.37904 | 30.14 | 3.23 |  |
|  |  |  |  |  |  | 48.53% |  |

Table 1: Throughput comparison analysis on Intel

| A | B | C | D | E | F | G |
| --- | --- | --- | --- | --- | --- | --- |
| Block size (bytes) | NAS message size distribution (max. length) | Throughput AES-256-CTR (ns/block) | Throughput GMAC (ns/block) | Throughput AES-256-GCM (ns/block) | GCM efficiency gain over C+D (%) | B × F |  |
| 16 | 80.36% | 14.4623209 | 209.601956 | 119.92878 | 46.48 | 37.35 |
| 32 | 5.36% | 14.9689906 | 211.48431 | 122.13364 | 46.07 | 2.47 |  |
| 48 | 3.57% | 16.8713102 | 212.986056 | 123.52597 | 46.26 | 1.65 |  |
| 16384 | 10.71% | 2831.69656 | 2638.36568 | 3825.5692 | 30.06 | 3.22 |  |
|  |  |  |  |  |  | 44.69% |  |

Table 2: Throughput comparison analysis on Galaxy

Based on these results, we can derive the following conclusions:

- AES-256-GCM is more efficient for all block sizes tested. The average throughput gain is approximately 48.53% for on the PC and 44.69% on the mobile device.

- Since all new 256-bit AICI algorithms specified by ETSI SAGE rely on the Mac5G framework which leverages GMAC, it can be assumed that for smaller message sizes, their performance would at least be limited by the performance of GMAC (if not by other factors).

Note: It is assumed that AEAD algorithms, when adopted in the 3GPP System and used to provide both encryption and integrity protection, would be operated in single-pass mode. If AEAD algorithms are not adopted, and encryption and integrity are provided separately, the performance is expected to be worse.

## Energy consumption Evaluation

Additionally, an analysis was conducted to compare the power consumption comparing the standalone encryption and integrity protection with the combined AEAD mode. Similar conditions apply to the above test, assuming that both encryption and integrity protection are required, i.e. encryption-only or integrity protection-only scenarios were considered out of scope. This particular test was conducted on a mobile device with Snapdragon 8 +Gen1. Hence, the results –although are conducted on the same set of algorithms with similar conditions of block sizes, power efficiency etc.– are independent from the previous dataset. The algorithms tested remain the same (i.e., AES-256-CTR, GMAC, AES-256-GCM).

In order to compare the average power / battery consumption, we measure the device’s power consumption during the execution of the openssl speed command for each algorithm and each block size of 16, 32, 48, 64, 256, 1024, 8192, and 16385 bytes. During the execution of speed command, one of the CPU cores is almost fully used. But faster algorithms can process more data than slower algorithms, i.e. more efficiently. The calculated energy per byte for each algorithm is summarized below.

| A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- |
| Block size (bytes) | AES-256-CTR -Consumed Energy per byte (nJ/byte) | GMAC - Consumed Energy per byte (nJ/byte) | B+C | AES-256-GCM Consumed Energy per byte (nJ/byte) | GCM energy consumption efficiency gain over B+C (%) |
| 16 | 3.59 | 40.97 | 44.56 | 24.28 | 46% |
| 32 | 1.81 | 20.28 | 22.09 | 12.28 | 44% |
| 48 | 1.41 | 14.14 | 15.55 | 8.40 | 46% |
| 64 | 1.33 | 10.25 | 11.58 | 6.73 | 42% |
| 256 | 0.79 | 2.78 | 3.57 | 2.36 | 34% |
| 1024 | 0.56 | 0.85 | 1.41 | 1.11 | 21% |
| 8192 | 0.53 | 0.31 | 0.84 | 0.75 | 10% |
| 16384 | 0.50 | 0.27 | 0.77 | 0.72 | 6% |
|  |  |  |  |  | 31% |

Table 3: Electric Power Consumption Analysis

From these results, we can derive the following conclusions:

- AES-256-GCM has an advantage in energy consumption for all block sizes. The average energy consumption efficiency gain is approximately 31% on the mobile device.

- A larger efficiency gain can be observed for smaller message sizes. Hence, devices may benefit from the use of AES-256-GCM when both encryption and integrity are required, particularly for small or medium bock sizes.

Note: It is assumed that AEAD algorithms, when adopted in the 3GPP System and used to provide both encryption and integrity protection, would be operated in single-pass mode. If AEAD algorithms are not adopted, and encryption and integrity are provided separately, the performance is expected to be worse.

# 5	Detailed proposal

It is proposed to thoroughly analyse requirements of a next-generation 3GPP System for supporting AEAD algorithms for AS/NAS security, identify potential challenges, including but not limited to the forementioned challenges outlined in this paper, and utilize these findings during the development of the next-generation 3GPP System.