3GPP TSG-SA3 Meeting #120	S3-250612

# Athens, Greece, 17 - 21 February 2025

Source:	Sony

Title:	KI#3, Conclusions

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_AIOT_Sec

Comments

The Ambient IoT Check point at TSG SA#106 resulted in Way Forward in SP-241980.

The Way Forward includes important aspects on how to focus the work in Q1 to reach conclusions that can be implemented during the normative phase during Q2 and Q3, 2025.

The most important guidance is to focus the work on device type 1 (the most constrained device type) and topology 1 (base station reader), as this is what RAN will specify in Rel-19 according to approved WID in RP-243326.

In TR 33.713 v.0.5.0 there are 18 solutions documented related to KI#3 Privacy by protecting AIoT device identifiers. Solutions addressing KI#3 shall provide “Mechanisms for mitigating privacy threats (described above) by identifying, linking, and tracking the identifiers of AIoT Device(s) shall be supported”.

Proposal 1: As device type 1 is very constrained it can be concluded that these devices cannot use an USIM. Solution 11 and 12 cannot be considered.

SA#106 is specifically asking SA3 whether to use a temporary ID or not. Temporary ID is an ID used to hide the permanent ID of the AIoT device when the network addresses the AIoT device. The solutions documented in TR 33.713 use different names for this ID: Temporary ID, Protected ID or Concealed ID, but the all serve the same purpose - to hide the Permanent ID. The differences between the solutions are how and when the network/device derives the ID.

Solutions 20, 21, 22, 30, 35 and 38 require the AIoT device to perform computational tasks after receiving the paging/trigger message to detect that the paging/trigger message was intended for the device. Such paging/triggering message will wake up all AIoT devices within the Reader’s range and all AIoT devices will start executing computational tasks. This means that AIoT devices will waste their power even if the paging/trigger message was not intended for them, resulting in that it might not be possible to reach them for some time until enough power has been harvested again.

Observation: To not deplete the AIoT device’s limited power storage, the computational tasks must consume less power than what the AIoT device can harvest from the paging/trigger message or RF signal accompanying the paging/trigger message. The AIoT device must have enough power remaining after these tasks to respond in case the AIoT device was paged/triggered. There is no evidence that the power available to harvest from the paging/trigger message meets the power budget of the computational tasks.

Proposal 2: Not consider solutions 20, 21, 22, 30, 35 and 38 as there is high risk that such solution will result in situations the device will not be able to respond when it is actually paged/triggered.

Solutions 23, 31 and 34 includes either the full permanent ID or partial permanent ID (Filter info or group ID) in the paging/trigger message. The response includes a protected ID. In case the response can be linked to the paging/message, then the permanent ID/partial permanent ID is exposed.

Observation: As device type 1 is using backscattering to respond, it will be possible to link the response to the trigger message. As the trigger message includes the permanent ID or partial permanent ID these solutions cannot provide privacy protection.

Proposal 3: Not consider solutions 23, 31 and 34.

The remaining solutions: 13, 24, 25, 26, 27, 28 and 29 require non-volatile memory (NVM) and can be divided into two groups:

- Network assigned temporary ID in an encrypted message: solution 24 and 29.

- Network provided security parameters allowing AIoT ID to derive temporary IDs locally: Solutions 13, 25, 26, 27 and 28.

Group 1: The solutions in this group require more messages exchanged between the AIoT device and network After the first pair of messages (Inventory messages) have been exchanged a second pair of protected messages including the encrypted new temporary ID must be exchanged.

Group 2: The second pair of messages as required for group 1 is not needed. But there is a risk that the algorithm used the device may get out-of-synch with the network algorithm.

- Solution 13 proposes to always provide a new Nonce to the device in the Inventory request.

- Solution 25 proposes that when the network detect the out-of-synch case and in that case restarts using the default ID.

- Solution 26 proposes that when the network to detect the out-of-synch case and a new seed is provided to the device.

- Solution 27 and 28 do not provide any solution to the out-of-synch case.

Observation: Use of NVM increases the complexity of the device and writing to the NVM requires energy. This means that a device shall not engage in AIoT communication if it has not harvested enough power to perform both communication and writing to the NVM.

Proposal 4: As group 1 above always requires (even for Inventory service) an extra pair of encrypted messages exchanged compared to group 2, it is proposed to not select solutions out of group 1.

Proposal 5: It is proposed to select baseline principles from the remaining solution in Group 2.

Observation: Solution 13: The initial page includes the default ID/permanent ID and a Nonce. The Nonce is used to calculate the next Temporary ID to be used in the communication with the AIoT device. However, the assumption in RAN2 is that the paging message includes ID(s). As an alternative to providing the AIoT device with a Nonce in the paging message the Nonce can be provided using write command after the initial paging message, the initial paging is done when the network does not have a context of the specific AIoT device.

Observation: Solution 28 encrypts device ID based on preconfigured information. This could be way to conceal the permanent ID in the initial paging message.

Observation: Solution 26 propose to use a temporary ID derivation algorithm running both on the AIoT device and AIOTF. If out-of-synch is detected by the network, the network provides a new Nonce to restart and re-synchronize the temporary ID algorithms running both on the AIoT device side and network side.

Observation: Solution 27 is similar to solution 26 but does not address the out-of-synch issue.

Observation: Solution 25 adds one additional message at the end of the procedure in which the network provides the device with a new freshness parameter to be used to calculate the next temporary ID. It would be desirable to avoid the extra message, this may need further consideration when specifying the algorithm to derive new temporary ID during the normative work.

Proposed Changes

* * * First Change * * * *

## 7.3	Conclusion on KI #3

For key issue #3 (Privacy by protecting AIoT device identifiers) described in clause 5.3, it is concluded that the normative work is based on the following principles:

- If the AIOTF does not have a context of the AIoT device, then (when triggered by an AF) the AIOTF “pages” the AIoT device with a concealed ID derived using pre-configured parameter(s) to conceal the permanent ID. After the receiving the Inventory response, the AIOTF sends a write command to provide the AIoT device with a NONCE.

- After the AIOTF received the command response from the AIoT device, the AIOTF creates a device context and store at least the permanent ID, NONCE and the latest protected ID or the next protected ID that shall be used for next communication with the AIoT device.

- Both the AIOTF and the AIoT device locally derives the first protected ID using the NONCE provided to the AIoT device. This protected ID is used in the next communication between the network and AIoT device.

NOTE X: The details of the protected ID derivation algorithm will be specified during the normative work.

- An AIoT device that received a NONCE, shall listen for paging messages that can include either a protected ID or the concealed ID. This will allow the AIOTF to always reach the AIoT device to restart the protected ID derivation algorithm, if the protected ID derivation algorithm running on both sides are no longer synchronized.

- The AIOTF pages the AIoT device with the next unused protected ID and the AIoT device responds with that protected ID. Both the AIOTF and AIoT device locally derives the next protected ID that shall be used for next communication with the AIoT device.

- If the AIOTF pages a group of AIoT devices, then the AIoT devices will responds with their unused protected IDs. Both the AIOTF and the AIoT devices locally derive the next protected IDs that shall be used for next communication with the AIoT devices.

- If the AIOTF does not receive a response from the AIoT device the AIOTF when paging with protected ID, the AIOTF may page the device with the concealed ID to provide a new NONCE using Command service.

* * * End of Changes * * * *