3GPP TSG-SA3 Meeting #120	S3-250982

# Athens, Greece, 17 - 21 February 2025	merger of 542, 626, 694

Source:	China Mobile, ZTE, Huawei, Nokia

Title:	Pseudo-CR on system architecture and security assumption

Document for:	Approval

Agenda item:	4.16

Spec:	3GPP TS 33.545

Version:	0.0.0

Work Item:	5G_Femto_Sec

Comments

This contribution proposes system architecture and comparison table for the gap between security aspects of NR Femto and TS 33.320, and the extent to which the clauses in TS 33.320 can be reused. .

Proposed Changes

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[y]	3GPP TS 23.501: "System Architecture for the 5G System".

[z]					3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

* * * Second Change * * * *

4	Overview of Security Architecture and Requirements

In 5GS, an NR Femto node connects to 5GC directly via NG interface or optionally connects to 5GC via NR Femto Gateway (NR Femto GW) as specified in TS 38.300 [x] and in Annex V of TS 23.501[y]. Additionally, other functions that will be used for security purpose are further depicted in Figure 4.X.1. Such security aspect enhancements to NR Femto are described in clause 5 of the present document.

UE

NR Femto

SeGW

Insecure link

Operator’s security domain(s)

NR Femto GW

NR Femto MS

NR Femto MS

AUSF / UDM

UPF

Figure 4.X.1: System Architecture of NR Femto

The following architecture and security assumptions are applied to the present document:

-    The security requirements specified in TS 33.320[z] shall be derived for NR Femto with the following modifications: H(e)NB replaced with NR Femto, H(e)MS replaced with NR Femto MS, optional L-GW replaced with the UPF, Iurh/X2 interface replaced with Xn interface. The extent to which TS 33.320 shall be reused is captured as follows:

- Security architecture and assumption: Clause 4 in TS33.320[z] shall be reused. Text in TS33.320[z] clause 4.4, the security requirements are extended in clause 4 of this document.

- Security Features: Clause 5 in TS33.320[z] shall be reused. Text in TS33.320[z] clause 5.2, the device authentication is extended in clause 5.1 of this document. Text in TS33.320[z] clause 5.3, the Hosting Party authentication is extended in clause 5.2 of this document.

- Security Procedures in NR Femto: Clause 6 in TS33.320[z] shall be reused.

- Security between NR Femto and SeGW: Clause 7 in TS33.320[z] shall be reused. Text in TS33.320[z] clause 7.2, the device authentication is extended in clause 5.1 of this document. Text in TS33.320[z] clause 7.3, the Hosting Party authentication is extended in clause 5.2 of this document. Text in TS33.320[z] clause 7.4, the IPSec tunnel establishment is extended in clause 5.4 of this document.

- Security aspects of NR Femto management: Clause 8 in TS33.320[z] shall be reused. Text in TS33.320[z] clause 8.1, the location verification mechanism is extended in clause 5.3 of this document. Text in TS33.320[z] clause 8.2, access control mechanism for NR Femto is extended in clause 5.5 of this document.

- Security aspects of emergency call handling: Clause 9 in TS33.320[z] shall be reused.

- Security aspects for mobility: Clause 10 in TS33.320[z] shall be reused.

- Security Procedures for direct interface between base stations: Clause 11 in TS33.320[z] shall be reused.

* * * End of Changes * * * *