3GPP TSG-SA3 Meeting #120	S3-251126

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.558 | CR | DraftCR | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on skeleton for TS 33.558 on ID verification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, China Unicom, NOKIA, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This proposal is to add the ID verification concluded in the clause 7.2 of TR 33.749, i.e., The following mechanism design principles are to be followed in normative phase: - The information used in Nnef_UEId API call invocation and UE Identifier API call invocation needs to prevent the use of permanent or semi-permanent UE identifiers subject to be abused/compromised by malicious applications. - The entity responsible for the generation of the information needs to be part of the 5G system in HPLMN, and needs to be verified by the 5G system in HPLMN. Alternatively, the information can be generated by both the UE and the 5G system. - The information may be used by the AF invoking the NEF APIs, so that the NEF APIs can identify the UE by using the information. NOTE 1: The API invoker does not need to obtain the permanent 5G UE ID, which improves the privacy. Alternative mechanisms having the design principles above will be captured in an informative annex in the normative work. NOTE 2: The principles above will not prevent usage of existing mechanisms of Nnef_UEId API and UE Identifier API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add the ID verification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 33.558 on ID verification is not captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.X, 6.Y, Annex Z. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of The First Change ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[4]	Void

[5]	3GPP TS 23.558: "Architecture for enabling Edge Applications."

[6]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs"

[8]	Void

[9]	Void

[10]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[11]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[12]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[13]	Void

[14]	Void

[15]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[16]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[17]	IETF RFC 7519: "JSON Web Token (JWT)".

[18]	IETF RFC 7515: "JSON Web Signature (JWS)".

[19]	IETF RFC 9113: "HTTP/2".

[20]	IETF RFC 9110: "HTTP Semantics".

[xx]	3GPP TS 23.502: "Procedures for the 5G System"

*** End of The First Change ***

*** Start of The Second Change ***

5.1.X Secure retrieval of 5G system UE Ids requirements

-	The non-permanent information used in Nnef_UEId API call invocation and Eees_UEIdentifier API call invocation should prevent the use of permanent subject to be abused/compromised by malicious applications.

-	The entity responsible for the generation of the non-permanent information should be part of the 5G system in HPLMN and should be verified by the 5G system in HPLMN. Alternatively, the non-permanent information can be generated by both the UE and the 5G system.

-	The non-permanent information can be used by the AF invoking the NEF APIs, so that the NEF APIs can identify the UE by using the non-permanent information.

- 	The non-permanent information can be used by the EAS/EEC invoking the Eees_UEIdentifier API, so that the EES APIs can identify the UE by using the non-permanent information.

NOTE 1:	The API invoker does not need to obtain the permanent 5G UE ID, which improves the privacy.

NOTE 2:	The principles above do not prevent usage of existing mechanisms of Nnef_UEId  API and UE Identifier API.

Example implementation specific mechanisms having the design principles above are captured in an informative Annex Z of this document.

*** End of The Second Change ***

*** Start of The Third Change ***

Annex Z (informative): Mechanisms for secure retrieval of 5G system UE Ids and privacy related information

Z.1 General

This clause describes the alternative mechanisms aligning with the priciples that decribed in the Clause 5.1.X of the present specification, i.e., UE ID Token based mechanism, Hash based mechanism, Temporary ID based mechanism, and AKMA based mechanism.

These principles can be addressed by solutions having the main steps:

-	The network provides a non-permanent information (e.g., temporary UE ID, a UE ID Token, Hash, etc.) to the UE if required.

-	The UE presents this non-permanent information to the AF.

-	The AF sends a request towards the network using the non-permanent information.

-	The network executes the request for the corresponding UE associated to the received non-permanent information.

The present Annex gives examples of possible procedures/mechanisms that could realize the principles/steps defined above. Described examples are not standardized in the present document and their realization requires implementation specific means.

Z.2	UE ID Token based mechanism

Z.2.1	Introduction

According to the mechanism, a new entity called UE ID Server located inside the operator domain provides a separate UE ID Token to the Client running on the UE for each request coming from the Client. The Client interacts with the AF by using the UE ID Token and then the AF interacts with network by using the received UE ID Token. The network resolves the UE ID Token by interacting with the UE ID Server to identify the UE. The UE ID Server and the interactions with the UE ID Server can be realized by implementation specific means.

Z.2.2	Details

Generic representation of the mechanism is presented in Figure Z.1.2-1.

Figure Z.2.2-1: Generic view of UE ID token based mechanism

Step 1: The client obtains a UE ID token from the UE ID server which is located inside the operator domain. UE ID server can identify the UE by using the private IP address of the PDU session. The UE ID Server can fetch the IP address from the IP packets received from the Client. The UE ID Server as a trusted AF can use the Nbsf_Management_Discovery service operation with UE address and IP domain to retrieve the session binding information of the UE. For the UE ID Server service consumption by the client, the UE where the client is running on needs to execute the primary authentication successfully and have a PDU session. The client can authenticate the UE ID Server by using server certificate. The Client needs to be configured with the root certificate or certificate of the UE ID Server, or the Internet (public) PKI mechanism can be used. The client does not need to authenticate with the server since the server issues the UE ID token, which does not include cleartext sensitive information, to the client running on the UE who has already executed the primary authentication.

NOTE 1:	Since the issuer and resolver of the UE ID token is the UE ID Server, the details of the UE ID token (e.g., whether it is a signed token or a random number) can be left to implementation.

Step 2: The client forwards the token to the AF.

Step 3: The AF invokes a NEF service and use the UE ID token as UE identifier.

NOTE 2:	The UE ID token can be provided as input in any NEF Service that takes GPSI as input (as NAI/External Id format).

Step 4: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 5: The UE ID server returns the requested information.

Step 6: The NEF interacts with other NFs as per existing NEF service procedures as per TS 23.502.

Step 7: The NEF returns the result of the NEF service execution.

Note that if the AF is a trusted AF, it can directly interact with the UE ID Server to identify the UE and the obtain required information such as IP address, and then invoke the NF/NEF directly with the required inputs.

Application of the mechanism to the case of NEF API invocation by the EAS/EES is presented in Figure 6.9.2-2.

Figure Z.2.2-2: The EAS/EES invokes NEF API

Step 1: The AC/EEC obtains a UE ID token from the UE ID server.

Step 2: The AC or the EEC respectively forward the token to the EAS or the EES.

Step 3: The EAS/EES invokes a NEF service and use the UE ID token as UE identifier.

Step 4: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 5: The UE ID server returns the requested information.

Step 6: The NEF interacts with other NFs as per existing NEF service procedures as per TS 23.502.

Step 7: The NEF returns the result of the NEF service execution.

Application of the mechanism to the case of EES API invocation by the EAS is presented in Figure 6.9.2-3.

NOTE 3:	The invocation to existing APIs in step 3 and 4 by this mechanism in the following figure is left to implementation.

Figure Z.2.2-3: The EAS invokes the EES API

Step 1: The AC obtains a UE ID token from the UE ID server.

Step 2: The AC forwards the token to the EAS.

Step 3: The EAS invokes a EES service and use the UE ID token as UE identifier.

Step 4: The EES invokes the NEF API by sending the UE ID token.

Step 5: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 6: The UE ID server returns the requested information.

Step 7: The NEF sends the UE ID to the EES.

Step 8: The EES returns the result of the EES service execution.

Z.3	Hash based mechanism

Z.3.1	Introduction

This mechanism proposes to use fresh hash value with count, which is a fresh value shared between the UE and the 5GC, to authorize the AF to retrieve the UE ID. This solution addresses the principles in clause 5.1.X. if the NAT is not deployed after the UPF.

Z.3.2	Details

This mechanism supports the 5GC in implementation specific way to verify whether the AF is authorized by the UE to retrieve the UE ID based on the IP address, and verify whether the parameters used for verification is replayed by the adversary. Specifically, when the UE receives the UE ID request, it in implementation specific way generates a hash value based on the count, salt, SUPI and IP address, and then sends the generated hash value and count to the AF, which uses the IP address to request for the UE ID from the NEF. The NEF then requests BSF to provide information of the UE (i.e., SUPI). Before providing the SUPI of UE, the BSF in implementation specific way also generates a hash' based on the shared count, salt, SUPI and IP address, and then compare the hash' value with the received hash value. If the hash value is equal to the hash' value, the BSF can provide the SUPI to the NEF. Otherwise, the BSF will provide a failure indication to the NEF.

Figure Z.3.2-1: Authorization of AF and verification of AF provided IP address

0.	During the PDU session establishment procedure, SMF generates the salt, which is a random value, and then sends the salt to both UE and BSF in an implementation specific way. SMF can send the salt to the BSF along with SM policy Association Establishment or SM policy Association Modification message in clauses 4.16.4 or 4.16.5 of TS 23.502 [xx]. If the SM policy Association Establishment and SM policy Association Modification procedures are not performed, SMF will not send the salt to the UE.

1.	UE receives the UE ID request.

2.	The UE generates a hash value based on the SUPI, private IP address, salt, and count, and other values that is only known to both the EEC and BSF (e.g., DNN of the PDU Session, and S-NSSAI of the PDU Session). After that, the UE increases the count by 1, which is used for the next hash value generation procedure. Note that the count can be configured to start from 0 in each PDU session. In order to mitigate the rainbow table attack, the following mechanisms should be adopted:

(1) increasing the length of the hash algorithm's input.

The length of the salt can be set as 128bits to make the rainbow table attack infeasible [15].

(2) key stretching. The inputs can be run through the hash function multiple times to increase the time to build a precomputed rainbow table.

The UE sends the hash value and the count together to avoid the out-of-synchronization between the UE and BSF.

3.	The UE sends the generated hash value and count to the AF.

4.	The AF in an implementation specific way invokes the Nnef_UEId_Get request with the parameter of hash value, count, and public IP address to get the UE ID.

5-8.	This mechanism reuses Steps 3-6 in Clause 4.15.10, TS 23.502 [8].

9.	When the NEF invokes the Nbsf_Management_Discovery service operation, the NEF in an implementation specific way provides the hash value and private IP address to the BSF.

10.	In an implementation specific way, the BSF determines the SUPI, salt and count based on the private IP address and then generates the hash' value, and then compares the hash' value with the received hash value.

11.	If the hash' value is equal to the received hash value, the BSF in an implementation specific way will respond to the NEF with a SUPI. Otherwise, the BSF in an implementation specific way will respond to the NEF with an indication that request is denied with the reason that the private IP address is not associated with the UE.

12-14.	This mechanism reuse Steps 9-11 in Clause 4.15.10, TS 23.502 [xx].

Z.4	Temporary ID based mechanism

Z.4.1	Introduction

This mechanism proposes to use temporary ID to addresses the security requirements illustrated in clause 5.X.1.

The main design principle of the mechanism is based on the utilization of a temporary identifier (ID) as UE identity to fetch the Application Function (AF) specific UE Id from Nnef_UEId service. The temporary ID is generated by the 5G Core, such as UDM (this is left to implementation), it includes a randomly generated value, and it is for example associated to the PDU session.

Z.4.2	Details

Figure Z.3.2-1 illustrates the procedure to fetch the 5G system UE Id and privacy related information.

Figure Z.4.2-1: Procedure to fetch the 5G system UE Id and privacy related information

Precondition:

0. The EEC requests a Temporary ID to UE through interface provided by UE module.

1. PDU session establishment / modification request is sent from the UE to the 5G system.

2. The 5G Core generates (per UE) an internal temporary ID (with a randomly generated part) for PDU session.

NOTE 1:	The concrete network entity in 5G Core responsible for generating the internal temporary ID is to be decided by the implementation.

NOTE 2: The UE could request multiple temporary IDs during the same PDU session (PDU Session modification). The UE can use this to invalidate previous IDs which might have been compromised and/or at fix intervals without initializing another PDU session establishment/modification.

3. The 5G Core returns the internal temporary ID to the UE as part of the NAS PDU session establishment/modification response.

4. The EEC uses the allocated temporary ID, received from the UE, inside the Edge application layer communication with the application function AF (e.g., EAS).

5. The AF uses the received temporary ID to invoke the Nnef_UEId API intended to fetch the 5G system UE identifier.

6. The NEF verifies the temporary ID, authorizes the request coming from the AF and accordingly retrieves from the 5G Core the AF specific UE Id corresponding to the temporary ID.

7. The NEF replies to the AF with the corresponding AF specific UE Id.

8-9 (Optional). Assuming the application logic requires to fetch the UE location, a location request is sent to the NEF using the AF specific UE Id, and the corresponding information is provided by the 5G system through the NEF.

10. The AF responds to the UE (EEC).

Z.5	AKMA based mechanism

Z.5.1	Introduction

The main design principle of the solution is based on AKMA feature, more specifically on the A-KID, an application specific ID derived from the long term symmetric pre-shared key K between the UE and the network. The EDGE application server (EAS) and/or enabler server (EES), use the AKMA protocol to retrieve the 5G UE privacy related information.

The solution assumes that AF (EES/EAS) communicates with 5GC through NEF.

Z.5.2	Details

Figure Z.4.2-1 illustrates the procedure to fetch the 5G system UE privay related information based on AKMA.

Figure Z.5.2-1: Procedure to fetch the 5G system UE privacy information based on AKMA

Preliminary steps:

- Primary authentication
- AKMA Key derivation (A-KID) as per procedure in TS 33.535 [11].
- Creation of AKMA context in AAnF (SUPI, KAKMA, A-KID) as per procedure in [11].

Step 1. The application client in the UE (EEC) requests a service to the AF, i.e., EDGE application server (EAS, EES), including the A-KID (AKMA Key Identifier) in the request.

Step 2-5. AKMA procedure intended to provide KAF to the AF from the AAnF via NEF as specified in clause 6.3 of [11].

NOTE 1: AF specific UE Id does not need to be part of the of the incoming request (step 2), since the NEF will provide the GPSI (external ID) to AF.

Step 6. The AF requires to know privacy information of the UE available in the 5G Core, such as the the location of UE, and makes the request to the 5G Core via NEF, using the given GPSI as UE identifier.

Step 7. The NEF checks the GPSI and if the request is authorized, the corresponding information, in this example location of the UE, is provided to the AF.

Step 8. The AF responds to the UE (EEC).

NOTE 2:	Special consideration can be needed for the point that the same A-KID will be used until the next execution of the primary authentication, and it will be same for all the AFs.

*** End of The Third Change ***