3GPP TSG-SA3 Meeting #122	S3-252278-r3

Fukuoka, Japan, 19 - 23 May 2025			   merger of S3-252126, 252157, 252120, 252007, 252080, 251903, and 252043

Source:	Huawei, HiSilicon, CATT, Qualcomm Incorporated, China Telecom Corporation Ltd,  Xiaomi Technology, Samsung, InterDigital, Philips International B.V.

Title:	Resolving EN in Annex of Split MME by implementation

Document for:	Approval

Agenda item:	4.19

Spec:	3GPP TS 33.401

Version:	18.3.0

Work Item:	5GSAT_Ph3_SEC

Comments

This contribution aims to delete the (D)DoS attack EN, the way to prevent (D)DoS attack can be left for implementation.

* * * First Change * * * *

# X.2	Security aspects of Split MME architecture

Mutual authentication between the UE and the network in a split MME architecture can involve more than one satellite (i.e., more than one MME-onboard), in which case the ground segment of the network is responsible for the selection and provisioning of MME-onboard the same, or another satellite, with the necessary information (e.g., Authentication Vector) to perform or finish an authentication procedure. The MME on-board obtains the EPS authentication vectors when the feeder link is available and stores the authentication vectors when the service link is unavailable.

The NAS security shall be terminated on the MME-onboard. The ground segment of the network shall ensure that the latest NAS security context of the UE, or an Authentication Vector, is available at the MME-onboard.

NOTE 1: 	The distribution and synchronization of the latest NAS security context between the MME-ground and MME-onboard is out of the scope of 3GPP.

NOTE 2: 	The security environment on board a satellite is characterized by relatively low processing and storage capabilities of the equipment on board the satellite compared to terrestrial communication equipment. Due to the feeder link temporary unavailability in split-MME deployment, a malicious entity can flood satellites with spoofed Attach Requests (unauthenticated Attach Requests). This consumes storage space in the MME-onboard before authentication occurs, hence potentially resulting in (D)DoS attack on the Satellite. Addressing said (D)DoS attack in the Split MME deployment is left to implementation.

* * * End of Changes * * * *