3GPP TSG-SA3 Meeting #122																			S3-251865

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2136 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Selection of authentication mode for MRI in forwarded mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2-Sec |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current document proposes to add CCM_8 authentication mode as a raccomandation to be used with AES in forwrded mode. RFC 9147 presents a general analalysis of CCM_8 that is not limited to it’s applicability within DTLS. In particular in section 4.5.3 states: Therefore, TLS_AES_128_CCM_8_SHA256 MUST NOT be used in DTLS without additional safeguards against forgery. Implementations MUST set usage limits for AEAD_AES_128_CCM_8 based on an understanding of any additional forgery protections that are used. In Appendix B is stated that: This analysis supports the view that TLS_AES_128_CCM_8_SHA256 is not suitable for general use. Specifically, TLS_AES_128_CCM_8_SHA256 cannot be used without additional measures to prevent forgery of records, or to mitigate the effect of forgeries. This might require understanding the constraints that exist in a particular deployment or application. Moreover, there is currently no evidence that a shorter authentication tag is needed for efficiency. Without an explicit need, standardizing a possibly insecure algorithm should be avoided. In conclusion it is proposed to only note the possible use of CCM_8, to facilitate the usage of a different authentication mode, in the case where the limitiation of CCM_8 affect the security of the whole solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose to suggest the authentication method as part of a NOTE to avoid selecting an authentication method which present well-known limitations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No authentication method will be reccomended for MRI protection in forwarded mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.2.3, 18.2.4, 18.2.5, 18.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 18.2.3	Using Forwarded mode

When the end-to-end traffic is using QUIC, the Forward Mode in draft-ietf-masque-quic-proxy [126] can be used as an optimisation. In forward mode, end-to-end QUIC packets can be sent in parallel to the tunnel, which has the benefit of not having to encrypt the e2e encrypted data twice, see Figure 18.2.3-1. Forwarded packets can be transformed with a packet transform mechanisms that are negotiated in the CONNECT request. For XRM data, a packet transform is used where the media related information is protected and appended to the packet. The AS appends and encrypts the media related information, the UPF decrypts and consumes the media related information and removes it before forwarding the end-to-end packet towards the UE.

The QUIC aware Forwarded mode [126] negotiates a packet transform that modifies the packet in addition to the minimal necessary to make the forwarding work. In the QUIC-aware Forwarded mode the AS proxy replaces the Connection ID (CID) field with the virtual CID (VCID) field and rewrites the IP/UDP header so that the packet arrives at the tunnel endpoint (UPF). More details on how the vCID is created is provided in 18.2.7.

The packet transform needs to add the following data:

Media related information (MRI) Length field: The length of the protected Media related information (including integrity tag).

Nonce Counter field: Contains the 16-bit least significant bits of a 64-bit counter used to construct the nonce to the AEAD protection. The 64-bit counter is incremented with one for each payload protected by the transform for the current VCID. The 64-bit counter is initialized to zero (0) for each VCID. For more information about the counter, see clause 18.2.6.

Protected Media related information: The N bytes of AEAD output after protection operation using the below defined AEAD algorithm.

If there is no Media related information, the transform shall set the length field to zero (0), and no other additional fields (Counter LSB or Media related information) are inserted into those packets.

Figure 18.2.3-1: Overview of the connect-UDP Forward mode architecture for protection of XRM Media related information

### 18.2.4	Security parameters to be used in the Forwarded mode

When the Forwarded mode is used, Media related information (MRI) is encrypted and integrity protected using a separate security context from that being used in the QUIC tunnel between AS and UPF.

Media related information is encrypted and integrity protected using AES.

NOTE: CCM_8 can be used as authentication mode for AES.

Unique keys shall be used per direction. In the present document data is only transmitted in one direction is used. The encryption key is derived as described in clause 18.2.5. Nonce is created as described in clause 18.2.6.

### 18.2.5	Key derivation in the Forwarded mode

The keys are extracted from the QUIC connection between UPF and AS proxy using the TLS 1.3 key exporter defined in RFC 8446 [77].

The following input shall be used for the TLS 1.3 key exporter:

When CCM_8 is selected, the following parameter should be used:

Label: ‘EXPORTER-MASQUE-Transform-xrm-AESCCM128_64’ (To be registered with IANA)

Context_value: The VCID value || The forty (40) most significant bits of the counter

Key_length: 128 bits

### 18.2.6	Nonce and counter values in the Forwarded mode

For each invocation of AES, a 96 bit nonce is created by concatenating the last (rightmost) 32 bits of the VCID (VCID’) with the 64-bit counter C: VCID’||C.

To maintain synchronization even for large burst losses of XRM transformed packets, the full 64-bit counter is sent every time the 16-bit Least Significant Bit counter wraps. The counter is sent reliably on the corresponding HTTP/3 request QUIC stream.

The receiver of the transformed packet, i.e. the UPF for AS to UE traffic, shall perform replay protection to ensure that each 64-bit counter is only accepted once. In case the receiver detects reuse of the counter, the e2e packet shall be discarded (i.e. not forwarded).

* * * End of Changes * * * *